3GPP TSG SA WG 1 Meeting #113 	S1-261018

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	Moderator (China Unicom)

pCR Title:	Consolidation of KPI Requirements with Updated Value on AI Section

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1

Document for:	Approval

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Prepare and propose a way for consolidation on KPI requirements on AI section based on agreed table header, with updated values.

1. Introduction

Based on the meeting preparations, this document only provides the consolidation way forward and discussion regarding the AI KPIs, based on TR 22.870 and do not include performance requirements with editor’s note.

2. Reason for Change

To provide communication performance requirements contribution and reflect key points regarding the performance requirements of AI sections.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to new version of 3GPP TR 22.870.

Discussion Part

Besides the basic template consolidation, the following comments need to be considered：

General:

CM1: Table y.1-2 the “joint e2e latency” is probably to be properly defined or decoupled?

CM2: Table y.1-2 Joint E2E latency value needs to be divided into a communication value and a compute value.

CM3: Table y.1-1 primarily consists of communication KPIs. Reflect the table name.

CM4: The latency title in table y.1-2 need to align with “Max allowed end-to-end latency” but also need to highlight the difference from existing KPIs.

CM5: Is the table y.1-2 related with computing service?

CM6: A definition needed for Joint E2E latency.

CM7: “Joint e2e latency” is more from service level of SA1.

UC 6.48 Use case on service robot for power grid

The value and note 3 updated based on approved S1-254342.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

……

[y]	https://www.thinkwithgoogle.com/_qs/documents/2340/bc22e_The_Need_for_Mobile_Speed_-_FINAL_1.pdf

* * * Next Change * * * *

<Proposed change in revision marks>

## 14.2	Consolidated Performance Requirements

Editor’s Note: This clause is FFS and was based on S1-254015. The Tables (and titles) will be updated as the KPI consolidation is completed.

### 14.2.1	AI

[CPR 14.2.1-1] The 6G system shall provide various communication and AI services with the following KPIs.

Table 14.2.1-1: Communication performance requirements for burst traffic

| Use Cases | Burst size | Max Allowed end-to-end latency for a burst (note B-0) | Service bit rate: user-experienced data rate | transmission latency of a packet | Average packet size | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement (UC 9.3 A) | UL: [150 Mbits] Nokia: UL/DL: [20 MB] (note A-1) | UL: [1500ms] Nokia: UL/DL: [750ms] (note A-2) | UL: [100 Mbps] Nokia: UL/DL: [200 Mbps] | N/A | > 800B (Comment #112 ad-hoc: any value?) | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement (UC 9.3 A) | UL: [5-20 Mbits] Nokia: UL: [1-3 MB] (note A-1) | UL: [50ms] (note A-2) Nokia: UL: [50ms] (note A-3) | UL: [100 – 400 Mbps] | N/A | > 800B (Comment #112 ad-hoc: any value?) | Stationary or Pedestrian | Countrywide |
| Image based GenAI app Nokia: Image based GenAI app (Text to image generation) (UC 6.26 B) (note B-1) | 400 KB Nokia: UL: 400 KB? | 50ms Nokia: UL: 50ms | UL: 64 Mbps | 20ms | > 800B | Stationary or Pedestrian | Countrywide |
| Video based GenAI app Nokia: Video based GenAI app (visual assistant) (UC 6.26 B) (note B-1) | 20 MB Nokia: UL: 20 MB | 400ms Nokia: UL: 400ms | UL: 400 Mbps | 20ms | > 800B | Stationary or Pedestrian | Countrywide |
| Chatbot (UC 6.26 B) (note B-2) | 0.5 KB Nokia: UL: 0.5 KB? | 20ms Nokia: UL: 20ms | UL: 200 Kbps | 30ms | < 800B | Stationary or Pedestrian | Countrywide |
| NOTE A-1: Assuming 6 x 4K raw pictures and compression ratio in [21] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [23] for gaming enhancement. NOTE A-2: 1500ms is derived from the E2E latency of 3 s (based on users' patience statistics as shown in [22][y]) and 1.5 s for processing in the cloud and downloading. 50ms uplink latency is derived from [24]. Nokia: NOTE A-2: 750ms is derived from the round-trip time of 3 s (based on users' patience statistics for website loading time as shown in [22][y]) minus around 1.5 s for processing in the cloud and downloading, divided equally between UL and DL (photo enhancement implies to send back the enhanced photo which is assumed to have at least the same size). Nokia: NOTE A-3: 50ms uplink latency is derived from [24]. NOTE B-0: Max allowed end to end latency for a burst: max latency for sending out the whole packets within a burst. |  |  |  |  |  |  |  |

Editor’s Note: Average packet size is FFS.

NOTE: 	“UC 6.26 B” has no technical meaning and serves solely to maintain correspondence with the respective use case and NOTEs.

Table 14.1.1-2: Joint Performance requirements for communication and AI services

| Use Cases | Traffic type | Data frame size (byte) | Transfer interval | Service bit rate: user-experienced data rate | Service latency |  |  | Reliability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Max Allowed end-to-end latency （note 3） (i) | Beyond-communication latency (note 2) (ii) | Total value of the service latency (note 1) |  |
| Home robot perceiving overall context of the scene (UC 6.10 A) (note A-5) | UL camera data (note A-1) | [<1000] | [10ms] | [20-60 Mbps] (note A-2) | [30ms - 50ms] | [50ms - 90ms] (note 4) | [150ms] (note A-3) | [99.9 %] |
| Identifying individual objects in the scene (UC 6.10 A) (note A-5) | UL camera data (note A-1) | [<1000] | [10ms] | [20-60 Mbps] (note A-2) | [30ms - 50ms] | [100ms - 140ms] -[200ms - 240ms] (note 4) | [200-300ms] (note A-4) | [99.9 %] |
| Merge first two rows: Robot perceiving overall context of the scene, identifying individual objects in the scene | UL camera data (note A-1) | [<1000] | [10ms] | [20-60 Mbps] (note A-2) | [30ms - 50ms] | [50ms - 90ms] - [200ms - 240ms] (note 4) | [150ms-300ms] perceiving overall context: <150ms (note A-3) identifying individual objects: [200ms. 300ms] (note A-4) | [99.9 %] |
| Merge first three rows: Home robot overall context of the scene, Identifying individual objects, and conscious awareness for human interaction (Nokia: merge UC 6.10 6.49) (E///: merge and use name as Service robot) | UL camera data (note A-1) | [<1000] | [10ms] | 80-400Mbps note 5 | [30ms - 50ms] | perceiving overall context: [50ms-90ms] individual objects: [200ms-240ms] Conscious awareness: <[100ms - 140ms] (note 4) | perceiving overall context: <150ms (note A-3) identifying individual objects: [200ms. 300ms] (note A-4) Conscious awareness: [<200ms] (note C-3) | [99.9 %] |
| Service robot with enhanced capabilities (E///: Add "with enhanced capabilities" to distinguish it from the merged row.) (UC 6.48 B) | UL sensor data (without LiDAR) (note B-1) | 1250-12500 | 10ms | UL: 1-10 Mbps | [30ms - 50ms] | [<40ms] - [50ms - 90ms] (note 4) | 100-150ms (note B-4) | 99.99% |
|  | UL LiDAR | 345600 (note B-3) | 100ms | UL: 27.6 Mbps (note B-3) |  |  |  | 99.99% |
|  | Control command (high level task, action plan, etc.) (note B-2) | 625-12500 | 50ms | DL: 0.1-2 Mbps |  |  |  | 99.99% |
| Support robot conscious awareness for interacting with human user (UC 6.49 C) | UL cameras data (note C-1) | [<1000] | [10ms] | UL: [20-60 Mbps] (note C-2) | [30ms - 50ms] | <[100ms - 140ms] (note 4) | [<200ms] (note C-3) (note C-5) | [99.9%] |
| NOTE 1: The total value of the service latency refers to the maximum allowed round-trip service latency experienced by the user for services provided by the operator, which is calculated as a of i(UL) + ii +i(DL). NOTE 2: 3GPP beyond-communication service latency refers to the processing latency of service (e.g. 6G AI Service), which could be provided by the operator or a 3rd party. NOTE 3: Max Allowed end-to-end latency: Refers to either the uplink or downlink direction. NOTE 4: The target value for this parameter is derived from the total Max-allowed Joint Latency, considering the portion typically allocated to communication latency. For instance, given a total latency of 150ms and a typical one-way communication latency of 30ms, a non-communication latency of 90ms (calculated as 150ms - 30ms * 2) is expected. Respectively the Max allowed Joint latency ‘s Lower and upper boundary (100ms-150ms), subtracted by the expected communication latency (30ms-50ms) results in the indicative non-communication Latency. NOTE 5: Lower bound of 80 is an approximation for 81.285Mbps is for 336x336 3x 8bit channel raw stream sent by the UE to a multimodal model encoder at 30fps after internal processing of the camera-captured 1080p frames to fit the input size of the model on the server side (or 144.506Mbps for 448x448 channel). In alternative, split processing of the images at the UE can result in encoded tokens: 336x336 images result in 24x24 768 dimensional vectors of floats (2 bytes each, uncompressible), resulting in 212Mbps. 448x448 images result in 32x32 768 dimensional vectors of floats (2 byte each) resulting in 377Mbps.(Nokia: for the merged one) NOTE A-1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE A-2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE A-3: Based on psychophysical and neurophysiological studies [281], [282], human beings can perceive the gist (e.g. overall meaning) of complex visual scenes within around 150ms after stimulus onset. This rapid initial perception enables us to grasp the general context of a scene, even if the stimulus is briefly presented (around 10ms). NOTE A-4: For human beings, it takes longer for a human being to identify individual objects, e.g. finer object identification requires larger than 200ms [283] and around 300ms [284]. NOTE A-5: In the target scenarios, robots are expected to have similar perception time as an average human being. NOTE B-1: Refers to the kinematic state, environment perception, manipulation status info except LiDAR to be sent from the service robot to the network to enable effective motion planning, object interaction and navigation. NOTE B-2: Refers to the control command towards service robot, e.g. high-level task, action plans, motion strategy, gripper command, etc. NOTE B-3: the frame size and data rate of LiDAR are based on frame rate 10Hz, 28800 points/frame, 12 byte for one point. The frame size is calculated by points/frame * bytes per point whereas the data rate is calculated by points/frame * bytes per point * bits per byte * frame rate (i.e. 28800*12*8*10). NOTE B-4: The typical robot control loops require 100-150ms latency [273] for AI inference, communication and control. For example, the communication may take about 40ms while the AI inference may take about 100ms [274] for the service robot. NOTE C-1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE C-2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE C-3: For physical AI robot interacting with a human user, the robot is expected to mimic the similar basic human brain reaction time including conscious awareness/recognition and decision-making based on various stimuli. Such human brain reaction time ranges from mean auditory reaction time 140-160ms, touch 155ms, to visual reaction time 180-200ms [279]. NOTE C-5: The human to robot latency (vice versa) is not included. |  |  |  |  |  |  |  |  |

NOTE 1:	The latency attributes doesn’t imply or preclude any architecture assumption or solution.

NOTE 2: 	“UC 6.48 B” has no technical meaning and serves solely to maintain correspondence with the respective use case and NOTEs.

Editor’s Note: Joint is FFS.

* * * Next Change * * * *

<Proposed change in revision marks>