3GPP TSG-SA WG1 Meeting #113	S1-261026

# Goa, India, 9 – 13 February 2026

Source:	Skylo Technologies, Verizon, Terrestar, Viasat, Boost Mobile

Title:	Discussion on voice over GEO call setup time

Agenda item:	5

Document for:	Discussion

# 1	Discussion

The current requirement for IMS voice call using GEO "Call Set-up Time" (Table 7.4.2-1 in TS 22.261) relies on reference 57, titled "Call Center Performance Affects Patient Perceptions of Access and Satisfaction" (available at https://www.ncbi.nlm.nih.gov/pmc/articles/PMC8177735/). The reference paper assesses “Average Speed of Answer” (ASA) in the context of VHA (Veterans Health Administration) hospital call centres in the United States.

A review of this paper reveals that the term "Call Set-up Time" does not appear anywhere in the document. The paper does not measure, analyze, or define the telecommunications signaling phase of a call. Instead, the value of 30 seconds in TS 22.261 is derived from a misinterpretation of the paper's "Average Speed of Answer" (ASA) metric. The reference defines ASA as the time a caller waits in a queue (e.g., listening to hold music) after the connection is established:

ASA=Sum(Answer speed x Call volume)Sum (Call volume)

The 30-second value cited in the requirement appears to be taken from a VHA administrative goal ("...goal for each facility is to have an ASA of less than or equal to 30 seconds…"), rather than the “Call Set-up Time” in the context of network signalling performance.

Using a call centre queueing metric to derive network signalling performance is a fundamental domain error. Users tolerate queueing (after connection) significantly longer than they tolerate call set-up time because they know the call has already been connected. Consequently, this reference is methodologically invalid for defining “Call Set-up Time” parameter in TS 22.261 [1] for voice over GEO.

# 2. 	Call set-up time for Voice over GEO

TS 22.261 refers to ITU-T E.800 [2] for the definition of "Call Set-up Time". Specifically, ITU-T E.800 defines this metric as the period between the user sending the address information and receiving the audible ringing tone, i.e.

The period starting when the address information required for setting up a call is received by the network (recognized on the calling user's access line) and finishing when the called party busy tone, or ringing tone or answer signal is received by the calling party (i.e., recognized on the calling user's access line). Local, national and service calls should be included, but calls to Other Licensed Operators should not, as a given operator cannot control the QoS delivered by another network.

Based on this definition, we reviewed relevant ITU Recommendations to establish an appropriate baseline in the following sections.

## 2.1 ITU-T E.774

Upon review ITU-T E.774 [8] (approved in Oct 1996), the only parameter in this recommendation relevant to “time duration” is “Authentication delay” (the other parameters, such as Link Blocking and Handover Failure, are probability ratios, not time durations). Clause 6.1.4 of E.774 explicitly defines authentication as a “credit/charge/calling card” process involving external “credit card agency database systems” chosen by the service provider. Therefore, this “Authentication delay” parameter isn’t applicable to modern networks and has nothing to do with “call set-up”.

In 3GPP NTN, authentication uses AKA (Authentication and Key Agreement) via the HSS before the user is allowed to access the system. This is an internal network procedure that takes milliseconds, not the seconds required for legacy credit card verifications. Therefore, we do not think E.774 is an appropriate baseline because its only relevant time parameter “Authentication delay” budgets significant latency for workflows that do not exist in modern Voice over IoT-NTN (GEO).

## 2.2 ITU-T E.771

ITU-T E.771 [7] (also approved in Oct 1996) specifies Grade of Service parameters for land mobile services. However, it is critical to note that the metric defined in E.771 is “Post-Selection Delay”, which is fundamentally different from the “Call Set-up Time” defined in ITU-T E.800 and used in TS 22.261.

- ITU-T E.771 Post-Selection Delay is an aggregate metric that explicitly includes “Paging/Alerting” (4.0 s), "Authentication" (2.5 s), and fixed network transmission latency (8.0 s). These values reflect the legacy circuit-switched architecture of 1996.

- ITU-T E.800 defines “Call Set-up Time” as the period starting when address information is received and finishing when the ringing tone is received, but calls to Other Licensed Operators should not, as a given operator cannot control the QoS delivered by another network. For a modern UE in Connected Mode, this period is strictly a signalling exchange

A detailed review of Table 1 in E.771 reveals three critical discrepancies that make it invalid for modern IMS-over-GEO:

- Paging (4.0s): E.771 includes paging in the delay budget. However, a connected UE performing an IMS signaling exchange does not require paging.

- Fixed Network Delay (8.0s): Most critically, E.771 includes an 8.0-second budget for “fixed network” transit in an international connection. This reflects legacy PSTN circuit-switching and copper trunks from 1996. In a modern IMS architecture, the “fixed” transit (IP Core/Backbone) introduces latency in the milliseconds level, not seconds.

Therefore, we should not refer to E.771 to define a modern IMS Voice over GEO “Call Set-up Time” KPI. The two technologies use totally different architectures and delivery methods. E.771 is tied to specific legacy hardware constraints (circuit switching).

## 2.3 ITU-T E.807

We believe ITU-T E.807 [3] is the most relevant reference. As stated in its title and scope, E.807 defines “user-centric parameters” intended to measure the “delivered service user experience” rather than focusing solely on network transport constraints. It sets a target that 95% of connected calls should have a call set-up time of less than 10 seconds. While E.807 was originally written for terrestrial cellular networks, the proposed IMS Voice service is a “Direct to Cell” offering. From a user's perspective, the device and the service interface are identical to a standard mobile phone. Therefore, the user expectation is defined by cellular standards (E.807), not by legacy satellite constraints. We must align our targets with what the user expects from a mobile phone call, rather than lowering the quality bar to match outdated satellite specifications.

ITU-T Rec. E.807 [3] identifies 10s as the maximum Call Set-up Time (CST) most users will tolerate before perceiving the service as unreliable. This benchmark is critical for the primary GEO voice use case: emergency communications in remote or off-grid areas. In such scenarios, long CSTs are not acceptable. A delayed connection may cause the caller to abandon the attempt, assuming the call has failed, with potentially life-threatening consequences.

Additionally, Call Set-up Time targets are equally critical for mobile-terminated calls, such as a callback from a Public Safety Answering Point (PSAP). In this scenario, a PSAP operator attempting to reconnect with a caller would have no knowledge of the underlying access technology (e.g., Voice over NB-IoT GEO). Expecting an emergency operator to wait up to 30 seconds for a connection is unrealistic and potentially dangerous, as they require immediate call establishment regardless of the user's network environment.

Furthermore, recent technical documents [4-6] in 3GPP SA2 provide analysis for Call Set-up Time showing that even with the constraint of channel bandwidth (1-3kbps) in GEO transmission conditions, an IMS call over GEO can achieve a call set-up time of less than 10 seconds.

In summary, as the proposed IMS Voice using IoT-NTN (GEO) service operates in a "Direct to Cell" deployment, this user-centric metric is the most appropriate benchmark. We propose aligning with ITU-T E.807’s guidance. A target of 10 seconds is compliant with user expectations for "Direct to Cell" services and is technically supported by modern architecture analysis.

# 3 Conclusion

- The current Reference 57 in TS 22.261 is invalid as it relies on a medical call center metric “Average Speed of Answer”, which measures the average time a caller waits in a queue (e.g., listening to hold music) after the connection is established, rather than the true “Call Set-up Time” telecommunications signaling metric

- ITU-T Recommendation E.807 exists as the correct standard for user-centric call handling parameters in mobile voice services.

- Technical analysis confirms that IMS voice calls over GEO can be established in less than 10 seconds, even under constrained bandwidth conditions (1-3 kbps).

- TS 22.261 should align its requirement on call setup time for IMS voice calls using GEO with established telecommunications standards. The invalid VHA reference should be replaced with ITU-T Recommendation E.807.

- Accordingly, the Voice over GEO requirement should be revised to 10 seconds, aligning it with the relevant standard while accommodating GEO propagation latency.

# References

[1] 3GPP TS 22.261 V20.5.0: “Service requirements for the 5G system”

[2] ITU-T E.800: “Definitions of terms related to quality of service”

[3] ITU-T E.807: “Definitions, associated measurement methods and guidance targets of user-centric parameters for call handling in cellular mobile voice service”

[4] 3GPP TS 23.700-19 V1.2.0: “Study on Integration of satellite components in the 5G architecture; Phase 4”

[5] S2-2509941: “Discussion on GEO voice signalling”, SA2#172

[6] S2-2510964: “Interim Conclusion for KI#2”, SA2#172

[7] ITU-T E.771: “Network grade of service parameters and target values for circuit-switched public land mobile services”

[8] ITU-T E.774: “Network grade of service parameters and target values for maritime and aeronautical mobile services”