3GPP TSG SA WG 1 Meeting #113 	S1-261030

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	China Mobile

pCR Title:	Discussion of localized network and 5G PALS

Document for:	Discussion

Agenda item:	8.1.8

Contact:	Huazhang Lv (lvhuazhang@chinamobile.com)

Xiaonan Shi (shixiaonan@chinamobile.com)

Abstract: Discuss the differences between localized network and 5G PALS.

# 1. Introduction

Some of the company considers that there exists some of the similarity between 6G localized network and 5G PALS. This paper proposes to give the analysis of the differences between 5G PALS and 6G localized network, and try to give a way forward.

2. Discussion

Table 14.1.1-2: Enhancements to legacy services and capabilities

| Localized network PR | 5G PALS | Comment |
| --- | --- | --- |
| [PR 5.9.6.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | The 5G system shall support means for the service provider to request the hosting network via standard mechanisms to provide access to 3rd party services at a specific period of time and location. This period of time shall be flexible, so that a change in service provision can be decided at any time (e.g., to cancel or prolong local services in the locality of service delivery) based on localized services agreements. The 5G system shall enable a UE to use credentials provided by the hosting network with or without coordination with the home network of the UE, to make use of localized services via the hosting network with a certain time (including starting time and the duration) and location validity. | Observation from CMCC: 5G PALS has the similar wording of requirements, but still need hosting network to support that. Not mentioned stop a service. |
| [PR 5.9.6.6-2] Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | The 5G system shall support means for the service provider to request the hosting network via standard mechanisms to provide access to 3rd party services at a specific period of time and location. This period of time shall be flexible, so that a change in service provision can be decided at any time (e.g., to cancel or prolong local services in the locality of service delivery) based on localized services agreements. Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support: - access to the hosting network and use home network services or selected localized services via the hosting network, - seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network. | Observation from CMCC: 5G PALS needs the interaction between hosting network and home operator. But 6G localized network is also deployed by the same operator, compared to home operator. 5G PALS and 6G localized network are all indicate the service continuity. But, 5G PALS is the mobility between two hosting networks, or between hosting network and home network. But 6G localized network refers to that a UE moves between localized network and PLMN. |

Table Y.1.14-2 – Localized network

| Localized network PR | 5G PALS | Comment |
| --- | --- | --- |
| Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party). | The 5G system shall be able to authenticate and authorize the UE of a user authenticated to a hosting network to access the hosting network and its localized services on request of a service provider. | Observation from CMCC: Similar requirements, but 6G localized network and perform the authorization/authentication locally, without interaction with PLMN. |
| Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. NOTE: The realization or deployment of localized network may be an enhancement of NPN, PALS or other network topology options. | Subject to localized services agreements, the 5G system shall enable a home network operator to authorize a UE for using its home network services via a hosting network for a certain period of time and/or location. | Observation from CMCC: Similar requirements, but 5G PALS is two network interaction, 6G localized network is the same PLMN and not interactions are needed. 6G localized network can be seen as the enhancement, not totally replacement of 5G PALS, slicing. |
| Subject to regulatory requirements and the operators’ policy, the 6G system shall provide the possibility of local authorization for a new device or UE joining the local network. | Subject to localized services agreements, the 5G system shall enable a home network operator to authorize a UE for using its home network services via a hosting network for a certain period of time and/or location. | Observation from CMCC: The same as above, new requirements that localized network can perform local authorization. |
| The 6G network shall support a mechanism to allow the operator or authorized 3rd party to perform local control of the localized network when the localized network loses connection with the PLMN network. | No related requirements | Observation from CMCC: 5G PALS doesn’t refer to the cases that how to localized network behave if loss connection to home operator. This is new PR compared to 5G PALS, can be seen as the enhancement. |
| The 6G network shall support a mechanism to allow the operator or authorized 3rd party to provide connectivity to a UE in a local network when the local network loses connection with the UE’s home PLMN network. | No related requirements | Observation from CMCC: 5G PALS doesn’t refer to the cases that how to localized network behave if loss connection to home operator. This is new PR compared to 5G PALS, can be seen as the enhancement. |
| Subject to regulatory requirements and the operators’ policy, 6G system shall be able to support (partial-)autonomous local networks for the communication between UEs and devices. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | The 5G system shall enable the home network to allow a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g., predefined time, location) are fulfilled. | Observation from CMCC: 5G PALS clarifies the network selection, but localized network focuses on how to maintain the communication connection. |
| Subject to regulatory requirements and the operators’ policy, 6G network shall be able to support communication between 3GPP UEs in a local network. NOTE: In the requirements above, management of local network can be done locally in case of the local network loses connectivity with the PLMN network, e.g. based on PLMN configuration. | No related requirements | Observation from CMCC: Communication between UEs in localized network, seems the similar requirements in PIN or 5G LAN, but still in PLMN and via 3GPP access. |
| Subject to regulatory requirements and operator’s policy, the 6G network shall allow the operator to provide the required control information including configurations to the local network that enables it to provide necessary configurations for the local communication and to operate also out of coverage of the mobile network. | Subject to operator’s policy and agreement between a 3rd party service provider and operator, the 5G system shall enable a UE to receive and use configuration provided by a 3rd party service provider to discover and access a hosting network and localized services, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network. If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network. If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection. | Observation from CMCC: Similar requirements, 5G PALS focuses on access, and 6G localized network focuses on communication. 5G PALS doesn’t refer to out of coverage. |
| Subject to regulatory requirements and the operators’ policy, 6G network shall allow the operator to provide the required control and configuration information to support (partial-)autonomous local networks for the communication between UEs and devices also out of coverage of the 6G network. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | Subject to operator’s policy and agreement between a 3rd party service provider and operator, the 5G system shall enable a UE to receive and use configuration provided by a 3rd party service provider to discover and access a hosting network and localized services, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network. If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network. If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection. | Observation from CMCC: Similar requirements as above, but localized network still enhances the communication between UE and device. 5G PALS doesn’t refer to out of coverage. |
| Subject to operator’s policy, the 6G network shall support means to enable communication between UEs and a service hosting environment in a local 6G network, managed by an authorized 3rd party, when the connection with the remote (non-local) network fails. NOTE 1: The local 6G network is composed of at least 6G RAN and core network of 6G which are deployed in the local area e.g. factory. NOTE 2: The above assumes agreement with the 3rd party and control by the core network of 6G system |  | Observation from CMCC: The similar requirements as above, focus on network fails. |
| Subject to operator’s policy, the 6G network shall support means to enable communication between UEs and a service hosting environment in a local network, managed by an authorized 3rd party, when the connection with the UE’s PLMN network fails. NOTE 1: The local network is composed of at least RAN and core network which are deployed in the local area e.g. factory. NOTE 2: The above assumes agreement with the 3rd party and control by the PLMN core network |  | Observation from CMCC: The similar requirements as above, focus on connection loss, can be all merged into how to behave if loss connection. |
| The 6G system shall support a mechanism to allow the UE to discover and select the services provided by a localized network. | The 5G system shall support secure means for a UE to select and access localized services which may be provided by a 3rd party service provider via a hosting network, independent of prior subscription to the hosting network or 3rd party service provider. | Observation from CMCC: The similar requirements as 5G PALS, but in 5G PALS needs home operator to support. |

3. Conclusion and proposal

Based on the above observations, we have the following proposal:

Proposal 1: 6G localized network can be seen as the enhancement or evolution of such as 5G PALS, slice, edge computing, NPN. And do not use 6G localized network to replace all of the 5G related technology. And, the realization or deployment of localized network may be an enhancement of NPN, PALS or other network topology options.

Proposal 2: 6G localized network has the following enhancement compared to 5G PALS:

1. The operator PLMN network and the 6G localized network belongs to the same PLMN.

2. 6G localized network is still workable, for example including UE-device communication, UE to localized network communication and local control or authorization, when loss connection to PLMN network or the PLMN network fails. The 6G localized network is more isolated to PLMN network and can perform local control.

3. 5G PALS and 6G localized network are all indicate the service continuity. But, 5G PALS is the mobility between two hosting networks, or between hosting network and home network. But 6G localized network refers to that a UE moves between localized network and PLMN.

4. 5G PALS authorizes the UE with assistance of home network operator, but the 6G localized network can perform authorization/authentication locally.