| 3GPP TSG-SA WG1 Meeting #113 S1-261033 Goa, India, 9 – 13 February, 2026 (revision of S1-26xxxx) CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.215 | CR | 0057 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | × |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new specific service requirements for Non-UAS-capable UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the core technical features for networked UAS within the 3GPP framework are primarily tailored to UAS-capable UE, whereas there is a clear gap in technical support for non-UAS-capable UE. Therefore, it is imperative to carry out targeted research on the requirements of non-UAS-capable UE and advance the relevant technical standardization process, so as to enhance the end-to-end support capability of the 3GPP system for cooperative networked UAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the additional requirement for Non-UAS-capable UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UAS with non-UAS-capable UE cannot access 3GPP network correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=============== First Change =============

5.1	General

[R-5.1-001] The 3GPP system should enable UTM to associate the UAV and UAV controller, and the UTM to identify them as a UAS.

[R-5.1-002] The 3GPP system shall be able to provide UTM with the identity/identities of a UAS.

[R-5.1-003] The 3GPP system shall enable a UAS to send UTM the UAV data which can contain: unique identity (this may be a 3GPP identity), UE capability of the UAV, make & model, serial number, take-off weight, position, owner identity, owner address, owner contact details, owner certification, take-off location, mission type, route data, operating status.

[R-5.1-004] The 3GPP system shall enable a UAS to send UTM the UAV controller data which can contain: unique identity (this may be a 3GPP identity), UE capability of the UAV controller, position, owner identity, owner address, owner contact details, owner certification, UAV operator identity, UAV operator license, UAV operator certification, UAV pilot identity, UAV pilot license, UAV pilot certification and flight plan.

[R-5.1-005] The 3GPP system shall enable a UAS to send different UAS data to UTM based on the different authentication and authorizations level which are applied to the UAS.

NOTE 0: 	Subject to the regional regulation, the different authentication and authorization levels can be: the initial network access authentication and authorization, UAS identity authentication, UAV flight plan authorization, additional UTM service authentications, such as flight monitoring, collision avoidance services, so on.

[R-5.1-006] The 3GPP system shall support capability to extend UAS data being sent to UTM with the evolution of UTM and its support applications in future.

[R-5.1-007] Based on regulations and security protection, the 3GPP system shall enable a UAS to send UTM the identifiers which can be: IMEI, MSISDN, or IMSI, or IP address.

[R-5.1-008] Void

[R-5.1-009] The 3GPP system should enable an MNO to augment the data sent to a UTM with the following: network-based positioning information of UAV and UAV controller.

NOTE 1: 	This augmentation may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information, such as OTDOA, cell coordinates, synchronization source, etc.

NOTE 2: 	This requirement will not be applied to the case which the UAS and UTM has direct control communication connection without going through MNO, such as OTDOA, cell coordinates, synchronization source, etc.

[R-5.1-010] The 3GPP system shall enable UTM to inform an MNO of the outcome of an authorisation to operate.

[R-5.1-011] The 3GPP system shall enable an MNO to allow a UAS authorisation request only if appropriate subscription information is present.

[R-5.1-012] The 3GPP system shall enable a UAS to update a UTM with the live location information of a UAV and its UAV controller.

[R-5.1-013] The 3GPP network should be able to provide supplement location information of UAV and its controller to a UTM.

NOTE 3: 	This supplement may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information.

[R-5.1-014] The 3GPP network shall support UAVs and the corresponding UAV controller are connecting to different PLMNs at the same time.

[R-5.1-014a] The 3GPP system shall support UAVs and the corresponding UAV controller are connecting to different PLMNs at the same time.

[R-5.1-015] The 3GPP system shall provide the capability for network to obtain the UAS information regarding its support of 3GPP communication capabilities designed for UAS operation.

[R-5.1-016] The 3GPP system shall support the UAS identification and subscription data which can differentiate the UAS with UAS-capable UE and the UAS with non-UAS-capable UE.

NOTE 4: 	UAS-capable UE refers to the UE which support interaction capability with UTM and certain 3GPP communication features which 3GPP provides for UAS.

[R-5.1-017] The 3GPP system shall support the UTM in detection of UAV operating without authorization.

NOTE 5: the scenarios covered by the requirement above are FFS.

[R-5.1-018] The 5G system shall be able to detect that a connected UE is airborne, when the UE doesn’t have an aerial subscription.

[R-5.1-019] The 5G system shall be able to support a mechanism to enable a network operator to track an airborne connected UE which doesn’t have an aerial subscription.

[R-5.1-020] Based on operator and UTM policy, the 5G system shall be able to provide UTM with an airborne UE’s location and 3GPP identity to fulfil the UTM’s request.

NOTE 6: The 3GPP identity and UE location is expected to be used by the UTM to determine the position of airborne objects containing the UE.

[R-5.1-021] Based on operator and UTM policy, the 3GPP system shall support a mechanism to enable UTM to perform authentication and authorization with the UAS with non-UAS-capable UE.

NOTE 7: Non-UAS-capable UE refers to the UE which have aerial subscription and  support interaction capability with UTM，but lack of certain 3GPP communication features which 3GPP provides for UAS.

=============== End of Change =============