3GPP TSG SA WG 1 Meeting #113 	S1-261045

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.14-1 UAV, UAM and aircraft

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.8

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254410. This table was discussed during SA1 #112. Yellow highlight refers to CPRs still under discussion, green highlight refers to consensus achieved during discussion.

Based on the outcome of discussion in SA1 #112 Ad hoc meeting, the following wording is changed:

User consent – subscriber permission

Based on operator policy – subject to operator’s policy

This pCR proposes to update Table 14.1.14-1 (UAV, UAM and aircraft) in TR 22.870 with CPRs for inclusion into the draft TR.

* * * First Change (All New CPRs)  * * * *

Table 14.1.14-1 – UAV, UAM and aircraft

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.14-1-1 | 6G system with satellite access shall provide a means to obtain and expose communication service interruption information (e.g. expected interruptions to the communication service) to authorized third party. | PR 11.5.6-2 | QC:[should go under Satellite CPRs] exposure |
| CPR 14.1.14-1-2 | Subject to regulatory requirements and subscriber permission , the 6G network shall provide secure means to expose to an authorized 3rd party application the information related to a UAV or a UAM aircraft from the stored service data. | PR 11.7.6-2 | Exposure ZTE: Is the service data system data? move to data CPRs |
| Alternative CPR 14.1.14-1-2 | Subject to regulatory requirements and subscriber permission , the 6G network shall provide secure means to store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry) and expose the information to an authorized 3rd party application. NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-2 PR 11.7.6-1 | Store and Exposure ZTE: Is the service data system data? move to data CPRs |
| CPR 14.1.14-1-3 | Subject to regulatory requirements and subscriber permission , the 6G network shall be able to securely store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry). NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-1 | Data storage ZTE: Is the service data system data? Prefer to move to data CPRs |