3GPP TSG SA WG 1 Meeting #113 	S1-261047

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.14-3 Mission critical services

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.8

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254410. This table was discussed during SA1 #112. Yellow highlight refers to CPRs still under discussion, green highlight refers to consensus achieved during discussion.

Based on the outcome of discussion in SA1 #112 Ad hoc meeting, the following wording is changed:

User consent – subscriber permission

Based on operator policy – subject to operator’s policy

This pCR proposes to update Table 14.1.14-3 (Mission critical services) in TR 22.870 with CPRs for inclusion into the draft TR.

* * * First Change (All New CPRs) * * * *

Table 14.1.14-3 – Mission critical services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.14-3-1 | Subject to regulatory requirements and operator’s policy, the 6G system shall support MCX service continuity during UE mobility across inter-PLMN, as well as between PLMN and PSBN, (e.g. based on agreed QoS, priority and/orpre-emption settings). NOTE 1: PSBN is a network generally owned and operated by a public safety network operator and specially designed for public safety users (e.g. police, ambulance, fire service), having their own PLMN identity. | PR 11.14.6-1 |  |
| CPR 14.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g. terrestrial access and satellite access), and/or when MCX service switch between on-network and off-network operations with or without UE relay through UAVs, HAPS, and Satellites access. NOTE 2: The requirement above may include also ProSe-enabled access, where relay UEs are equipped with co-located routing functionality, capable of maintaining connectivity in dynamic and infrastructure-less environments. | PR 11.14.6-2 |  |
| Alternative CPR 14.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g. terrestrial access and satellite access), and/or when MCX service switches between on-network and off-network operations, with or without UE relay. NOTE 2: The requirement above may include also ProSe-enabled access, where relay UEs are capable of maintaining connectivity in infrastructure-less environments. |  | Mobility should be a separate requirement |
| CPR 14.1.14-3-3 | The 6G system shall enable an MCX UE to determine its position using multiple positioning methods, including network-assisted GNSS via satellite access, and to convey its position information to the MCX server, triggered by an event, or periodically. | PR 11.14.6-3 |  |
| CPR 14.1.14-3-4 | The 6G system shall enable the MCPTT Service to interwork with non-3GPP PTT Systems that are compliant with the PDT standards, in alignment with P25 and TETRA. | PR 11.14.6-4 |  |
| CPR 14.1.14-3-5 | The 6G system shall support interworking between the MCData SDS and PDT Short Data Service, in alignment with TETRA. | PR 11.14.6-5 |  |