3GPP TSG SA WG 1 Meeting #113 	S1-261052

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Title:	Discussion on CPRs for Ubiquitous Connectivity

Agenda Item:	8.1.5

Source:	CATT, China Telecom

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution discusses the possible consolidated potential requirements for Clause 14.1.11.

1. Introduction

The temporary results of the consolidation work in SA1#112(Dallas) were captured in S1-254410. Besides, there are additional 6 PRs(as below) to be considered in the next consolidation drafting.

- PR8.17.6-1 and PR8.17.6.2 for satellite backhaul.

- PR8.19.6-1 for HAPS-based communication

- PR 8.20-1 and PR 8.20-2 for communication between cooperating UEs.

- PR 8.21.6-1 for satellite access

Based on the agreed table framework and associated scope in SA1#112-adhoc, all the functional PRs of clause 8 are proposed to be categorized and consolidated for CPRs considering the relevant services or capabilities as below.

The functional PRs in Table 14.1.11-1 Satellite-based communication:

- Satellite access relevant: PRs from UC 8.3, UC8.4, UC8.5, UC 8.6, UC8.8, UC8.18;

PR8.9.6-3/4 from UC8.9

- Satellite backhaul relevant: PRs from UC8.12, UC8.17(not in S1-254410)

The following functional PRs in Table 14.1.11-2 Satellite-based positioning:

- General: PR8.3.6-4, PRs from UC8.16.

- Positioning service relevant: PRs from UC8.7, UC8.10, UC8.11

- Timing relevant: PRs from UC8.14

The following functional PRs in Table 14.1.11-3 Other aspects:

- HAPS-based communication relevant: PRs from U8.13, UC8.19(not in S1-254410)

- Satellite onboard computing relevant：PR.8.9.6-1, PRs from UC8.15

- Cooperating UEs: PRs from UC8.20(not in S1-254410)

- Inter-PLMN coordination: UC8.21(not in S1-254410)

The detailed PR assignment can be shown in the following table.

| PR No. | Table14.11.1-1 | Table14.11.1-2 | Table14.11.1-3 | KPI(clause 14.2.3) |
| --- | --- | --- | --- | --- |
| 8.2.6-1 |  |  |  | x |
| 8.3.6-1 | x |  |  |  |
| 8.3.6-2 | x |  |  |  |
| 8.3.6-3 | x |  |  |  |
| 8.3.6-4 |  | x |  |  |
| 8.4.6-1 | x |  |  |  |
| 8.4.6-2 |  |  |  | x |
| 8.4.6-3 | x |  |  |  |
| 8.5.6-1 |  | x |  | x |
| 8.5.6-2 |  | x |  |  |
| 8.6.6-1 |  |  |  | x |
| 8.6.6-2 |  |  |  | x |
| 8.6.6-3 | x |  |  |  |
| 8.7.6-1 |  | x |  |  |
| 8.7.6-2 |  |  |  | x |
| 8.8.6-1 | x |  |  |  |
| 8.8.6-2 |  |  |  | x |
| 8.9.6-1 |  |  | x [SAT Comp] |  |
| 8.9.6-2 |  |  |  | x |
| 8.9.6-3 | x |  |  |  |
| 8.9.6-4 | x |  |  |  |
| 8.10.6-1 |  |  |  | x |
| 8.11.6-1 |  | x |  |  |
| 8.11.6-2 |  |  |  | x |
| 8.12.6-1 | x[SAT BH] |  |  |  |
| 8.12.6-2 | x[SAT BH] |  |  |  |
| 8.13.6-1 |  |  | x [HAPS] |  |
| 8.14.6-1 |  | x[Timing] |  |  |
| 8.15.6-1 |  |  | x [SAT Comp] |  |
| 8.15.6-2 |  |  | x [SAT Comp] |  |
| 8.16.6-1 |  | x |  |  |
| 8.17.6-1 | x[SAT BH] |  |  |  |
| 8.17.6-2 | x[SAT BH] |  |  |  |
| 8.18.6-1 | x |  |  |  |
| 8.19.6-1 |  |  | x [HAPS] |  |
| 8.20.6-1 |  |  | x[Coop UE] |  |
| 8.20-6-2 |  |  | x[Coop UE] |  |
| 8.21-6-1 |  |  | resilience [inter-PLMN] |  |

2. Discussion (with markups)

Taking the pCR proposal from the Rapporteur(S1-261xxx) as a baseline, the grouping and wording suggestion for CPRs are illustrated with the revision markups as below.

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimise the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| CPR 14.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support bi-directional text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 PR8.6.6-3 | [Satellite access] PWS Text Messaging CATT: NEW. merge PR8.6.6.3 some wording change. |
| CPR 14.1.11-1-5 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | [Satellite backhaul] Satellite backhaul links selection/switching Should be moved out of this table (about SAT access), e.g. can go in “others” [CATT: Keep in this table. Satellite backhaul is also satellite-based communication.] |
| CPR 14.1.11-1-8 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to share the data(provided by Service Hosting Environment on aboard satellite) among multiple UEs (e.g. UAV) using satellite access. NOTE: The shared data can be the processed results based on non-3GPP sensing data from the UEs (e.g. UAV). | PR 8.9.6-3 | [Satellite access] Data sharing between UEs CATT: NEW: prefer to leave it here because SHE onboard satellite has been included in TS22.261. Extend to generic data sharing between UEs via satellite access. In both Table 14.1.11-1: Satellite-based communication and Table 14.1.11-3: Other aspects (ubiquitous). Resolve to which table these PRs/CPRs belong |
| CPR 14.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | [Satellite access] Routing modification In both Table 14.1.11-1: Satellite-based communication and Table 14.1.11-3: Other aspects (ubiquitous). Resolve to which table these PRs/CPRs belong |
| CPR 14.1.11-1-10 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, | PR 8.18.6-1 | QoS Improvement |
|  |  |  | CATT: NEW merged to CPR14.1.11-2 for the messaging services |
| CPR 14.1.11-1-NEW (CATT proposed move) | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | [Satellite Backhaul] New connection method Previous EN was resolved |
| NEW CPR b (CATT proposed move) | The 6G system with satellite backhaul links between 6G base station on board of UAV and the 6G CN, shall be able to minimise service interruption when the satellite backhaul link changes. | PR 8.12.6-2 | [Satellite Backhaul] Not included in previous table |
| NEW CPR c (CATT proposed move) | Subject to operator’s policy and the regulatory requirements and operator’s policy, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic. NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite). | PR 8.17.6-2 | [Satellite Backhaul] Not included in previous table |

Table 14.1.11-2: Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Alt CPR 14.1.11-2-1 (Qualcomm) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location, based on 3GPP positioning technologies for UE using only satellite access. | PR 8.3.6-4? PR 8.5.6-2 | UE Location Regulatory Support QC: suggest some edits (regulatory reqs is redundant), and prefer to re-merge with 8.5.6.2 CATT: NEW 5G has a general req for regulatory support. The 5G system shall provide 5G positioning services in compliance with regulatory requirements.( TS22.261 6.27.2) If 5G positioning service is still applicable, PR8.3.6-4 is redundant. how much content of PR8.3.6-4 to contribute in this CPR need to be clarified. |
| Orig PR | The 6G system with satellite access shall be able to provide the location service meeting the regulatory requirements for both sparse satellite constellation and the dense satellite constellation. | PR 8.3.6-4 | Provided for info |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location. | PR 8.5.6-2 | Provided for info |
| Orig PR | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | Provided for info In Table 14.1.11-1, this PR was moved to KPI |
| New CPR 14.1.11-2-4 (CATT) | Subject to operator’s policy, regulatory requirements, and subscriber permission, the 6G system with satellite access shall be able to determine the UE location based on 3GPP positioning technologies for UE only using satellite access. | PR 8.5.6-2 | Determine UE location using satellite-based positioning QC: prefer to merge with CPR 2-1 |
|  |  |  | QC:can be removed (since the one below is green (should be the baseline) |
| Alt CPR 14.1.11-2-2 (Qualcomm) | The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices CATT proposal in S1-254121 was marked to be not pursued in S1-254412 |
| Alt CPR 14.1.11-2-3 (Qualcomm) | The 6G system with satellite access shall be able to provide a positioning service supported by single 3GPP satellite positioning technologies or the combination of 3GPP satellite and non-3GPP positioning technologies(e.g. Network-based Assisted GNSS). | PR 8.11.6-1 PR 8.5.6-1 | Support of NTN & GNSS positioning technologies CATT: NEW.Suggest to take QC proposal as the baseline, and merge PR8.5.6-1. Refer to TS22.261 The 5G system shall provide different 5G positioning services, supported by different single and hybrid positioning methods to supply absolute and relative positioning. NOTE 2: hybrid positioning methods include both the combination of 3GPP positioning technologies and the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as, GNSS (e.g. Beidou, Galileo, GPS, Glonass), Network-based Assisted GNSS and High-Accuracy GNSS, Terrestrial Beacon Systems, dead-reckoning sensors (e.g. IMU, barometer), WLAN/Bluetooth-based positioning. |
| CPR 14.1.11-2-4 | The 6G system with terrestrial and/or satellite access shall be able to provide positioning integrity for a positioning service. | PR 8.16.6-1 | Positioning integrity CATT: NEW.2 questions to be clarified: (1) will it be a generic requirement for any 6G system with TN and/or NTN access? (2) satellite-based positioning technologies is amust in this case? |
| CPR 14.1.11-2-5 (Hytera) | The 6G system with satellite access shall enable an MCX UE to use the 6G positioning services to obtain its position, including support for network-assisted GNSS via satellite access, on request, triggered by an event or periodically. | PR 11.13.6-4 | MCX positioning with support of A-GNSS via satellite access [moving from industrial part]. QC: clarify “on request”, by WHO? CATT: The support to non-3GPP positioning method is covered by 5G requirement. If satellite positioning technologies is not the focus, suggest to move it back to Table 14.1.14-3 MCX. |
| Ori PR | The 6G system shall enable an MCX UE to determine its position using hybrid positioning methods, including network-assisted GNSS via satellite access, to improve availability and resiliency, and to convey its position information to the MCX server, triggered by an event, or periodically. | PR 11.13.6-4 | Provided for info |
|  |  |  | CATT: NEW. 5G has requirements: Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE independently of the UE’s GNSS capability. Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. If highlight the satellite positioning method, Suggest to merge with PR8.11.6-1 |
| CPR 14.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization [CATT: Consider to move it to Table14.1.11-2? The timing is based on satellite-based positioning technology.] QC-New: we can live with the CPR as is |

Table 14.1.11-3: Other aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| New CPR Used to be CPR 14.1.11-1-11 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info Moved from Table 14.1.11-1 (Sat based comms) QC-New: I guess you can remove the 2 previous rows (as this was green) |
| Alt CPR 14.1.11-3-b (Qualcomm) | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a Service Hosting Environment onboard satellite(s) to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | PR8.9.6-1 PR8.15.6-1 | Onboard Computing Marked in yellow because of the controversial definition of SHE QC-New: ok with this version, with some edits CATT: NEW prefer to this version, with some wording change. |
| Orig PR | The 6G network using satellite access shall be able to provide the service hosting environment onboard satellite minimising the necessary bandwidth of the inter satellite links and feeder links. | PR 8.15.6-1 | Provided for info [CATT: This PR can be included in Table14.1.11-3. To merge with PR8.9.6-1 or not will be discussed later.] |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 | Provided for info |
| Orig PR | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimise service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | Provided for info |
| Orig PR | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Provided for info |
| Orig PR | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 | Provided for info |
|  |  |  | QC-New: better in enh. Legacy in my view. Made some edits CATT: suggest in enh legacy as it is. |
| New CPR d | Subject to operator’s policy and regulatory requirements, the 6G system shall support mobility between a base station on the ground and a base station on board a HAPS platform connected to the same core network, taking into account network load and the availability of the HAPS platform (e.g. due to varying power between night and day). | PR 8.19.6-1 | Not included in previous table |
| Alt New CPR d (Qualcomm) | Subject to operator’s policy and regulatory requirements, the 6G network shall support mobility between a base station on the ground and a base station onboard a HAPS platform connected to the same core network, taking into account network load and the availability of the HAPS platform (e.g. due to varying power between night and day). | PR 8.19.6-1 | HAPS mobility Not included in previous table QC-New: suggest few edits CATT: NEW, change “on board” to onboard. |
| Alt CPR 14.1.11-1-8 (CATT) | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | HAPS UE-HAPS-UE [CATT: Suggest to move from Table 14.11.1-1, for HAPS based communication] |
| New CPR x | Subject to operator’s policy, regulatory requirements and subscriber permission , the 6G network shall support a means for coordinating cooperating UEs to enhance QoS/QoE and service availability for one or more of the cooperating UEs of communication with the 6G network, compared to the QoS/QoE and service availability of communication by each UE individually. NOTE: The cooperating UEs are all expected to be in coverage of the network and have or are able to have a direct connection with the network for coordination by the network. | PR 8.20.6-1 | CATT: NEW. From improving the coverage, connectivity availability, it is OK to keep it in this table. With some wording change. |
| New CPR y | Subject to operator’s policy regulatory requirements and subscriber permission, the 6G network shall support the addition and removal of a UE to a set of cooperating UEs. | PR8.20.6-2 | CATT: NEW. With some wording change. |

3. Proposal (in clean version)

It’s proposed to accept the proposed suggestions about CPRs based on the above discussion.

-----------------------------------------------------------------------------------------------------------

To facilitate the reading, the clean version of all the three tables are listed for reference.

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimise the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| CPR 14.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support bi-directional text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 PR8.6.6-3 | [Satellite access] PWS Text Messaging CATT: NEW. merge PR8.6.6.3 some wording change. |
| CPR 14.1.11-1-5 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | [Satellite backhaul] Satellite backhaul links selection/switching |
| CPR 14.1.11-1-8 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to share the data(provided by Service Hosting Environment on aboard satellite) among multiple UEs (e.g. UAV) using satellite access. NOTE: The shared data can be the processed results based on non-3GPP sensing data from the UEs (e.g. UAV). | PR 8.9.6-3 | [Satellite access] Data sharing between UEs CATT: NEW: prefer to leave it here because SHE onboard satellite has been included in TS22.261. Extend to generic data sharing between UEs via satellite access. |
| CPR 14.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | [Satellite access] Routing modification |
| CPR 14.1.11-1-10 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, | PR 8.18.6-1 | [Satellite access] QoS Improvement |
| CPR 14.1.11-1-NEW (CATT proposed move) | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | [Satellite Backhaul] New connection method |
| NEW CPR b (CATT proposed move) | The 6G system with satellite backhaul links between 6G base station on board of UAV and the 6G CN, shall be able to minimise service interruption when the satellite backhaul link changes. | PR 8.12.6-2 | [Satellite Backhaul] |
| NEW CPR c (CATT proposed move) | Subject to operator’s policy and the regulatory requirements and operator’s policy, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic. NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite). | PR 8.17.6-2 | [Satellite Backhaul] |

Table 14.1.11-2: Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Alt CPR 14.1.11-2-1 (Qualcomm) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location, based on 3GPP positioning technologies for UE using only satellite access. | PR 8.3.6-4? PR 8.5.6-2 | UE Location Regulatory Support QC: suggest some edits (regulatory reqs is redundant), and prefer to re-merge with 8.5.6.2 CATT: NEW 5G has a general req for regulatory support. The 5G system shall provide 5G positioning services in compliance with regulatory requirements.( TS22.261 6.27.2) If 5G positioning service is still applicable, PR8.3.6-4 is redundant. how much content of PR8.3.6-4 to contribute in this CPR need to be clarified. |
| Alt CPR 14.1.11-2-2 (Qualcomm) | The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| Alt CPR 14.1.11-2-3 (Qualcomm) | The 6G system with satellite access shall be able to provide a positioning service supported by single 3GPP satellite positioning technologies or the combination of 3GPP satellite and non-3GPP positioning technologies(e.g. Network-based Assisted GNSS). | PR 8.11.6-1 PR 8.5.6-1 | Support of NTN & GNSS positioning technologies CATT: NEW.Suggest to take QC proposal as the baseline, and merge PR8.5.6-1. Refer to TS22.261 The 5G system shall provide different 5G positioning services, supported by different single and hybrid positioning methods to supply absolute and relative positioning. NOTE 2: hybrid positioning methods include both the combination of 3GPP positioning technologies and the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as, GNSS (e.g. Beidou, Galileo, GPS, Glonass), Network-based Assisted GNSS and High-Accuracy GNSS, Terrestrial Beacon Systems, dead-reckoning sensors (e.g. IMU, barometer), WLAN/Bluetooth-based positioning. |
| CPR 14.1.11-2-4 | The 6G system with terrestrial and/or satellite access shall be able to provide positioning integrity for a positioning service. | PR 8.16.6-1 | Positioning integrity CATT: NEW.2 questions to be clarified: (1) will it be a generic requirement for any 6G system with TN and/or NTN access? (2) satellite-based positioning technologies is amust in this case? |
| CPR 14.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization [CATT: Consider to move it to Table14.1.11-2? The timing is based on satellite-based positioning technology.] QC-New: we can live with the CPR as is |

Table 14.1.11-3: Other aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| New CPR Used to be CPR 14.1.11-1-11 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info |
| Alt CPR 14.1.11-3-b (Qualcomm) | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a Service Hosting Environment onboard satellite(s) to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | PR8.9.6-1 PR8.15.6-1 | Onboard Computing Marked in yellow because of the controversial definition of SHE QC-New: ok with this version, with some edits CATT: NEW prefer to this version, with some wording change. |
| Alt New CPR d (Qualcomm) | Subject to operator’s policy and regulatory requirements, the 6G network shall support mobility between a base station on the ground and a base station onboard a HAPS platform connected to the same core network, taking into account network load and the availability of the HAPS platform (e.g. due to varying power between night and day). | PR 8.19.6-1 | HAPS mobility QC-New: suggest few edits CATT: NEW, change “on board” to onboard. |
| Alt CPR 14.1.11-1-8 (CATT) | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | HAPS UE-HAPS-UE |
| New CPR x | Subject to operator’s policy, regulatory requirements and subscriber permission , the 6G network shall support a means for coordinating cooperating UEs to enhance QoS/QoE and service availability for one or more of the cooperating UEs of communication with the 6G network, compared to the QoS/QoE and service availability of communication by each UE individually. NOTE: The cooperating UEs are all expected to be in coverage of the network and have or are able to have a direct connection with the network for coordination by the network. | PR 8.20.6-1 | CATT: NEW. From improving the coverage, connectivity availability, it is OK to keep it in this table. With some wording change. |
| New CPR y | Subject to operator’s policy regulatory requirements and subscriber permission, the 6G network shall support the addition and removal of a UE to a set of cooperating UEs. | PR8.20.6-2 | CATT: NEW. With some wording change. |