8.3GPP TSG-SA WG1 Meeting #113 	S1-261053

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	CATT, China Telecom

pCR Title:	pCR on CPRs in Table 14.1.11-1 (Satellite-based communication)

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.5

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes the candidate CPRs in Table 14.1.11-1(satellite-based communication) to facilitate the consolidation work of Ubiquitous Connectivity.

1. Introduction

Regarding the discussion in S1-261052, this contribution provides the candidate CPRs to Table 14.1.11-1 for the consolidation of the identified functional requirements from Clause 8 Ubiquiteous Connectivity.

2. Reason for Change

Provide the candidate CPRs for the consolidation based on the discussed proposal in S1-261xxxx, and adjust the CPR order regarding the technical directions.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

14.1.11	Ubiquitous Connectivity

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimise the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| CPR 14.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support bi-directional text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 PR 8.6.6-3 | [Satellite access] PWS Text Messaging |
| CPR 14.1.11-1-4 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, | PR 8.18.6-1 | [Satellite access] QoS Improvement |
| CPR 14.1.11-1-5 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to share the data(provided by Service Hosting Environment on aboard satellite) among multiple UEs (e.g. UAV) using satellite access. NOTE: The shared data can be the processed results based on non-3GPP sensing data from the UEs (e.g. UAV). | PR 8.9.6-3 | [Satellite access] Data sharing between UEs |
| CPR 14.1.11-1-6 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | [Satellite access] Routing modification |
| CPR 14.1.11-1-7 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | [Satellite backhaul] Satellite backhaul links selection/switching |
| CPR 14.1.11-8 | The 6G system with satellite backhaul links between 6G base station on board of UAV and the 6G CN, shall be able to minimise service interruption when the satellite backhaul link changes. | PR 8.12.6-2 | [Satellite Backhaul] Service continuity |
| CPR 14.1.11-9 | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | [Satellite Backhaul] New connection method |
| CPR 14.1.11-10 | Subject to operator’s policy and the regulatory requirements and operator’s policy, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic. NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite). | PR 8.17.6-2 | [Satellite Backhaul] Connection setup |

* * * End of Changes * * *