3GPP TSG-SA WG1 Meeting #113 	S1-261054

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	CATT, China Telecom

pCR Title:	pCR on CPRs in Table 14.1.11-2 (Satellite-based positioning)

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.5

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes the candidate CPRs in Table 14.1.11-2(satellite-based positioning) to facilitate the consolidation work of Ubiquitous Connectivity.

1. Introduction

Regarding the discussion in S1-261052, this contribution provides the candidate CPRs to Table 14.1.11-2 for the consolidation of the identified functional requirements from Clause 8 Ubiquiteous Connectivity.

2. Reason for Change

Provide the candidate CPRs for the consolidation based on the discussed proposal in S1-261xxxx, and adjust the CPR order regarding the technical directions.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-2: Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-2-1 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location, based on 3GPP positioning technologies for UE using only satellite access. | PR 8.3.6-4 PR 8.5.6-2 | UE Location |
| CPR 14.1.11-2-2 | The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| CPR 14.1.11-2-3 | The 6G system with satellite access shall be able to provide a positioning service supported by single 3GPP satellite positioning technologies or the combination of 3GPP satellite and non-3GPP positioning technologies(e.g. Network-based Assisted GNSS). | PR 8.5.6 -1 PR 8.11.6-1 | Positioning service |
| CPR 14.1.11-2-4 | The 6G system with terrestrial and/or satellite access shall be able to provide positioning integrity for a positioning service. | PR 8.16.6-1 | Positioning integrity |
| CPR 14.1.11-2-5 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |

* * * End of Changes * * *