3GPP TSG-SA WG1 Meeting #113 	S1-261055

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	CATT, China Telecom

pCR Title:	pCR on CPRs in Table 14.1.11-3 (Other aspects)

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.5

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes the candidate CPRs in Table 14.1.11-3(other aspects) to facilitate the consolidation work of Ubiquitous Connectivity.

1. Introduction

Regarding the discussion in S1-261052, this contribution provides the candidate CPRs to Table 14.1.11-3 for the consolidation of the identified functional requirements from Clause 8 Ubiquiteous Connectivity.

2. Reason for Change

Provide the candidate CPRs for the consolidation based on the discussed proposal in S1-261xxxx, and adjust the CPR order regarding the technical directions.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-3: Other aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-3-1 | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a Service Hosting Environment onboard satellite(s) to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 PR 8.15.6-1 | Onboard Computing |
| CPR 14.1.11-3-2 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info |
| CPR 14.1.11-3-3 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | UE-HAPS-UE |
| CPR 14.1.11-3-4 | Subject to operator’s policy and regulatory requirements, the 6G network shall support mobility between a base station on the ground and a base station onboard a HAPS platform connected to the same core network, taking into account network load and the availability of the HAPS platform (e.g. due to varying power between night and day). | PR 8.19.6-1 | HAPS mobility |
| CPR 14.1.11-3-5 | Subject to operator’s policy, regulatory requirements and subscriber permission , the 6G network shall support a means for coordinating cooperating UEs to enhance QoS/QoE and service availability for one or more of the cooperating UEs of communication with the 6G network, compared to the QoS/QoE and service availability of communication by each UE individually. NOTE: The cooperating UEs are all expected to be in coverage of the network and have or are able to have a direct connection with the network for coordination by the network. | PR 8.20.6-1 | Cooperating UE Perf. Improvement |
| CPR 14.1.11-3-6 | Subject to operator’s policy regulatory requirements and subscriber permission, the 6G network shall support the addition and removal of a UE to a set of cooperating UEs. | PR 8.20.6-2 | Cooperating UE UE management |

* * * End of Changes * * *