3GPP TSG SA WG 1 Meeting #113 	S1-261056

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Title:	Discussion on CPRs for Computing

Agenda Item:	8.1.9

Source:	CATT

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution discusses the possible consolidation of potential requirements relevant to computing for Clause 14.1.9.

1. Introduction

Regarding the discussion of #112 ad-hoc meeting, the consolidation of all the potential requirements related to “computing” is expected to be done in one dedicated clause, and the service-specific CPRs will be covered by individual table.

So far excluding satellite onboard computing relevant, 42 computing related PRs have been identified from Clause 6 AI, Clause 9 Immersive Communication, Clause 11 Further Use Cases on Industry and Verticals, and Clause 12 Other Use Cases, and can be categorized into 3 groups:

- General Computing

- Computing aspects related to AI

- Computing aspects related to Immersive communication

Based on the latest pCR proposal from the Rapportuer(S1-261xxx), some merging and wording suggestions will be illustrated for each CPR candidate in details.

2. Discussion

Taking the pCR proposal from the Rapporteur(S1-261043) as a baseline, the tables has been updated with the following changes.

- Add the missing PR6.2.6-1

- Correct the PR number typo (mistake PR12.2.6 from PR6.5.6-2)

- Some grouping and wording suggestions are illustrated with the revision markups in each line.

Table 14.1.9-1 – General Computing requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.9-1-1 | Subject to subscriber permission, the 6G system shall support mechanisms to execute compute tasks in the Service Hosting Environment upon service request from UEs. | PR 6.5.6-1 | General, execute CATT: “execute” seems like “run the task”, then how to run the task could depend on the implementation. It is not clear how to understand it, e.g. including initiate a task upon request? |
| 14.1.9-1-2 | Subject to operator’s policy and subscriber permission, the 6G system shall support a mechanism for providing 6G Computing Service to user (via UE). | PR 12.2.6-1 | General, provisioning |
| 14.1.9-1-3 | Subject to regulatory requirements and subscriber permission, the 6G network shall be able to enable offloading compute tasks to a Service Hosting Environment for third party applications. | PR 9.4.6-1 | General, offloading CATT: “compute offloading” is a term without clear definition in 3GPP. can we just use its native meaning instead? |
| 14.1.9-1-4 | Subject to operator’s policy and subscriber permission , the 6G system shall support mechanisms to enable a UE to specify requirements for a compute task in the Service Hosting Environment(e.g. latency, when to execute a compute task) | PR 6.5.6-2 PR 6.5.6-3 | Proposed merged CPR on Requirements CATT: simplify the examples to remove too many details. |
| - | Subject to user consent, the 6G system shall support mechanisms that enable a UE to specify when a compute task should be executed (e.g. immediately, periodically, etc.) in the Service Hosting Environment. | PR 6.5.6-2 | Requirements |
| - | Subject to user consent, the 6G system shall support mechanisms that enable a UE to specify requirements for a compute task in the Service Hosting Environment, such as an overall latency indicating how fast the results of the compute task should be provided. | PR 6.5.6-3 | Requirements |
| 14.1.9-1-5 | Subject to operator’s policy and subscriber permission, the 6G system shall support translating the 6G Computing Service requirements of a 3rd party service provider (e.g. latency) into compute and communication resources of Service Hosting Environment for providing the subscribed 6G Computing Service. | PR 12.2.6-3 | Requirements |
| 14.1.9-1-6 | The 6G network shall support mechanisms to receive the requested computing requirements of 6G Computing Service. | PR 6.33.6-1 | Requirements CATT : some wording edits |
| 14.1.9-1-7 | The 6G network shall support the collection and monitoring of the status (e.g. computing workload, congestion information, available capability, power consumption) of trusted computing resources in the Service Hosting Environment on-demand or periodically. | PR 6.2.6-3 PR 9.15.6-2 | Proposed merged CPR on Status of computing resources CATT: take PR 9.15.6-1 out of this CPR. PR6.2.6-3 and PR9.15.6-2 are for status monitoring. PR 9.15.6-1 is for computing resource(information) management |
| 14.1.9-1-7a | The 6G network shall support mechanisms to for trusted computing resources (i.e. edge server(s)) to provide and update the capability information on demand. NOTE: The information can be related to computing capabilities (e.g. xPUs, storage), service capabilities (e.g. supported applications, status), and network capabilities (e.g. allowed bandwidth). | PR 9.15.6-1 | Reword CPR to reflect the intention of PR. |
| - | The 6G network shall support detailed monitoring and reporting of computational resource usage in the Service Hosting Environment. | PR 6.2.6-3 | Status of computing resources |
| - | The 6G network shall support mechanisms to acquire and maintain the information of trusted computing resources (i.e. edge server(s)) for coordinating the usage of computing resource on demand NOTE: The information can be related to computing capabilities (e.g. xPUs, storage), service capabilities (e.g. supported applications, status), and network capabilities (e.g. allowed bandwidth). | PR 9.15.6-1 | Status of computing resources |
| - | The 6G network shall support mechanisms to collect status information (e.g. computing workload, congestion information, available capability, power consumption) of trusted computing resources (i.e. edge server(s) or cloud server(s)) on-demand or periodically. | PR 9.15.6-2 | Status of computing resources |
| 14.1.9-1-8 | Subject to operator’s policy, the 6G network shall provide mechanisms to expose to an authorised 3rd party or a UE the information (e.g. computing capability, the location, allowed service types, status, power consumption, utilization, availability) of computing resources in the Service Hosting Environment | PR 6.2.6-1 PR 9.15.6-3 | Status of computing resources exposure CATT: PR6.2.6-1 is missing from original table. Can be merged for information exposure |
|  | Based on operator’s policy, the 6G network shall provide suitable means to allow authorised third parties and/or UEs to retrieve availability information about computational resources (e.g. storage, AI processing units, xPUs information etc.) inside the Service Hosting Environment and to utilise these computational resources for running workloads on demand. | PR 6.2.6-1 |  |
|  | The 6G network shall provide mechanisms to expose to an authorised 3rd party the information (e.g. computing capability, the location, allowed service types, status, power consumption, utilization) of computing resources (i.e. edge server(s) or cloud server(s)). | PR 9.15.6-3 |  |
| 14.1.9-1-9 | Subject to operator’s policy and subscriber permission, 6G network shall provide mechanisms to expose the information related to 6G Computing Service to an authorised 3rd party or a UE (e.g. to track usage patterns of the computing service, to assist a UE to invoke a computing service). | PR 12.1.6-2 PR 12.3.6-2 | Proposed merged CPR on Exposure CATT: it’s OK to merge them into one, or keep it as two CPRs which address different exposure objects. |
| - | The 6G network shall be able to inform UE of the information related to 6G Computing Service in the Service Hosting Environment (e.g. guaranteed computing capabilities) to assist an UE to invoke a computing service. | PR 12.1.6-2 | Exposure |
| - | Subject to operator’s policy and user’s consent, 6G network shall provide mechanism to expose information related to 6G Computing Service to an authorised 3rd party (e.g. to track usage patterns of the computing service for offline AI analysis). | PR 12.3.6-2 | Exposure |
| 14.1.9-1-10 | Subject to operator’s policy, 6G network shall support mechanism to allow sharing of information related to 6G Computing Service within the Service Hosting Environment (e.g. to predict overprovisioning and underutilization of computing resources). | PR 12.3.6-1 | Exposure |
| 14.1.9-1-11 | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall support to select one or more Service Hosting Environment to fulfil the computing service requirements from 3rd party, considering the UE location, computing capabilities and traffic load, etc, and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS. [CATT proposal] Subject to operator’s policy, the 6G system shall support the selection of compute resources in Serviced Hosting Environment for a compute task or a 3GPP service(e.g. 6G computing service, AI service) upon request from 3rd party application(e.g. AI-based application) | PR 6.15.6-1 PR 6.34.6-2 PR 6.14.6-3 PR 6.37.6-3 | Proposed merged CPR on Selection CATT: PR 9.15.6-4 is using 3rd party computing environment, need to be separated first. There will be two CPRs for the rest PRs. 14.1.9-1-11 includes 6.15.6-1, 6.34.6-2, 9.15.6-4, PR 6.14.6-3, PR 6.37.6-3(move from Table14.1.9-2) for the compute resource selection. 14.1.9-1-11a includes 6.53.6-1, 11.22.4-2 for the SHE selection. 3GPP services can cover AI service, computing service, immersive service, etc. The service-specific requirements will impact the selection. |
|  | Subject to operator policy, 6G network shall be able to support selection of computing resources in Service Hosting Environment for 6G Computing Service. | PR 6.15.6-1 | Selection |
|  | Subject to operator policy, the 6G network shall support selection of computing resources in the Service Hosting Environment for offloading task. | PR 6.34.6-2 | Selection |
|  | Subject to the operator’s policy, the 6G network shall support the selection of computing resources in the Service Hosting Environment upon request from AI-based applications. | PR 6.14.6-3 | Selection |
|  | Based on operator's policy and agreement with 3rd party, the 6G network shall support a mechanism to assist in selecting computing resources inside Service Hosting Environment for AI service considering requirement of the 3rd party. | PR 6.37.6-3 | Selection |
| 14.1.9-1-11a | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall support mechanisms to select one or multiple Service Hosting Environment(s) for 3GPP services or 3rd party service to fulfil the requirements from 3rd party. | PR 6.53.6-1 PR 11.22.6-2 | CATT: 14.1.9-1-11a includes 6.53.6-1, 11.22.4-2 for the SHE selection. |
|  | Subject to operator policy and regulatory requirement, the 6G network shall be able to support the selection of multiple Service Hosting Environments for 3GPP services and 3rd party services. | PR 6.53.6-1 | Selection |
|  | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall support to select one or more Service Hosting Environment to fulfil the computing service requirements from 3rd party, considering the UE location, computing capabilities and traffic load, etc. | PR 11.22.6-2 | Selection |
|  | Subject to operator's policy, application needs or both, the 6G network shall support the selection of computing resource(s) (i.e. edge server(s) or cloud server(s)) based on e.g. requested computing capabilities, and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS. | PR 9.15.6-4 | Selection, Routing between UE-computing resources CATT: PR 9.15.6-4 is using 3rd party computing environment, need to be separated |
|  | Subject to operator's policy, application needs or both, the 6G network shall support the selection of computing resource(s) (i.e. edge server(s) or cloud server(s)) based on e.g. requested computing capabilities, and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS. | PR 9.15.6-4 | Selection, Routing between UE-computing resources |
| 14.1.9-1-12 | Subject to operator’s policy and regulatory requirement, the 6G network shall be able to support the coordination amongst multiple Service Hosting Environments for 3GPP services and 3rd party services. | PR 6.53.6-2 | Coordination |
| 14.1.9-1-13 | Subject to operator’s policy, 6G system shall be able to guarantee the overall E2E latency (i.e. latency of communication service, and latency of 6G Computing Service) requested by the application (e.g. target object detection). NOTE: UE is only considered to contribute to the communication service latency. | PR 6.15.6-2 | Performance |
| 14.1.9-1-14 | The 6G network shall be capable of providing appropriate Service Hosting Environment in order to accommodate compute and communication (e.g. traffic load) resources to meet user-requested service requirements (e.g. bitrate and latency) and 6G Computing Service requirements (e.g., route, latency tolerance, computation intensity). [CATT proposal] The 6G network shall be able to meet user-requested performance requirements for communication service (e.g. bitrate and latency) and 6G Computing Service(e.g., route, latency tolerance, computation intensity) e.g. by providing appropriate Service Hosting Environment. | PR 6.24.6-2 PR 12.1.6-1 | Proposed merged CPR on Performance CATT: PR12.1.6-1 seems have a bigger technical scope than PR6.24.6-2. If merge together, some wording edits. Another option is to leave them separately. |
|  | [PR 6.24.6-2] The 6G network shall be capable of providing appropriate Service Hosting Environment in order to accommodate compute and communication (e.g. traffic load) resources to meet communication service requirements (e.g. bitrate and latency) and 6G Computing Service requirements (e.g., route, latency tolerance, computation intensity). | PR 6.24.6-2 | Performance |
|  | Subject to operator’s policy, the 6G network shall be able to fulfil the user-requested performance requirement (e.g. latency) for the 6G Computing Service. | PR 12.1.6-1 | Performance |
| 14.1.9-1-15 | Subject to operator policy, the 6G network shall support efficient ways to transfer a computational task from one computational resource of the Service Hosting Environment at one location to another computational resource of the Service Hosting Environment at another location, or support reselection of computing resources in Service Hosting Environment to support service continuity due to, e.g.: UE mobility [CATT proposal] Subject to operator policy, the 6G network shall support mechanisms to reselect computing resources within Service Hosting Environment(s) and migrate the ongoing compute task(s) between the selected compute resources to maintain user experience e.g. during UE mobility, computing overloaded. | PR 6.2.6-4 PR 9.15.6-6 PR 6.19.6-1 PR 6.33.6-2 | Proposed merged CPR on Performance, Service continuity CATT: some wording edits. |
|  | Based on operator policy, the 6G network shall support efficient ways to transfer a computational task from one computational resource of the Service Hosting Environment at one location to another computational resource of the Service Hosting Environment at another location, based on the mobility of UEs and the desired performance requirements. | PR 6.2.6-4 | Performance, Service continuity |
|  | The 6G network shall support reselection of computing resources in Service Hosting Environment for third party application workload offloading from the UE to Service Hosting Environment (e.g. where AI inference is performed based on video input from UE), considering UE mobility. | PR 6.19.6-1 | Performance, Service continuity |
|  | Subject to operator's policy, application needs or both, the 6G network shall support the reselection of computing resource(s) inside Service Hosting Environment and support the continuity due to, e.g.: UE mobility. | PR 6.33.6-2 | Reselection |
|  |  |  | Proposed merged CPR on Performance, Service continuity CATT: PR9.2.6.4 and PR9.15.6-6 are for computing task migration. Suggest to merge into CPR14.1.9-1-15. PR 9.15.6-5 and PR9.6.6-2 are about service continuity of immersive service. Move PR9.15.6-5 to CPR14.1.9-3-4. |
|  | Subject to operator's policy, the 6G network shall be able to support migration of ongoing computing task from one selected computing resource (e.g. overloaded) to other computing resource(s) in Service Hosting Environments in order to maintain user experience. | PR 9.15.6-6 | Performance, Service continuity |
|  | Subject to operator’s policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources switch between those within the Service Hosting Environment and those in 3rd party computing environment. | PR 9.15.6-5 | Performance, Service continuity |
| 14.1.9-1-17 | Subject to operator’s policy and subscriber permission, the 6G network shall be able to prioritize the processing of offloading task request(s) from a high-priority user per service. NOTE: the prioritization of processing of computing is with more stringent requirement from high-priority user | PR 6.34.6-3 | Prioritize the processing of computing task |
| 14.1.9-1-18 | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall provide a mechanism for operators to configure, adjust and manage the computing resource in Service Hosting Environment based on e.g. UE location, computing service requirements from 3rd party. | PR 11.22.6-1 | Resource management |
| 14.1.9-1-19 | Subject to operator’s policy, the 6G system shall support mechanisms to ensure security and privacy when providing 6G Computing Service. | PR 12.2.6-2 | Security CATT: correct PR number. |
| 14.1.9-1-20 | The 6G network shall be able to collect energy related data of the Service Hosting Environment. | PR 6.24.6-1 | Energy consumption CATT: move this PR to energy efficiency part Table 14.1.4-1? |
| 14.1.9-1-21 | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 PR 8.15.6-1 | Computing service provided via NTN (pending to be here or in Y.1.11) CATT: Prefer to discuss it in Table14.1.11-3 |
| 14.1.9-1-22 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Computing service provided via NTN (pending to be here or in Y.1.11) CATT: Prefer to discuss it in Table14.1.11-3 |

Table 14.1.9-2 – Computing aspects related to AI

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.9-2-1 | The 6G network shall be able to provide computing resource in the Service Hosting Environment for AI services. | PR 6.3.6-1 | General, resource |
| 14.1.9-2-2 | Subject to operator’s policy and subscriber permission, the 6G network shall support mechanisms for scheduling and provisioning network resources and computing resources in Service Host Environment for AI services (e.g. distributed AI inference). | PR 6.10.6-4 | Scheduling and provisioning CATT: some wording edits. |
| 14.1.9-2-3 | The 6G network shall be able to manage and coordinate various AI tasks offloading into Service Hosting Environment(s) e.g. considering AI workload. | PR 6.31.6-1 | AI offloading CATT: some wording edits. |
| 14.1.9-2-4 | Subject to operator’s policy, the 6G network shall be able to take into account third party’s requirement(s) (e.g. latency) and the computing resource availability for the AI/ML model training and inference within Service Hosting Environment. | PR 6.25.6-2 | Requirements CATT: editorial change. |
| 14.1.9-2-5 | Subject to operator's policy and agreement with 3rd party, the 6G network shall support the selection computing resources inside Service Hosting Environment upon request from AI-based applications and requirement of the 3rd party for AI service. |  | Proposed merged CPR for Selection CATT: suggest to merge to CPR14.1.9-1-11. The reason refers to the above. |
|  | Subject to the operator’s policy, the 6G network shall support the selection of computing resources in the Service Hosting Environment upon request from AI-based applications. | PR 6.14.6-3 | Selection |
|  | Based on operator's policy and agreement with 3rd party, the 6G network shall support a mechanism to assist in selecting computing resources inside Service Hosting Environment for AI service considering requirement of the 3rd party. | PR 6.37.6-3 | Selection |
| 14.1.9-2-6 | Subject to operator’s policy, the 6G network (e.g. core network) shall support hosting of e.g. an AI/ML model in the Service Hosting Environment e.g. based on latency, transport load or data privacy requirements. | PR 9.5.6-3 | Host model CATT: some wording edits. |

Table 14.1.9-3 – Computing aspects related to Immersive communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.9-3-1 | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to provide 6G Computing Service controlled by the core network of the 6G system to an authorized third-party for rendering. | PR 9.5.6-1 PR 9.6.6-1 | Proposed merged CPR on General, rendering |
|  | Subject to operator policy, the 6G network (e.g. core network) shall enable the support of computing tasks in the Service Hosting Environment to render 6DoF video and spatial audio. | PR 9.5.6-1 | General, rendering |
|  | Subject to operator’s policy and regulation, the 6G network shall be able to provide 6G Computing Service controlled by the core network of 6G system to an authorized third-party for supporting rendering. | PR 9.6.6-1 | General, rendering |
| 14.1.9-3-2 | Subject to operator’s policy, the 6G network (e.g. core network) shall enable the offloading of computing tasks to the Service Hosting Environment to render 6DoF video and spatial audio to a 3rd party. | PR 9.5.6-2 | General, offloading |
| 14.1.9-3-3 | Subject to subscriber permission, the core network of the 6G system shall provide means to coordinate with a UE to make decisions on splitting/offloading rendering tasks dynamically based on the real time network status and the computing resource availability in the Service Hosting Environment, edge or central cloud. NOTE 1: In the case of tethered XR headset connected to a UE, the latency, computing resource consumption and power consumption (incurred by application processing and communication processing) of both the UE and the XR Glasses need to be also considered. NOTE 2: It is assumed that the core network is able to obtain the information of computing resource availability in the Service Hosting Environment, edge or central cloud, which enables the core network to locate appropriate computing resources for the rendering tasks. | PR 9.3.6-2 | Coordination with UE, immersive |
| 14.1.9-3-4 | Subject to operator’s policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources changes within the Service Hosting Environment or between between the Service Hosting Environment and 3rd party computing environment used for the 6G Computing Service for XR rendering e.g. during UE mobility. | PR 9.6.6-2 PR 9.15.6-5 | Performance, Service continuity CATT: the two address different sources of computing resources |

3. Proposal

It’s proposed to accept the change suggestions as the above illustration in the final CPR proposals.