3GPP TSG-SA WG1 Meeting #113		S1-261075

09-13 February 2026, Goa, India	(revision of S1-254314, S1-254243, S1-254031)

# Source:	Huawei

# Title:	New Study on Wireless Power Transfer

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Wireless Power Transfer

Acronym: FS_WPT

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-21

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

Power consumption has always been the key problem for IoT devices. Considering the rise of factory management due to labour and device cost for battery changing, deployed base station could act as steady source of energy via electromagnetic radiation of wireless signals to support operation of those devices, i.e. 5G base station provides a wireless power transfer service.

One example to consider are the instruments/sensors deployed in a chemical factory to monitor the processing procedures. Currently, these instruments/sensors are typically deployed with wired power or equipped with a battery. For the wired power, deployment usually involves labour cost, plastic pipe cost, construction of shallow trench, etc., which leads to high deployment complexity. Wired power requires fixed positioning of the device and is subject to ongoing manual maintenance such as wire replacement & repair. On the other hand, for the case of battery equipment, periodic or temporary battery replacement (every two/three years) involves high management cost due to manual maintenance. Under some hazardous areas/core production areas, both wired deployment or battery replacement are even impossible due to inaccessibility for humans. Considering the 3GPP Release 19 work on Ambient IoT, Ambient energy harvesting could be used to power sensors but this is dependent on the external environment which cannot reliably provide a steady supply of energy.

Therefore, it is important to consider a reliable method of wireless power transfer that can been the demands of predictable and on-demand power delivery.  As the delivered power available at the device will decrease with range from the transmitter, different use cases and device types may be supported at different distances. This might correlate with different deployment scenarios such as lower inter-site distances (see figure 1). Due to the decrease of delivered power with range, preferred scenarios are where the devices are either at a close range to a power transmitter (e.g. Indoor environments where power is delivered from indoor base stations), or where the power delivery network is closely designed (e.g. for vertical use cases in a defined area such as a factory or refinery). Other scenarios such as those related to outdoor macro coverage are less preferred due to the variability in delivered power at increased range.

Figure 1: Delivered power vs range and associated use cases

# 4	Objective

The objectives are to study new use cases and potential service and performance requirements to support the 5G wireless power transfer service primarily for indoor and vertical scenarios, including:

- 5G wireless power transfer performance considering deployment scenarios including, environment, and service range (e.g. inter-site distance);

- supported devices and expected performance requirements;

- 5G wireless power transfer service management and control etc.

NOTE: any networks delivering the wireless power transfer service are expected to comply with all relevant local regulations in this activity, including spectrum management regulations such as maximum transmission power.

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.xxx | Study on 5G wireless power transfer service | TSG#113 (Sep 2026) | TSG#114 (Dec 2026) | Eddy Hall, Huawei (edward.hall@huawei.com) |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Eddy Hall, Huawei

(edward.hall@huawei.com)

# 7	Work item leadership

SA WG1

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Turk Telecom

BUPT

EDF

Philips International B.V.

KPN

TNO

ZTE