3GPP TSG-SA WG1 Meeting #113 	S1-261082

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Title:	Comments on Immersive Communication related to IMS CPRs

Agenda Item:	8.1.6

Source:	Qualcomm

Contact:	Lola Awoniyi-Oteri (oawoniyi@qti.qualcomm.com)

# 1. Introduction

Based on the latest CPR proposals from the Rapportuer draft S1-261xxx Table 14.1.12-2 Immersive IMS . docx) shared through the reflector. Qualcomm presents in this contribution some proposals for further merging and refinement. Details are presented in tables Table 14.1.12-2, below.

Table 14.1.12-2: Immersive communication related to IMS

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.12-2-1 | Subject to operator’s policy, the 6G system including IMS shall support the synchronization of independent traffic flows of one or more applications, to be delivered to more than one device (i.e. UE or tethered devices). NOTE: The applications whose traffic flows are synchronized are associated. It is assumed that the association is known to the 6G system. | PR 9.7.2-1 | IMS Synchronization of associated independent media flows Delivery to multiple devices Revised as part of SA1 112 Ad Hoc-e |
| CPR 14.1.12-2-2 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system (including IMS) shall support intelligent immersive calling service for users via various UEs (e.g. smart wearables, mobile phones, intelligent devices in the home, low-power devices). NOTE 1: Intelligent immersive calling service: An immersive calling service that is empowered by AI capabilities, e.g. generative AI, multi-modal model etc. The service takes various input from different kinds of smart devices and sensors, e.g. camera, smart watches, AR/VR glasses, to enable users to communicate with each other. It can be provided natively by the operators. | PR 9.10.6-1 | IMS Intelligent Immersive Calling Service Revised as part of SA1 112 Ad Hoc-e QC: propose to move to table with other intelligent immersive calling PRs in Table14.1.8-4 |
| CPR 14.1.12-2-3 | Subject to operator’s policy, the 6G system (including IMS) shall provide mechanisms for the intelligent immersive calling service to render media based on the received intent from a user (e.g. voice, gesture) during the calling. NOTE 2: Media rendering could include e.g. switching video input, altering facial expression. | PR 9.10.6-4 | IMS Intelligent Immersive Calling Service Media rendering Intent based Revised as part of SA1 112 Ad Hoc-e QC: propose to move to table with other intelligent immersive calling PRs in Table14.1.8-4 |
| CPR 14.1.12-2-4 | The 6G network and IMS shall support communication between UEs with different capabilities, e.g., that requiring multi-channels video and/or multi-channels voice. | PR 9.16.6-1 | Reception of application info from UE/3rd party App Could also go into Table 14.1.12-1-15 Y.1.12-1-15 [ZTE] It cannot go into Y.1.12-1-15. Let’s keep it as a CPR. |
| CPR 14.1.12-2-5 (S1-254190) | Subject to operator’s policy and regulatory requirement, the intelligent immersive calling service shall comply to the existing regulatory requirements (e.g. lawful interception). | PR 9.10.6-2 | intelligent immersive calling service Revised as part of SA1 112 Ad Hoc-e QC: propose to move to table with other intelligent immersive calling PRs in Table14.1.8-4 |

* * * End of Changes * * *