3GPP TSG-SA WG1 Meeting #113 	draft S1-261099

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.4-1 (Energy-related Aspects) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.4-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254160 (ZTE)

- S1-254250 (Qualcomm)

- November 2025 reflector comments

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Removed PR 5.8.7.6-1 since it in Table 14.1.7-1: NDT

- Removed PR 7.5.6-2 since it is in Table 14.1.10-1: ISAC

- Removed PR 6.16.6-1 since it is in the TR as approved CPR 14.1.8-1-3

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.4-1: Energy-related Aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Alt CPR 14.1.4-1-1 (ZTE) | Subject to regulatory requirements and subscriber permission, the 6G system shall jointly improve the network and UE energy efficiency compared to the 5G system, while meeting the service performance requirements. | PR 5.8.1.6-1 PR 5.8.1.6-4 PR 5.8.4.6-3 | System Energy Efficiency. ZTE: It is suggested to merge the PR 5.8.4.6-3 in this CPR. |
| Orig PR | The 6G system shall improve energy efficiency of 6G system compared to 5G system. | PR 5.8.1.6-1 | Provided for info |
| Orig PR | The 6G system shall provide means to improve the joint network and UE energy efficiency when providing services to the subscribers. | PR 5.8.1.6-4 | Provided for info |
| OR PR | Subject to regulatory requirements and subscriber permission, the 6G system shall be able to optimise network and UE energy saving jointly, while meeting the service performance requirements. | PR 5.8.4.6-3 | Provided for info |
| Orig PR | The 6G network shall be able to support mechanisms to improve UE energy efficiency when providing services to the subscribers. | PR 5.8.1.6-2 | Provided for info |
| Orig PR | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: The 6G network takes into account this preference when improving UE energy efficiency. Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | Provided for info |
| Orig PR | Subject to regulatory requirements and subscriber permission, the 6G system shall be able to optimize the UE energy saving considering the current traffic conditions of the UE. | PR 5.8.4.6-2 | Provided for info Huawei: Suggest keeping the original wording |
| Orig PR | Subject to regulatory requirements and subscriber permission, the 6G system shall support mechanisms to optimize the energy consumption of a UE via enabling task offloading (e.g. XR rendering/AI inference) from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE. | PR 5.8.5.6-1 | Provided for info Huawei: Suggest keeping the original wording after aligning the handling on SHE. E.g., clarifying Service Hosting Environment (excluding RAN) |
| CPR 14.1.4-1-2 | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (subject to regulatory requirements and subscriber permissions), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE, and provide means for a subscriber to provide a preference for UE energy efficiency. NOTE: The 6G network uses this preference when improving UE energy efficiency. Examples of preference include: - subscriber preference on prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency Huawei: Suggest to separate PR 5.8.4.6-2 PR 5.8.5.6-1 from the merged PCR. This merged one is not perused. Align the handling on SHE. E.g., clarifying Service Hosting Environment (excluding RAN) Nokia: prefer ZTE split proposal below (alt 1) |
| Alt 1 CPR 14.1.4-1-2 (ZTE/S1-254160) | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (regulatory requirements and subscriber permission), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE. ZTE Revision Subject to regulatory requirements and subscriber permission, the 6G system shall support mechanisms (e.g. enabling task offloading from the UE to the Service Hosting Environment) to improve UE energy efficiency when providing services to the subscriber considering the current traffic conditions of the UE battery life and thermal issue of UE. | PR 5.8.1.6-2 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency [ZTE] move PR5.8.1.6-3 to seperate CPR Align the handling on SHE. E.g., clarifying Service Hosting Environment (excluding RAN) Nokia: ok ZTE: revise the CPR proposal from S1-254160 |
| New CPR 14.1.4-1-ZTE (S1-254160) | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | [ZTE] it is for user’s preference, better to seperate. Should “user preference” be “subscriber preference? How does this relate to subscriber permission? Huawei: Suggest to remove the note, which is solution oriented. Nokia :ok |
| Alt 2 CPR 14.1.4-1-2 (10 Nov reflector comments) | The 6G system shall be able to support mechanisms (including a mechanism to consider user preference) to improve UE energy efficiency when providing services to subscribers. Examples of such preferences - prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 | UE Energy Efficiency Should “user preference” be “subscriber preference? How does this relate to subscriber permission? Huawei: Suggest to remove the example of preference, which is solution oriented. Nokia: prefer ZTE proposal above |
| Alt CPR 14.1.4-1-3 (Nokia) | Nokia Proposal The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or estimation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS, targeted service area and/or targeted set of users. InterDigital Proposal The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS/QoE, targeted service area and/or targeted set of users. | PR 5.8.2.6-1 | Access Energy Consumption Huawei: suggest to remove not clear about the relationship between the energy consumed calculation and the one-to-many service. Need clarify. [InterDigital: inline addition. QoE for the media services] |
| CPR 14.1.4-1-4 | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that - define a maximum slice energy credit limit for services without QoS criteria, - support a mechanism to perform slice energy credit limit control for services without QoS criteria. NOTE: The result of the credit control is not specified by this requirement. | PR 5.8.3.6-1 PR 5.8.3.6-2 | Slice Energy Management Huawei: maybe first consider the how to handle slice in the legacy service. It is not clear about the credit. |
| Orig PR | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that define a maximum slice energy credit limit for services without QoS criteria. | PR 5.8.3.6-1 | Provided for info |
| Orig PR | Subject to operator’s policy, the 6G system shall support a mechanism to perform slice energy credit limit control for services without QoS criteria. | PR 5.8.3.6-2 | Provided for info |
| Orig PR | Subject to operator’s policy and regulatory requirements, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time), with the respective information on predicted energy consumption and carbon equivalent emissions per scenario. NOTE: The following network energy information about energy consumption and carbon equivalent emissions can be calculated and/or estimated. | PR 5.8.6.6-1 | Provided for info |
| Orig PR | Subject to regulatory requirements, the 6G system shall be able to optimise the network energy saving considering the current traffic conditions of the network. | PR 5.8.4.6-1 | Provided for info Huawei: suggest to keep the original wording. |
| CPR 14.1.4-1-6 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and optimise the network energy saving considering the current traffic conditions of the network. ZTE (S1-254160) Proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and/or the current traffic conditions of the network. ZTE (S1-254160) Proposal with TNO Revisions Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) with the respective information on predicted energy consumption and carbon equivalent emissions per scenario and/or the current traffic conditions of the network. ZTE Revision of proposal from S1-254160 w/updated PR 5.8.6.6-1 Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) with the respective information on predicted energy consumption and carbone equivalent emissions per scenario, and/or the current traffic conditions of the network. | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving Huawei: if merge PR 5.8.6.6-1, PR 5.8.4.6-1, we prefer this one. |
| Alt CPR 14.1.4-1-7 (ZTE/S1-254160) | ZTE Proposal Subject to operator’s policy and regulatory requirements, the 6G network shall be able to monitor and expose to a trusted third-party the network energy consumption information including the energy consumption related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). Nokia Proposal Subject to operator’s policy and regulatory requirements, the 6G network shall be able to expose to an authorised third-party the network energy consumption information (e.g. through monitoring and/or estimation) related to 3GPP services, over a specific time period (e.g. month etc.). ZTE Proposal with TNO revisions Subject to operator’s policy and regulatory requirements, the 6G network shall be able to monitor and expose to a trusted third-party the network energy consumption information including the energy consumption and carbon equivalent emissions related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 PR 6.37.6-1 PR 6.37.6-2 | Network (Sensing, AI, computing) related Energy Consumption Huawei: not clear how to expose the energy consumption for service. Need clarify. Huawei: not clear how to monitor the energy consumption for service. Need clarify. Nokia: prefer to avoid the word monitor, which may not always be achievable (prefer original CPR) |
| Orig PR | Subject to operator’s policy and regulatory requirementsregulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related with to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Provided for Info |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party. | PR 6.37.6-1 | Provided for Info Potential dupe with Table 14.1.8-5 – AI model training and inferencing |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party | PR 6.37.6-2 | Provided for Info Potential dupe with Table 14.1.8-5 – AI model training and inferencing |
| Alt CPR 14.1.4-1-8 (Nokia) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall support a means for a group of cooperating UEs to reduce the energy consumption or one or more UEs of the group for the communication service whilst meeting requested service performance. | PR 5.8.8.6-1 | Cooperating UEs |
| Alt NEW CPR 14.1.4-1-CC (InterDigital) | The 6G network shall be able to collect energy related information exposed by the Service Hosting Environments and/or by the energy management system, which is outside of 3GPP networks. | PR 6.24.6-1 PR 6.24.6-3? | ZTE: According to the use case of 6.24, revise the “energy related data” to “energy related information”: Huawei: need to clarify what is the energy related information of SHE.what the relationship between SHE and energy management system?. Align the handling on SHE. E.g., clarifying Service Hosting Environment (excluding RAN) [InterDigital – new CPR wording for clarity and completeness. Receiving data from outside 3GPP is not in scope for us unless it can be correlated for use] |
| Orig PR | The 6G network shall be able to collect energy related data of the Service Hosting Environment. | PR 6.24.6-1 | Provided for info Potential dupe with Table 14.1.9-1 – General Computing requirements |
| Orig PR | The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier. | PR 6.24.6-3 | Provided for info Potential dupe with Table 14.1.9-1 – General Computing requirements |
| New CPR 14.1.4-1-FF Moved from 14.1.4-1-14 (S1-254160) | The 6G network shall be able to support mechanisms to effectively use the supplied energy from the energy supplier, considering energy related information from energy management system, which is outside of 3GPP networks. ZTE/Softbank Proposal The 6G network shall be able to support mechanisms to effectively use the supplied energy via selection of Service Hosting environment and computing resources considering energy related information from energy management system. | PR 6.24.6-3 | According to the use case of 6.24, revise it as: Huawei: clarify what is the energy supplier, and what is the relationship to 3GPP. ZTE: offline discussed with Softbank, and revised the CPR to resolve ZTE comment. |
| New CPR 14.1.4-1-HH | The 6G network shall be able to support means to provide the network operator an indication of the energy consumed by the available access networks for the provision of a one-to-many service. NOTE 1: The network operator may take into account this energy consumption information when deciding how to deliver the service. NOTE 2: The energy consumption information relates to the access network’s energy needed (taking into account the energy already consumed by the network over its life duration) for the delivery of a given one-to-many service. | PR 5.8.2.6-2 | Energy consumption of access networks Huawei: slightly rewording. what is the indication? Solution oriented. Seems covered by CPR 14.1.4-1-3 Nokia: suggest to merge into CPR 14.1.4-1-3 ZTE: What is the meaning of “indication”? Is it energy consumption exposure? |
| Alt New CPR 14.1.4-1-## | Huawei Proposal Subject to operator’s policy, the 6G network shall provide means to configure 3GPP services (e.g. sensing, AI, computing etc.) considering energy efficiency. InterDigital Proposal Subject to operator’s policy, the 6G network shall provide means to consider energy consumption information and carbon equivalent emissions of energy sources when configuring 3GPP services (e.g. sensing, AI, computing etc.) targeting different scenarios. | PR 5.8.6.6-3 | Huawei: wording suggestion. [InterDigital: alternative adding clarification of emissions] |

* * * End of Changes * * *