3GPP TSG SA WG 1 Meeting #113	S1-261126

9-13 February 2026, Goa, India	(merge of S1-261104, 1072)

Source:	Nokia (Moderator)

pCR Title:	Consolidation of KPI requirements on Ubiquitous Connectivity

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.5

Document for:	Approval

Contact:	Feifei Lou, feifei.lou@nokia.com

Abstract: this contribution provides an update of the performance requirements consolidation on ubiquitous connectivity, based on the end of discussion from SA1#112 ad hoc e-meeting.

1. Introduction

The consolidated performance requirements for clause 8 are categorised into two parts.

The consolidated performance requirements for positioning services covers the following use cases and PRs. The scenarios in the KPI table are sorted from less demanding to more demanding cases.

- [PR 8.5.6-1] from use case on resilient positioning in satellite networks

- [PR 8.7.6-2] from use case on low-energy positioning in satellite networks (updated in SA1#112)

- [PR 8.10.6-1] from use case on hybrid TN and NTN positioning

- [PR 8.11.6-2] from use case on hybrid NTN and GNSS positioning

The consolidated performance requirements for NTN covers the following use cases and PRs. Based on SA1#112 discussion, all of the KPIs in TS 22.261 Table 7.4.2-1 Performance requirements for satellite access are included.

- [PR 8.2.6-1] from use case on ubiquitous and resilient network (EN addressed in SA1#112)

- [PR 8.4.6-2] from use case on service continuity for wearable mobile devices

- [PR 8.6.6-2] from use case on disaster relief

- [PR 8.8.6-2] from use case on global mobile video (updated in SA1#112)

- [PR 8.9.6-2] from use case on low-altitude logistics supported by NTN

- [PR 11.5.6-1] from use case on immersive media services for advanced air mobility (AAM) enabled by 6G NTN (from industry and verticals)

- [PR 11.6.6-1] from use case on high-rate aircraft communication services in 6G (from industry and verticals)

2. Summary of changes

The update is from the following PRs, including the performance requirements for satellite access for UAM and Airplane from industry and verticals.

- Add “ubiquitous” in the title of sub-clause 14.2.3.2 and Table 14.2.3.2-1.

- Update note 2 to be clear it is about 3GPP satellite-based positioning.

- After double checking the service flow in UC 8.7, original note 5 is replaced with note 2 as it is also about 3GPP satellite-based positioning.

- “Maritime” is reversed to “Airborne and maritime” as in UC 8.11. Airborne in this use case uses hybrid NTN and GNSS positioning, which is different from other airborne use cases.

- With the above changes, the two ENs are solved.

- “Containers on board” is reversed to “Containers” as in UC 8.7. The use case is about tracking containers, not tracking containers on vessels.

- Add the referred use case for each scenario.

- Merge of S1-261104.

3. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

* * * First Change * * * *

## 14.2.3	Ubiquitous Connectivity

### 14.2.3.1	Consolidated performance requirements for satellite-based positioning services

Table 14.2.3.1-1: Consolidated performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Positioning service area/Environment of use | MAX UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |
| Containers (note 2) (UC 8.7) | [100] m | N/A | [95] % | N/A | Outdoor | [37] km/h | Container mounted | Battery life expectancy (note 1) 12 years |
| Pallets (note 2) (UC 8.7) | [100] m | N/A | [95] % | N/A | Outdoor | [100] km/h | Vehicle mounted | Battery life expectancy (note 1) 7 years |
| Wagons (note 2) (UC 8.7) | [100] m | N/A | [95] % | N/A | Outdoor | [350] km/h | Train mounted | Battery life expectancy (note 1) 20 years |
| Airplane en-route (note 2) (UC 8.5) | [50] m | [50] m | [99] % | [1] s | Outdoor | [1500] km/h | Airplane mounted | - |
| Airplane landing (note 2) (UC 8.5) | [10] m | [10] m | [99] % | [1] s | Outdoor | [350] km/h | Airplane mounted | - |
| UE with partly obstructed sky (note 5) (UC 8.11) | [10] m | [3] m | [95] % | [1] s | Outdoor rural and urban | [250] km/h | Handheld, vehicle mounted or IoT | - |
| Agriculture & farming (e.g. livestock, equipment tracking) (note 2) (UC 8.7) | [5-10] m [376] |  | [95 %] |  | Outdoor (Rural/remote areas) | [30] km/h | Animal mounted (e.g. collar) or passive asset/equipment mounted (e.g. non-powered harvesting crates, trailers, wheelbarrows) | Battery life expectancy (note 1) 10 years |
| Airborne and maritime (note 5) (UC 8.11) | [3] m | [3] m | [99] % | [1] s | Outdoor rural | [500] km/h | Airborne or maritime mounted | - |
| UAV facility monitoring (note 3) (UC 8.10) | [3] m | [3] m | [99] % | [0,1-0,5] s | Outdoor | [160] km/h | UAV mounted | - |
| UAV positioning (note 2) (UC 8.5) | [1] m | [1] m | [99] % | [1] s | Outdoor | [160] km/h | UAV mounted | - |
| Airplane taxiway (note 3, 4) (UC 8.10) | [1] m | N/A | [99] % | [0,1-0,5] s | Outdoor | [100] km/h | Airplane mounted | - |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency. The frequency assumed here is the number of times per day a device is involved in a positioning operation using terrestrial/6G Satellite Positioning, with the required accuracy anywhere within the required coverage area, which could be followed by a communication request from the UE to send this position into a message to a third party application server, unless this information can be obtained from the 6G network itself. In this table, frequency is assumed to be 1h (that is approximately 24 operations per day) and typical message size around 200 bytes. NOTE 2: Positioning services are provided with 3GPP satellite-based positioning technologies, independently of non-3GPP positioning technologies (e.g. GNSS). Multiple satellites can be used to support 3GPP positioning technologies. NOTE 3: Requirements for Airplane taxiway and UAV facility monitoring are in [25]. Hybrid TN and NTN positioning is considered in the use case. NOTE 4: Airplane taxiway refers to an airplane on ground during taxi operations. NOTE 5: Hybrid NTN and GNSS positioning is considered in the use case. KPIs may not be fully under control of the 6G system. |  |  |  |  |  |  |  |  |

### 14.2.3.2	Consolidated performance requirements for ubiquitous communication

Table 14.2.3.2-1: Consolidated performance requirements for ubiquitous communication

| Scenario | Experienced data rate | Area traffic capacity (note 1) | Overall user density | Activity factor | UE speed | UE type | Reliability | Service availability | End-to-end latency | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite access |  |  |  |  |  |  |  |  |  |  |
| Disaster relief (note 2) (UC 8.6) | N/A | N/A | Up to [100] devices/km2 for SMS only | N/A | 3 km/h | Handheld | [99,9-99,999] % | Up to 99,9 % | Up to [600] ms | Peak data rate DL: Up to [20] Mbit/s UL: Up to [2] Mbit/s |
| Global video service in remote area (note 5) (UC 8.8) | High video resolution DL: [2-5] Mbit/s UL: [1] Mbit/s Low video resolution DL: [2] Mbit/s UL: [250] kbit/s | DL: Up to [10] kbit/s/km2 (note 6) | [< 0,1] person/km2 | 2% | N/A | Handheld | N/A | N/A | N/A | - |
| Global video service in deep rural area (note 5) (UC 8.8) | High video resolution DL: [2-10] Mbit/s UL: [2] Mbit/s Low video resolution DL: [2] Mbit/s UL: [250] kbit/s | DL: [1] Mbit/s/km2 (note 7) | [0,1-10] person/km2 | 4% | N/A | Handheld | N/A | N/A | N/A | - |
| Immersive media service via AAM/UAM (direction: from network to user, uncompressed) (UC 11.5) | DL: [1,3] Gbit/s (note 9) (note 11) | DL: TBD | - | - | Up to 260 km/h (note 14) | UAM mounted | - | - | - | Service interruption time [10] ms for 60 fps, [20] ms for 30 fps (note 8) |
| Immersive media service via AAM/UAM (direction: from network to user, compressed) (UC 11.5) | DL: [25] Mbit/s (note 10) (note 11) | DL: TBD | - | - | Up to 260 km/h (note 14) | UAM mounted | - | - | - | Service interruption time [10] ms for 60 fps, [20] ms for 30 fps (note 8) |
| Airplanes connectivity (note 12) (UC 11.6) | DL: 6 Gbit/s/plane UL: 3 Gbit/s/plane | - | - | - | Up to [1500] km/h | Airplane mounted (note 13) | - | - | - | - |
| Pedestrian (note 15) | DL: 5 Mbit/s UL: 2 Mbits | DL: 0.1 Mbit/s:km² UL: 0.05 Mbit/s:km² | 2/km2 | 1% | Pedestrian | Handheld | N/A | N/A | N/A | N/A |
| High speed train connectivity (note 16) | DL: 40 Mbit/s UL: 40 Mbits per wagon | DL: 1 Mbit/s:km² UL: 1 Mbit/s:km² | 0.5/km2 | 5% | Up to 500 km/h | Train mounted | N/A | N/A | N/A | N/A |
| Drone/vehicular connectivity (note 16) | DL: 40 Mbit/s UL: 40 Mbits per drone or vehicle | DL: 1 Mbit/s:km² UL: 1 Mbit/s:km² | 1/km2 | 2.5% | Up to 250 km/h | Drone/vehicle mounted | N/A | N/A | N/A | N/A |
| Large vessel connectivity (note 16) | DL: 60 Mbit/s UL: 60 Mbits Per vessel | DL: 1.5 Mbit/s:km² UL: 1.5 Mbit/s:km² | 0.25/km2 | 10% | Up to 50 km/h | Vessel mounted | N/A | N/A | N/A | N/A |
| Terrestrial access |  |  |  |  |  |  |  |  |  |  |
| Ubiquitous and Resilient Network (note 3) (UC 8.2) | DL: [0,1-25] Mbit/s UL: [2] Mbit/s (note 4) | N/A | [0,1] devices/m2 | N/A | up to [120] km/h | Handheld | [99.9-99.999] % | [98.5] % | [10-100] ms (note 4) [10-500] ms | Coverage [99.9] % |
| NOTE 1: Area capacity is averaged over a satellite beam. NOTE 2: Positioning accuracy is low (≈ [100] m). Positioning service availability is up to [99] %. Positioning service latency is [1] s. NOTE 3: Location accuracy is low (≈ [10] m). Positioning availability is [99] %. Positioning latency is [1] s. NOTE 4: When using base station on board HAPS for NTN, end-to-end latency should be up to 10 ms (operating at 20 km altitude) and user experienced data rate should be up to [500] Mb/s for DL and [50] Mb/s for UL to enable eMBB service [374]. NOTE 5: Reduced video quality is acceptable in the remote areas and deep rural area, see chapter 3.9.1.3 and 3.9.1.5 in [31]. NOTE 6: The DL area traffic capacity corresponds to an average user density of 0,1 person/km2. NOTE 7: The DL area traffic capacity corresponds to an average user density of 5 persons/km2 equally shared by two operators. The area traffic is here split between two operators, with [2] Mbit/s/km2 in total. NOTE 8: It is assumed that the interruption time is less than a single frame (16ms for 60 fps, 32 ms for 30 fps) plus a margin of an order of millisecond (e.g. for other processing time). NOTE 9: Uncompressed (0.8 M points per frame, total 300 frames are considered: i.e. 43.2 Mbits/s, 30 fps) NOTE 10: Compressed (Video-based Point Cloud Compression (V-PCC), bpip (bits per input points) total 1.14 (Color 0.9, Geometry 0,11, Occupancy 0.13) are considered: 25 Mbits/s) NOTE 11: It is assumed that the number of passengers using immersive media service per UAM is up to 2. NOTE 12: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 2:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple satellite links NOTE 13: This airplane mounted UE is operated as a mobile base station relay, providing connectivity to normal UEs used by passengers. These KPIs enable the case of XR UEs for applications in highly dynamic environments, where UEs offload processing to an edge-cloud. NOTE 14: The number is based on [408], [409]. NOTE 15: The values are assuming channel bandwidth of 30 MHz per direction for UE with omnidirectional antenna. NOTE 16: The values are assuming channel bandwidth of 400 MHz per direction for UE with directional antenna. |  |  |  |  |  |  |  |  |  |  |

[CPR 14.2.3.2-1]: The 6G system with satellite access, shall support SMS delivery to UEs with up to [1000] devices/km2 density.

[CPR 14.2.3.2-2] The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) at the altitudes from 0 to 3 km via satellite access and/or terrestrial access.

* * * End of changes * * * *