3GPP TSG SA WG 1 Meeting #113	S1-261128

9-13 February 2026, Goa, India	(merge of S1-261106, 1074)

Source:	Nokia (Moderator)

pCR Title:	Consolidation of KPI requirements on Industry and Verticals

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.8

Document for:	Approval

Contact:	Feifei Lou, feifei.lou@nokia.com

Abstract: Update the consolidated KPI tables for Industry and Verticals.

1. Introduction

S1-260046 was for information in SA1#112 ad hoc e-meeting. The contribution provides an update to the consolidation of the KPI tables for industry and verticals based on v1.1.0 of the TR. Table 11.29.6-1 is considered with EN and not consolidated.

2. Summary of Changes

- Align sub-clause and table number and title.

- Change the format of the reference to use cases.

- Merge S1-261106

3. Conclusions

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

* * * First Change * * * *

## 14.2.6	Industry and Verticals

### 14.2.6.1 UAV, and UAM and Airplane Consolidated Performance Requirements

Table 14.2.6.1-1: KPIs for streaming services for UAM

| Scenario (note 7) | User Experienced Data rate (DL) | End to end Latency | Typical transmission interval |  | Reliability (note 5) | Service area |
| --- | --- | --- | --- | --- | --- | --- |
| Sensing result to a UAM aircraft (note 1) (UC 11.2) | – | [20] ms | [20] ms |  | [99.9] % | – |
| 8K video live broadcast (UC 11.2) | [100 Mbps] Traffic from the UAM aircraft (note 2) | [200] ms (note 2) | – |  | [95] % | Urban, scenic area |
|  | [600 kbps] Traffic towards the UAM aircraft (note 2) | [20] ms (note 2) | – |  | [95] % |  |
| Video streaming (UC 11.2) | [4 Mbps] for 720p video [9 Mbps] for 1080p video Traffic from the UAM aircraft (note 2) | [100] ms (note 2) | – |  | [95] % | Urban, rural area |
|  | [100 Mbps] for 8K video Traffic from the UAM aircraft (note 2) | [100] ms (note 2) | – |  | [95] % |  |
| Remote controller through HD video (UC 11.2) | [>=25 Mbps] Traffic from the UAM aircraft (note 2) | [100] ms (note 2) | – |  | [99] % | Urban, rural area |
|  | [300 kbps] Traffic towards the UAM aircraft (note 2) | [20] ms (note 2) | – |  | [99] % |  |
| Video conferencing or video chat (UC 11.2) | [25 Mbps] Traffic from passengers onboard UAM aircraft (note 2) (note 3) | [100] ms (note 2) | – |  | [99] % | Urban, rural area |
|  | [25 Mbps] Traffic towards passengers onboard UAM aircraft (note 2) (note 3) | [100] ms (note 2) | – |  | [99] % |  |
| Immersive multimedia service (e.g., cloud gaming) (UC 11.2) | [500 kbps] Traffic from passengers onboard UAM aircraft (note 3) | [50] ms (note 4) | – |  | [99] % | Urban, rural area |
|  | [100~500 Mbps] Traffic towards passengers onboard UAM aircraft (note 3) (note 6) | [50] ms (note 4) | – |  | [99] % |  |
| NOTE 1: Typically, the message size for one object detected and/or tracked via sensing is 1 kbyte [37]. It is assumed that around 25 objects (25 kbyte) per frame (20 ms) are sensed surrounding an aircraft. The reliability requirement for UAM aircrafts is about 99.9 %. NOTE 2: These values are aligned with the KPIs for services provided to the UAV applications in TS 22.125 [35], Table 7.1-1. NOTE 3: The value is per passenger; and it is assumed that up to 4 passengers per UAM aircraft use communication services simultaneously. NOTE 4: According to TR 26.928 [50], typically a 50ms latency is a required for cloud gaming use case. NOTE 5: According to [306], the reliability of real-time services is set to 99%, and that of video streaming is set to 95%. NOTE 6: Data rate is calculated assuming typical parameters (e.g. resolution, refresh rate and compression rate). Some codecs may further drop the bitrate requirement. NOTE 7: UAM in terms of altitude up to 1000m. |  |  |  |  |  |  |

Table 14.2.6.1-2: KPIs for massive data transfer for airplane

| Scenario | Experienced data rate (DL) (note) | Experienced data rate (UL) (note) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Airplanes connectivity in airport vicinity (within 20 km airport radius, up to 1500 m) (UC 11.6) | [11 Gbps]/ plane | [22 Gbps]/ plane | TBD | TBD | TBD | Up to [350] km/h | Airplane mounted |
| NOTE: These KPIs are related to massive data transfer including operational data when the aircraft is on the ground or near the ground. The need is to send 10 TByte from aircraft to the ground (aircraft 1 hour in 20 km radius → 22 Gbit/s) [125]. |  |  |  |  |  |  |  |

### 14.2.6.2	Consolidated Performance requirements for Digital Twin

Table 14.2.6.2-1: KPIs for Digital Twin

| Scenario | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Service Area | Location accuracy | Connection density [devices / m2] | UE Speed | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Real-Time Digital Twin (note 1) (UC 11.4) | [1-10] ms (note 2) | [<100] Mbps (UL) (note 3) | 99.999% (note 4) | [up to 1km2] (note 5) | [≤ 10] cm | [1-10] devices/m2 | Up to vehicular speed | Communication between physical system and the network (note 6) |
| NOTE 1: Wireless-sensing capability is expected to contribute to enrich the Digital Twin model, with KPIs related to e.g. location accuracy, resolution, range and latency NOTE 2: Very low latency for the Real-time aspect, both in UL (to report up-to-date data) and DL (to send timely controls) NOTE 3: It is expected uplink video will be most demanding. NOTE 4: The minimum requirement as defined in [415]. NOTE 5: Service coverage both outdoor & indoor (e.g. industrial plant, city area) [31] NOTE 6: Performance requirements related to the communication between the DT and the end-user device accessing the DT (including network to UE communication) are not included in this table and may depend on the actual application (e.g. web, immersive/XR etc.) used to access the DT |  |  |  |  |  |  |  |  |

### 14.2.6.3	Consolidated Performance requirements for Localized network

Table 14.2.6.3-1: KPIs for network managed localized communication

| Scenario | Service bit rate: user-experienced data rate | End-to-end latency |
| --- | --- | --- |
| Direct device communication in a local network – Factory (UC 11.20) | [≤ 1.5] Gbps (note 1) | [5] ms (note 1) |
| Direct device communication in a local network – Vehicle (UC 11.20) | [0,1 to [1]] Gbps (note 2) | [5] ms (note 2) |
| NOTE 1: Refer to the KPI requirement in TS22.261 [14] Table 7.10.2-1. NOTE 2: Refer to the performance of Gaming or Interactive Data Exchanging in TS22.261 [14] Table 7.6.1-1 |  |  |

### 14.2.6.4	Consolidated Performance requirements for other Industry Robotics

Table 14.2.6.4-1: KPIs for Robotics

| Scenario | Max allowed end-to-end latency | Application data rate | Area traffic capacity | Reliability | Message size | Service area | Transfer interval | Connection density | UE speed | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cooperating Mobile Robots – 6G Network Services (UC 11.3) | [1 to 10] ms | [<10] Mbps [<250] Mbps (note 2) | – | [99.999 to 99.99999] % (note 4) | (note 3) | [20 m² (indoor) up to 1 km2 (outdoor)] (note 5) | [≤ 10] ms | [≤ 0.5] devices/ m² (note 1) | [< 20] km/h | Robot to Network for services such as digital twin applications, AI/ML compute servers. |
| AMRs performing tasks with low acceleration-sensitivity (note 11) (UC 11.9) | [100] ms (note 6) (note 8) | – | DL: TBD UL: - | [99.9999] % (note 8) | – | >0.4 km (note 9) (note 10) | – | – | Up to [10] m/s (note 7) | UE Type: AMR |
| AMRs performing tasks with relatively high acceleration-sensitivity (note 11) (UC 11.9) | [10] ms (note 6) (note 8) | – | DL: TBD UL: - | [99.9999] % (note 8) | – | >0.6 km (note 9) (note 10) | – | – | Up to [10] m/s (note 7) | UE Type: AMR |
| NOTE 1: Local concentration of mobile robots, for instance, cooperative carrying assuming 8 robots on a space of 4 m x 4 m carrying an object. NOTE 2: The higher data rate applies when e.g. immersive XR, AI/ML traffic, digital twin data, is exchanged. Otherwise, the lower data rate applies. It is expected uplink video will be most demanding. NOTE 3: Any common message size to be expected. Message size depends on type of exchanged data, for instance, sensor data, control data, video streams, immersive XR, AI/ML traffic, digital twin data, etc. NOTE 4: The control of real-time industrial processes requires very high service reliability. Can be lower for non-real-time digital twin or in other contexts (e.g. training, engineering, planning). NOTE 5: Service area depends on scenario with cooperating mobile robots and on size of location/collaboration area for the scenario. Examples for service areas are the working area of collaborative carrying mobile robots (see 11.3.3.1), a factory floor or a workshop, a construction site deploying autonomous robots that work in unison to construct a building (see 11.3.3.2), or a farmer’s field in autonomous farming with self-driving, smart machines that work collaboratively in agricultural operations. NOTE 6: The latency considered for up to two hops only. The latency is measured in the time interval between the time that collaborative awareness data is ready to be sent and the time that the data is received so that the information can be delivered to a nearby UE to prepare for necessary action to take. NOTE 7: Relative speed between a pair of UEs (AMRs) is considered. NOTE 8: The latency and reliability may vary depending on the type of application and the role of AMR performing different sub-tasks. NOTE 9: It is assumed that AMRs with heavy payloads are operating passively (e.g. at a very low acceleration level) such that the time it takes for them to make a complete stop is long (e.g. more than 20 seconds). NOTE 10: This means the minimum required distance when the UEs need to start communication in order to provide minimum ample time for UEs (AMRs) to prepare. NOTE 11: This includes some scenarios where there are different AMRs using different NPNs. |  |  |  |  |  |  |  |  |  |  |

### 14.2.6.5	Consolidated Performance requirements for industry and vertical other requirements

Table 14.2.6.5-1: KPIs for services with high reliability and/or low latency

| Scenario | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency | Experienced Data rate | Peak Data rate | Message size | Transfer interval: target value | UE speed | Number of UE | Service area | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Direct Transfer Trip (note 1) (UC 11.24) | [> 99.999] % | – | [< 5-10] ms | DL: [0.05 – 5] Mbps UL: [0.05 – 5] Mbps | – | [1500] bytes (note 2) | [100] ms | – | [2-10] devices | – | – |
| Decentralized Local Grid Power Contract (UC 11.26) | 99.999 % | – | [1] ms | – | – | < 100 bytes (note 3) | 50Hz: 20 ms 60Hz: 17 ms | – | < 200 devices (note 4) | – | – |
| AR guided task (note 5) (UC 11.8) | – | Up to [99.999] % | [1-5] ms (note 6) | DL: [240 – 500] Mbps UL: [50] Mbps | – | – | – | Pedestrian | [20000] devices/km2 (note 7) | Factory | – |
| Monitoring Environment / Telemetry (UC 11.25) | [99.999] % | [99.999] % | [100-500] ms | – | UL: [0.04] Mbps | [200-400] bytes | [1-15] min (note 8) | – | [10-15] devices per km of transmission line | – | Battery life: [10-20] years |
| Monitoring vibration (UC 11.25) | [99.999] % | [99.999] % | [100-500] ms | – | UL: [0.8] Mbps | [2-10] kbytes | [15-60] min (note 8) | – | [5-15] devices per km of transmission line | – | Battery life: [10-20] years |
| Monitoring Arching/partial voltage leak (UC 11.25) | [99.999] % | [99.999] % | [5-500] ms (note 10) | – | UL: [24] Mbps | [300] kbytes | [1-5] min (note 9) | – | [10-15] devices per km of transmission line | – | Battery life: [10-20] years |
| Monitoring Grid assets & Photos on events (UC 11.25) | [99.999] % | [99.999] % | [100-500] ms | – | UL: [24] Mbps | [150-300] kbytes | [500 ms-5 min] | – | [10-15] devices per km of transmission line | – | Battery life: [10-20] years |
| Monitoring Grid assets & videos on events (UC 11.25) | [99.999] % | [99.999] % | [100-500] ms | – | UL: [200] Mbps | [300-2000] kbytes | [500 ms-5 min] | – | [10-15] devices per km of transmission line | – | Battery life: [10-20] years |
| Medical ultrasound monitoring for patients at home (PLMN) (UC 11.28) | > 99.99 % | > 1 month | < 100 s | UL: < 86 Mbps (note 12) | – | – | – | stationary | [10-100] devices/km2 | Urban and rural areas and indoor. (note 11) | Energy-efficient transmissions using a device powered with a 3.3V battery of capacity < 1000 mAh that can last at least several hours without recharging |
| Medical ultrasound monitoring for patients on the move (PLMN) (UC 11.28) | > 99.99 % | > 1 month | < 100 s | UL: < 86 Mbps (note 12) | – | – | – | < 250 km/h | [10-100] devices/km2 | Urban and rural wide | Energy-efficient transmissions using a device powered with a 3.3V battery of capacity < 1000 mAh that can last at least several hours without recharging |
| Distributed Energy Resources (UC 11.29) | 99.999 9 % | – | 3 ms | 5.4 Mbps | – | 140 kbytes | ≤ 1 ms | Stationary | ≤ 200 devices | 20 km x 30 km | – |
| Electrical Distribution – Distributed automated switching for isolation and service restoration (UC 11.29) | 99.999 9 % | – | < 5 ms | 1.5 Mbps | – | 1500 bytes | ≥ 1 ms | Stationary | 20 devices | 20 km x 30 km | Survival time: 5 ms |
| Distributed Voltage Control (UC 11.29) | 99.999 % | – | 100 ms | – | – | ~100 bytes | 200 ms | stationary | 400 devices | 20 km x 30 km | – |
| Distributed energy storage – monitoring (UC 11.29) | > 99.9 % | – | < 10 ms | UL: 16 Mbps DL: >100 kbps | – | 800 kbytes | 10 ms | – | >10 devices/km2 | 20 km x 30 km | – |
| Distributed energy storage – data collection (UC 11.29) | > 99.9 % | – | DL: <10 ms UL: 1 s | UL: 10.4 Mbps DL: >100 kbps | – | UL: 1.3 Mbytes DL >100 kbytes | 1 s | – | >10 devices/km2 | 20 km x 30 km | – |
| Ensuring uninterrupted communication service availability during emergencies (UC 11.29) | 99.999 9 % | – | 100 ms | < 1 kbps per distributed energy resource | – | – | – | Stationary | – | 20 km x 30 km | – |
| NOTE 1: DTT uses periodic and aperiodic communication service supporting messages for fault location, isolation, and service restoration. NOTE 2: Typical GOOSE message sizes are 500-byte in IEC 61850 [356] messages plus any overhead due to IP tunnelling, security etc. make 1500 kBytes approximately. DTT messages are always passed between 2 protection devices (UE). NOTE 3: The typical message size for instantaneous electric consumption information time stamped. NOTE 4: This is maximum users in a contract in France today. NOTE 5: All the values in this table are targeted values and not strict requirements. The DL data rate is assumed 4K video flow + control information + audio data with data compression ratio of 1:25 to 1:50; and UL data rate is assumed video flow 25 Mbps [24] + high resolution depth sensor data 25 Mbps + audio data. NOTE 6: It is for downlink or uplink one way delay. NOTE 7: Not all UEs are consuming the same 3GPP services at the same time. NOTE 8: Immediate transmission when a threshold is crossed. NOTE 9: Monitoring frequency varies depending on what is monitored and during which period of the year the voltage is monitored, Winter months are less frequent than summer months. NOTE 10: 5–15 ms is required only when the operator chooses to use the network in a rapid protection loop. NOTE 11: “deep indoor” term is meant to be places like e.g. elevators, building’s basement, underground parking lot. NOTE 12: The ultrasound cardiac-assessment patch must be able to transmit single-plane, B-mode (i.e. grey-scale, 2-dimensional) moving images to the edge. A resolution of 600 x 600 pixels with 8-bit pixel depth and a 30 fps framerate is expected. As the possibilities for image compression are limited (e.g. considering the quality requirements for medical imagery), this leads to uplink data generation at rates of up to 86 Mbps. One or a few assessments will be made per day and each assessment involves recording 1 minute of data. Transmission may be spread out over time to some extent (e.g. 20 Mbps for 5 minutes). |  |  |  |  |  |  |  |  |  |  |  |

Table 14.2.6.5-2: KPIs for Positioning

| Scenario | Positioning Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Number of UE | UE speed | UE type | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |
| AR remote guided task – outdoor (UC 11.8) | [1-3] m (note 5) | [3-5] m (note 5) | [99.9] % (note 6) | [15] ms (note 7) | TBD (note 8) | – | – | TBD (note 9) |
| AR remote guided task – indoor factory (UC 11.8) | [0,2-1] m (note 5) | [1-3] m (note 5) | [99.9] % (note 6) | [15] ms (note 7) | ≥200 (note 8) | – | – | 200,000m2 (note 9) |
| Cooperating Mobile Robots – Robot localization (UC 11.3) | [1] m (note 2) | – | – | – | – | [< 20] km/h | – | – |
| Human presence detection for mobile Robots (UC 11.3) | [40] cm (note 3) | – | – | – | – | Pedestrian | – | – |
| Cooperating Mobile Robots (UC 11.3) | [<10] cm (note 1) | – | – | – | – | slow (note 4) | – | – |
| Real-Time Digital Twin (UC 11.4) | [≤ 10] cm | – | – | [1-10] ms (note 14) | [1-10] devices/m2 | Up to vehicular speed | – | [up to 1] km2 (note 13) |
| Critical Infrastructure Monitoring – Outdoor (UC 11.16) | [10] cm (note 10) | [1] m | [99] % | [100] ms (note 11) | – | stationary | Infrastructure mounted | – |
| Machine Guidance, Machine Control – Outdoor (UC 11.17) | [2-10] cm (note 12) | [10] cm | [99] % | [30] s | – | [5] km/h | Machinery mounted | – |
| NOTE 1: Collaborative tasks such as handing over material and objects or robot navigation in close distance to other objects require more accurate positioning. NOTE 2: Tasks like robot localization can work with less accurate positioning. NOTE 3: Identification of human presence in areas where people are around: [40] cm resolution within an area defined by [1] m range around the robot [112]. NOTE 4: Velocity of robots is usually slow and adapted to the localization performance in such situations. NOTE 5: The positioning accuracies are based on the UEs on boarded on the AR devices and factory equipment which are used to operate AR remote guided tasks. They are almost same as the values in level 5 in TS 22.261 Table 7.3.2.2-1. NOTE 6: The availability reuses the values in level 4 in TS 22.261 Table 7.3.2.2-1. NOTE 7: It reuses the values in level 4 in TS 22.261 Table 7.3.2.2-1. NOTE 8: The number of positioning UE is the number of UEs which the 6G system is asked to position simultaneously in the positioning service area with the required positioning performance. NOTE 9: The 6G positioning service area is based on the typical single industry factory size. NOTE 10: Assuming the UEs are installed at each story of a building with story height of 4m. NOTE 11: Assuming the frequency of an earthquake is 10 Hz. NOTE 12: Assuming the similar performance as RTK-GNSS [231]. NOTE 13: Service coverage both outdoor & indoor (e.g. industrial plant, city area) [31]. NOTE 14: Very low latency for the Real-time aspect, both in UL (to report up-to-date data) and DL (to send timely controls). |  |  |  |  |  |  |  |  |

Table 14.2.6.5-3: KPIs for time synchronization for smart grids

| Number of devices | Time sychronization accuracy | Service area | Scenario |
| --- | --- | --- | --- |
| ≤10,000 (note) | [< 1 µs] | ~150 km x ~150 km (note) | intelligent, renewable-powered distribution systems (medium voltage (MV) cells, low voltage (LV) cells) |
| NOTE: Time synchronization is needed per Distribution System Operator (DSO). The service area corresponds to the service area of the DSO and can vary depending on the size of the DSO’s service area and might be based on geographical conditions and regional borders. The provided numbers are for the Austrian Bundesland Lower Austria. |  |  |  |

* * * End of changes * * * *