3GPP TSG SA WG 1 Meeting #113 	S1-261145

9-13 February 2026, Goa, India	(revision of S1-261044)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.13-1 Massive Communication

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.7

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This pCR proposes to update the Massive Communication CPR tables in TR22.870 v1.1.0 with the greened CPRs from drafting sessions.

* * * First Change (All New CPRs)  * * * *

Table 14.1.13-1 – Massive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.13-1-1 | The 6G system shall support UEs under wide-area (i.e. rural area and deep indoor) coverage NOTE: The wide-area coverage performance requirement will need to be confirmed by RAN working groups. | PR 10.2.6-1 |  |
| CPR 14.1.13-1-2 | The 6G system shall provide emergency services (e.g. emergency calls) support in rural area and deep indoor coverage scenarios, | PR 10.2.6-2 |  |
| CPR 14.1.13-1-3 | The 6G system shall support MO and MT basic services with low-throughput demands (e.g. messaging, SMS, and small data) in rural area and deep indoor coverage. | PR 10.2.6-3 |  |
| CPR 14.1.13-1-4 | The 6G system shall support low-complexity devices in massive communication scenarios. NOTE: Low-complexity devices are needed for massive deployment into next-generation smart meters. This could include low-power capabilities and limitations in the amount of antennas. | PR 10.3.6-2 |  |
| CPR 14.1.13-1-5 | Subject to operator’s policy, the 6G system shall enable an authorized 3rd party to set and configure parameters e.g., if the application depends on MO or MT traffic, small or high-data volumes. | PR 10.3.6-3 |  |