8.3GPP TSG-SA WG1 Meeting #113 	draft S1-261113

9-13 February 2026, Goa, India	(revision of S1-261094)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.11-1 (Satellite-based communication) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.5

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.11-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting. As agreed in SA1 #112 Ad Hoc-e, Table14.1.11-1 will include CPRs about the communication with satellite access and/or satellite backhaul.

2. Reason for Change

Previous versions of this pCR used S1-254020 as the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254412, the output of the initial consolidation discussions for Ubiquitous Connectivity (SA1 #112)

- S1-254120  and S1-254121 (CATT)

- S1-254163 (ZTE Corporation, CSCN)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

This pCR was updated to reflect the endorcsement of S1-254410.

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

There are some CPRs/PRs that are included in both Table 14.1.11-1 (Satellite-based comms) and Table 14.1.11-3 (Other aspects). This needs to be resolved, and I tried to mark those entries.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Moved PR 8.14.6-1, PR 8.13.6-1 to Table 14.1.11-3: Other aspects

- Merged comments from S1-251052 (CATT/China Telecom), but did not move any PRs across tables to retain stability/integrity of tables for discussions. Proposals for PRs PR 8.15.6-1/PR 8.9.6-1 and PR 8.15.6-2 (from CATT pCR S1-261055) were added.

NOTE: Please check that PRs “moved to the KPI discussions” (i.e., below the “End of Changes” do not have any PR-related text that should be captured in the CPR Table.

This revision captures the latest discussions. Changes from the last version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

- Removed text after “End of Changes”.

- Removed PRs 8.4.6-1 and 8.8.6-1. It was decided these were already covered in 5G.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimise the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 |  |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 |  |
| CPR 14.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services to/from UEs and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 PR 8.6.6-3 |  |
| CPR 14.1.11-1-5 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 |  |
| CPR 14.1.11-1-6 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions. | PR 8.18.6-1 |  |
| CPR 14.1.11-1-7 | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | [Satellite Backhaul] New connection method [Huawei: open to which table this requirement should belong to.] |
| CPR 14.1.11-1-8 | The 6G system with satellite backhaul links between 6G base station on board of UAV and the 6G CN, shall be able to minimise service interruption when the satellite backhaul link changes. | PR 8.12.6-2 | [Satellite Backhaul] |
| CPR 14.1.11-1-9 | Subject to operator’s policy and the regulatory requirements, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic. NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite). | PR 8.17.6-2 | [Satellite Backhaul] |
| Orig PR | The 6G network using satellite access shall be able to provide the service hosting environment onboard satellite minimising the necessary bandwidth of the inter satellite links and feeder links. | PR 8.15.6-1 | Provided for info |
| Orig PR | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | PR 8.9.6-1 | Provided for info |
| CPR 14.1.11-1-10 | Subject to operator’s policy, agreement with 3rd party and user permission, the 6G network with satellite access shall be able to provide a computing service via a Service Hosting Environment onboard satellite(s) to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing Both PRs are in this table and Table 14.1.11-3: Other aspects (ubiquitous) and PR 8.9.6-1 is also in Table 14.1.9-1 – General Computing requirements Resolve to which table these PRs/CPRs belong [Huawei: this CPR combines too many aspects. We need to discuss what services are exactly provided by the Onboard Service Hosting Environment. Regarding computing service provided by the Service Hosting Environment on aboard satellite, 6G RAN should be excluded.] |
| CPR 14.1.11-1-11 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to share data among multiple UEs (e.g. UAV), that is provided by the Service Hosting Environment on board satellite. | PR 8.9.6-3 | Computing data sharing In both Table 14.1.11-1: Satellite-based communication and Table 14.1.11-3: Other aspects (ubiquitous). Resolve to which table these PRs/CPRs belong [Huawei: what is the relationship between 14.1.11-1-7 and 14.1.11-1-8? There is some overlapping about the requirement wording. In addition, suggest to remove the note] Huawei: See 14.1.11-1-8 from the Table 14.1.11-1. We are open to which table this requirement should belong to. |
| CPR 14.1.11-1-12 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimise service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | In both Table 14.1.11-1: Satellite-based communication and Table 14.1.11-3: Other aspects (ubiquitous). Resolve to which table these PRs/CPRs belong {Huawei: exclude RAN from the Service Hosting Environment] |
| CPR 14.1.11-1-13 | The 6G system with satellite access shall be able to support the transfer of information related to a computing service, provided by the service hosting environment (excluding RAN) on board satellites, between successive serving satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info Huawei: exclude RAN from the Service Hosting Environment |
| CPR 14.1.11-1-14 | 6G system with satellite access shall provide a means to obtain communication service interruption information (e.g. expected interruptions to a media service) from an authorized third party. | PR 11.5.6-2 |  |
| CPR 14.1.11-1-15 | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 |  |

* * * End of Changes * * *