3GPP TSG-SA WG1 Meeting #113 	draft S1-261260

9-13 February 2026, Goa, India	(revision of S1-261115, S1-261091)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.12-1 (Immersive communication) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.12-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254296, Merged proposed changes on Clause Y.1.12 (Immersive) (SA1 #112)

- S1-254190 (ZTE Corporation)

- S1-254300r1 (Huawei)

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up Table numbering (to align w/TR) and CPR numbering

- Added comments from S1-251081 (Qualcomm)

This revision captures the status of the latest discussions. Changes from the last version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

- Removed text after “End of Changes”

- Removed PR 9.9.6-1 from the table. Decided it is consolidated into KPI table.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.12-1: Immersive communication

| CPR # | Consolidated Potential Requirement | Original PR # |  |
| --- | --- | --- | --- |
| CPR 14.1.12-1-1 | Subject to operator’s policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed QoS (e.g. current and/or predicted throughput within the boundary of the 6G system) for the associated ongoing session. | PR 9.3.6-3 |  |
| CPR 14.1.12-1-2 | Subject to operator’s policy, the 6G network (e.g. core network) shall support hosting of e.g. an AI/ML model in the Service Hosting Environment (excluding RAN) based on latency, traffic load or data privacy requirements. | PR 9.5.6-3 | Hosting AI/ML model in Service Hosting Environment QC: Agrees to move to 14.1.8-5 (AI model and inferencing) |
| CPR 14.1.12-1-3 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide means to allow a authorized third-party application (e.g. AI application) to provide assistance information for identification and delivery of the error tolerant data traffic associated with that third-party application. NOTE 1: The assistance information is used to identify which data is error tolerant . | PR 9.11.6-1 | Third party Support Assistance info to ID and deliver error tolerant data Nokia: NOTE is very technical, suggest to remove QC: Propose rewording but still have the following questions , Who is the trusted third party? How is the trusted party related to the traffic. The context is missing. NOTE also points towards a solution and requirement |
| CPR 14.1.12-1-4 | Subject to operator’s policy, the 6G system shall be able to transmit error tolerant data traffic with adaptive QoS . NOTE: The QoS (e.g. reliability and latency) can be adjusted automatically based on SLA with authorized third party application. | PR 9.11.6-2 |  |
| CPR 14.1.12-1-5 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide a suitable means for error-tolerant communication. | PR 9.11.6-4 | QoS Error tolerant data evaluation |
| CPR 14.1.12-1-6 | Subject to operator’s policy and privacy considerations, the 6G system shall support a mechanism, including enabling one or more authorized third party(ies), to adapt the QoS of an XR session dynamically at different levels of granularity (e.g. per type of media stream, per data flow, per burst) based on the change of the 6G connectivity conditions and QoE policies. | PR 9.12.6-3 | Exposure QoS adaptation QoE Does “privacy considerations” need alignment? |
| CPR 14.1.12-1-7 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide a means to expose to an authorized third-party application the status of the communication link specific to a user upon request from the authorizedthird-party (e.g. the communication link status can be used by the application to better recover lossy content during communication). | PR 9.11.6-3 |  |
| CPR 14.1.12-1-8 | Subject to operator’s policy and privacy considerations, the 6G system shall provide means (e.g. association of XR content with location and relevant positioning information) to ensure an optimal placement in viewer relative to the real environment (e.g. to ensure safety or optimal interactions between participants, etc.). | PR 9.12.6-1 | Check if covered by R19 metaverse Placement of content in real environment Does “privacy considerations” need alignment? |
| CPR 14.1.12-1-9 | Subject to operator’s policy and privacy considerations, the 6G system shall provide a means to synchronize media/flow and data for XR rendering. | PR 9.12.6-2 | Synchronization of media flow and data for rendering Does “privacy considerations” need alignment? Added NOTE from v1.1.0 QC: Are these examples components of the XR space of associated with the flows that need synchronization.? It is not clear what e.g. list are. |
| CPR 14.1.12-1-10 | Subject to operator’s policy and subscriber permission, the 6G system shall provide a means to synchronise, heterogeneous data flows (including media flows) from/to one or more UEs associated with a single user, and also among UEs associated with multiple users in a multimodal communication. | PR 9.12.6-4 PR 9.12.6-5 |  |
| CPR 14.1.12-1-11 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network may support exposure of predicted network status based on context information (e.g. user location, mobility path, and environmental knowledge to authorised 3rd parties. | PR 9.13.6-2 |  |
| CPR 14.1.12-1-12 | Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system (may include IMS) shall provide means to receive information about an application (e.g. previous, current or expected traffic characteristics) or information about target user service experience (e.g. for real-time multimedia applications) from authorized UE or 3rd party and leverage the information to deliver efficient communication service and meet the application’s performance requirements. | PR 9.14.6-1 PR 9.14.6-2 PR 9.17.6-1 PR 9.17.6-2 |  |
| CPR 14.1.12-1-13 | Based on operator’s policy, the 6G system shall enable a UE to indicate to the network coordination information (e.g. influencing policies for QoS handling) for transmitting the set of data flows for a multimodal communication session associated with an application. NOTE: One example of influencing policies can be alternative sets of data flows with different QoS requirements for the multimodal communication session, so that the network can dynamically determine optimal QoS handling, e.g. based on network resource availability. | PR 9.19.6-1 |  |
| CPR 14.1.12-1-14 | Subject to operator’s policy, the 6G system shall support mechanisms to adapt QoS of the media traffic in a interactive XR communication (e.g. holographic) to achieve consistent user experience under varying network conditions. | PR 9.8.6-2 | Adaptive QoS Holographic Cooms |
| CPR 14.1.12-1-15 | Subject to operator’s policy, the 6G network shall support QoS adjustment based on network status to ensure user experience. | PR 9.13.6-1 | Adaptive QoS is covered by other CPR. Adjustment of QoS based on network status QC: The NOTE is a solution instead of requirement so we propose to remove |
| CPR 14.1.12-1-16 | Subject to operator’s policy, the 6G system shall support the dynamic changes in QoS, when the traffic characteristic dynamically changes or is predicted to change, upon request by an authorised third party. | PR 6.61.6-1 | Duplicated in AI traffic table Dynamic QoS ZTE: It is suggested to merge PR 6.26.6-3, PR 6.52.6-3 with PR 6.61.6-1, which is proposed in AI traffic characteristics part. [Nokia-NEW]: Predicted by whom, NW or UE? AI might be able to predict something, but this is implementation specific. Point is that QoS can be changed under certain conditions and this is the requirement on network side. |

* * * End of Changes * * *