3GPP TSG SA WG 1 Meeting #113 	S1-261293

9-13 February 2026, Goa, India	(revision of S1-261271, 1262, 1046)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.14-2 – Localized network

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.8

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This pCR proposes to update the Localized network CPR tables in TR22.870 v1.1.0 with the greened CPRs from drafting sessions.

* * * First Change (All New CPRs)* * * *

Table 14.1.14-1 – Localized network

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.14-2-1 | Subject to operator’s policies, regulatory requirements and agreement between a PLMN operator and an authorized 3rd party, the 6G network shall support local authorization and authentication of the UEs owned by the authorized 3rd party (e.g. vertical industry) to access the 3GPP services available in the 6G localised network. NOTE: The realization or deployment of localised network may be an enhancement of NPN, PALS or other network topology options. | PR 11.10.6-1 PR 11.11.6-3 PR 11.10.6-2 |  |
| CPR 14.1.14-2-2 | Subject to operator’s policy, the 6G network shall support means to provide services continuity for the UEs in a localised network, when the connection between the localised network and the PLMN fails. NOTE 1: This requirement applies to scenarios where the execution of the 3GPP services involve, or rely on, coordination between the localised network and the PLMN. NOTE 2: Service continuity assumes services can be offered by parties in the localized network only. | PR 11.10.6-3 |  |
| CPR 14.1.14-2-3 | The 6G network shall support a mechanism to identify the services provided by a localised network or the PLMN for a UE owned by an authorized 3rd party. | PR 11.10.6-4 |  |
| CPR 14.1.14-2-4 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a localised network, e.g. adapting to the demand of users, 3rd party or the network operator. | PR 5.9.6.6-1 |  |
| CPR 14.1.14-2-5 | Subject to operator policies, an agreement between a PLMN operator and an authorized 3rd party and regulatory requirements, the 6G network shall support local authorization for PLMN’s subscribers to access 3GPP services provided by the authorized 3rd party via a localised network within a given area. | PR 5.9.6.6-2 |  |
| CPR 14.1.14-2-6 | Subject to operator policies, an agreement between a PLMN operator and an authorized 3rd party, for PLMN’s subscribers with local authorization to access localised networks, the 6G network shall support mechanisms to minimize the service interruption of the 3GPP services when the serving network changes between localised networks, or between the localised network and the PLMN. | PR 5.9.6.6-2 |  |