3GPP TSG-SA WG1 Meeting #113 	S1-261296

9-13 February 2026, Goa, India	(revision of S1-261xxx)

Source:	6G Study Rapporteurs

pCR Title:	EndOfDrafting Satellite Comms for approval

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides those CPRs from Satellite Comms that were agreeable for inclusion into the TR at the end of the meeting drafting sessions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the those CPRs that were “all green” at the end of the drafting sessions (Wednesday afternoon).

2. Reason for Change

To move the agreeable CPRs into the corresponding tables in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

### 14.1.11	Ubiquitous Connectivity

Editor’s Note: Table14.1.11-1 will include CPRs about the communication with satellite access and/or satellite backhaul; Table14.1.11-2 will include CPRs about determining position, velocity and/or time based on satellite-based positioning capability; Table14.1.11-3 will include CPRs relevant to other NTN such as HAPS, or other non-communication capabilities of satellite, etc.

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimise the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 |  |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 |  |
| CPR 14.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services to/from UEs and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 PR 8.6.6-3 |  |
| CPR 14.1.11-1-4 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 |  |
| CPR 14.1.11-1-5 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions. | PR 8.18.6-1 |  |
| CPR 14.1.11-1-6 | 6G system with satellite access shall provide a means to obtain communication service interruption information (e.g. expected interruptions to a media service) from an authorized third party. | PR 11.5.6-2 |  |
| CPR 14.1.11-1-7 | The 6G system with satellite access, shall be able to provide broadband services to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels). | PR 8.6.6-1 |  |

Table 14.1.11-2: Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-2-1 |  |  |  |

Table 14.1.11-3: Other aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-3-1 |  |  |  |

* * * End of Changes * * *