3GPP TSG-SA WG1 Meeting #113 	S1-261298

9-13 February 2026, Goa, India	(revision of S1-261xxx)

Source:	6G Study Rapporteurs

pCR Title:	EndOfDrafting Immersive Communications for approval

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides those CPRs from Immersive Communications that were agreeable for inclusion into the TR at the end of the meeting drafting sessions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the those CPRs that were “all green” at the end of the drafting sessions (Wednesday afternoon).

2. Reason for Change

To move the agreeable CPRs into the corresponding tables in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

### 14.1.12	Immersive Communication

Table 14.1.12-1: Immersive communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.12-1-1 | Subject to operator’s policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed QoS (e.g. current and/or predicted throughput within the boundary of the 6G system) for the associated ongoing session. | PR 9.3.6-3 |  |
| CPR 14.1.12-1-2 | Subject to operator’s policy, the 6G system shall be able to transmit error tolerant data traffic with adaptive QoS. NOTE: The QoS (e.g. reliability and latency) can be adjusted automatically based on SLA with authorised third party application. | PR 9.11.6-2 |  |
| CPR 14.1.12-1-3 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide a means to expose to an authorized third-party application the status of the communication link specific to a user upon request from the authorized third-party (e.g. the communication link status can be used by the application to better recover lossy content during communication). | PR 9.11.6-3 |  |
| CPR 14.1.12-1-4 | Subject to operator’s policy and subscriber permission, the 6G system shall provide a means to synchronise, heterogeneous data flows (including media flows) from/to one or more UE0s associated with a single user, and also among UEs associated with multiple users in a multimodal communication. | PR 9.12.6-4 PR 9.12.6-5 |  |
| CPR 14.1.12-1-5 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network may support exposure of predicted network status based on context information (e.g. user location, mobility path, and environmental knowledge to authorised 3rd parties. | PR 9.13.6-2 |  |
| CPR 14.1.12-1-6 | Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system (may include IMS) shall provide means to receive information about an application (e.g. previous, current or expected traffic characteristics) or information about target user service experience (e.g. for real-time multimedia applications) from authorized UE or 3rd party and leverage the information to deliver efficient communication service and meet the application’s performance requirements. | PR 9.14.6-1 PR 9.14.6-2 PR 9.17.6-1 PR 9.17.6-2 |  |
| CPR 14.1.12-1-7 | Based on operator’s policy, the 6G system shall enable a UE to indicate to the network coordination information (e.g. influencing policies for QoS handling) for transmitting the set of data flows for a multimodal communication session associated with an application. NOTE: One example of influencing policies can be alternative sets of data flows with different QoS requirements for the multimodal communication session, so that the network can dynamically determine optimal QoS handling, e.g. based on network resource availability. | PR 9.19.6-1 |  |

Table 14.1.12-2: Immersive communication related to IMS

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.12-2-1 |  |  |  |

* * * End of Changes * * *