3GPP TSG-SA WG1 Meeting #113 	draft S1-261312

9-13 February 2026, Goa, India	(revision of S1-261238, S1-261099)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.4-1 (Energy-related Aspects) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.4-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254160 (ZTE)

- S1-254250 (Qualcomm)

- November 2025 reflector comments

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Removed PR 5.8.7.6-1 since it in Table 14.1.7-1: NDT

- Removed PR 7.5.6-2 since it is in Table 14.1.10-1: ISAC

- Removed PR 6.16.6-1 since it is in the TR as approved CPR 14.1.8-1-3

This revision captures the discussions in the intial CPR discussions. Changes from the draft version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

This revision adds a CPR that was discussed and agreed in another session.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.4-1: Energy-related Aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.4-1-1 | Subject to regulatory requirements and subscriber permission, the 6G system shall jointly improve the network and UE energy efficiency compared to the 5G system, while meeting the service performance requirements. | PR 5.8.1.6-1 PR 5.8.1.6-4 PR 5.8.4.6-3 |  |
| CPR 14.1.4-1-2 | Subject to operator`s policy, regulatory requirements and subscriber permission, the 6G system shall be able to optimize the UE energy saving considering the current traffic conditions of the UE. | PR 5.8.4.6-2 |  |
| CPR 14.1.4-1-3 | Subject to operator`s policy regulatory requirements and subscriber permission, the 6G system shall support mechanisms to optimize the energy consumption of a UE via enabling task offloading (e.g. XR rendering/AI inference), considering the UE battery life. | PR 5.8.5.6-1 |  |
| CPR 14.1.4-1-4 | Subject to operator’s policy, the 6G system shall support network energy management policies at slice level that - define a maximum energy credit limit at slice level for services without QoS criteria, - support a mechanism to perform slice energy credit limit control for services without QoS criteria. NOTE: The result of the credit control is not specified by this requirement. | PR 5.8.3.6-1 PR 5.8.3.6-2 |  |
| CPR 14.1.4-1-5 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall support a means for a group of cooperating UEs to reduce the energy consumption for one or more UEs of the group for the communication service whilst meeting requested service performance. | PR 5.8.8.6-1 |  |
| CPR 14.1.4-1-6 | Subject to operator’s policy, the OAM shall use mechanisms (e.g. AI capabilities), to assist with network energy efficiency and carbon emissions reduction. | PR 6.16.6-1 | Discussed in another session and agreed as CPR 14.1.8-1-3 |
| Orig PR | The 6G network shall be able to support mechanisms to improve UE energy efficiency when providing services to the subscribers. | PR 5.8.1.6-2 | Provided for info |
| Orig PR | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: The 6G network takes into account this preference when improving UE energy efficiency. Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | Provided for info |
| Alt 2 CPR 14.1.4-1-2 (10 Nov reflector comments) | The 6G system shall be able to support mechanisms (including a mechanism to consider user preference) to improve UE energy efficiency when providing services to subscribers. Examples of such preferences - prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 | UE Energy Efficiency Should “user preference” be “subscriber preference? How does this relate to subscriber permission? Huawei: Suggest to remove the example of preference, which is solution oriented. Nokia: prefer ZTE proposal above |
| Orig PR | The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS, targeted service area and/or targeted set of users. | PR 5.8.2.6-1 | Provided for info |
| Orig PR | The 6G network shall be able to support means to provide the network operator an indication of the energy consumed by the available access networks for the provision of a one-to-many service. NOTE 1: The network operator may take into account this energy consumption information when deciding how to deliver the service. NOTE 2: The energy consumption information relates to the access network’s energy needed (taking into account the energy already consumed by the network over its life duration) for the delivery of a given one-to-many service. | PR 5.8.2.6-2 | Provided for info |
| Alt CPR 14.1.4-1-3 (Nokia) | The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through estimation) the energy consumed for the operation of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS/QoE, targeted service area and/or targeted set of UEs. | PR 5.8.2.6-1 PR 5.8.2.6-2 | Access Energy Consumption Huawei: suggest to remove not clear about the relationship between the energy consumed calculation and the one-to-many service. Need clarify. |
| Orig PR | Subject to operator’s policy and regulatory requirements, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time), with the respective information on predicted energy consumption and carbon equivalent emissions per scenario. NOTE: The following network energy information about energy consumption and carbon equivalent emissions can be calculated and/or estimated. | PR 5.8.6.6-1 | Provided for info |
| Orig PR | Subject to regulatory requirements, the 6G system shall be able to optimise the network energy saving considering the current traffic conditions of the network. | PR 5.8.4.6-1 | Provided for info Huawei: suggest to keep the original wording. |
| CPR 14.1.4-1-6 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable network energy saving methods targeting different scenarios (e.g. different working time) based on respective information regarding predicted energy consumption, carbon equivalent emissions per scenario, and/or the current network traffic conditions. NOTE: This requirement describes scenario where the user experience could be impacted. | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving Huawei: if merge PR 5.8.6.6-1, PR 5.8.4.6-1, we prefer this one. |
| Orig PR | Subject to operator’s policy and regulatory requirementsregulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related with to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Provided for Info |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party. | PR 6.37.6-1 | Provided for Info Potential dupe with Table 14.1.8-5 – AI model training and inferencing |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party | PR 6.37.6-2 | Provided for Info Potential dupe with Table 14.1.8-5 – AI model training and inferencing |
| Alt CPR 14.1.4-1-7 (ZTE/S1-254160) | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to expose to an authorised third-party the network energy consumption information and carbon equivalent emissions (e.g. derived through monitoring and/or estimation) related to 3GPP services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 PR 6.37.6-1 PR 6.37.6-2 | Network (Sensing, AI, computing) related Energy Consumption Huawei: not clear how to expose the energy consumption for service. Need clarify. Huawei: not clear how to monitor the energy consumption for service. Need clarify. Nokia: prefer to avoid the word monitor, which may not always be achievable (prefer original CPR) |
| Orig PR | The 6G network shall be able to collect energy related data of the Service Hosting Environment. | PR 6.24.6-1 | Provided for info Potential dupe with Table 14.1.9-1 – General Computing requirements |
| Orig PR | The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier. | PR 6.24.6-3 | Provided for info |
| Alt NEW CPR 14.1.4-1-CC | The 6G network shall be able to collect energy source (provided to SHE) related information from the energy management system, which is outside of 3GPP networks. | PR 6.24.6-1 Is PR 6.24.6-3 in this CPR or in the next one (potential dupe)? | ZTE: According to the use case of 6.24, revise the “energy related data” to “energy related information”: Huawei: need to clarify what is the energy related information of SHE.what the relationship between SHE and energy management system?. Align the handling on SHE. E.g., clarifying Service Hosting Environment (excluding RAN) [InterDigital – new CPR wording for clarity and completeness. Receiving data from outside 3GPP is not in scope for us unless it can be correlated for use] |
| New CPR 14.1.4-1-FF | The 6G network shall be able to support mechanisms to select computing resources in the SHE (excluding RAN) considering energy related information from authorized 3rd party (e.g. energy management system). | PR 6.24.6-3 Is this to be consolidated separately or in the CPR above (potential dupe)? | According to the use case of 6.24, revise it as: Huawei: clarify what is the energy supplier, and what is the relationship to 3GPP. ZTE: offline discussed with Softbank, and revised the CPR to resolve ZTE comment. |
| Alt New CPR 14.1.4-1-## | Huawei Proposal Subject to operator’s policy, the 6G network shall provide means to provide 3GPP services (e.g. sensing, AI, computing etc.) considering energy efficiency. InterDigital Proposal Subject to operator’s policy, the 6G network shall provide means to consider energy consumption information of carbon equivalent emissions (e.g. of network resources) when providing 3GPP services (e.g. sensing, AI, computing etc.) targeting different scenarios. | PR 5.8.6.6-3 | Huawei: wording suggestion. [InterDigital: alternative adding clarification of emissions] Interdigital version provides more details on the use case |

* * * End of Changes * * *