3GPP TSG-SA WG1 Meeting #113 	S1-261318

9-13 February 2026, Goa, India	(revision of S1-261312, S1-261238, S1-261099)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.4-1 (Energy-related Aspects) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.4-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254160 (ZTE)

- S1-254250 (Qualcomm)

- November 2025 reflector comments

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Removed PR 5.8.7.6-1 since it in Table 14.1.7-1: NDT

- Removed PR 7.5.6-2 since it is in Table 14.1.10-1: ISAC

- Removed PR 6.16.6-1 since it is in the TR as approved CPR 14.1.8-1-3

This revision captures the discussions in the intial CPR discussions. Changes from the draft version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

This revision adds a CPR that was discussed and agreed in another session.

This revision captures the (to date) unagreed CPRs for this table. CPRs that were “all greem” (aka agreeable) were moved to S1-261206.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.4-1: Energy-related Aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Orig PR | Subject to operator’s policy and regulatory requirements, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time), with the respective information on predicted energy consumption and carbon equivalent emissions per scenario. NOTE: The following network energy information about energy consumption and carbon equivalent emissions can be calculated and/or estimated. | PR 5.8.6.6-1 | Provided for info |
| Orig PR | Subject to regulatory requirements, the 6G system shall be able to optimise the network energy saving considering the current traffic conditions of the network. | PR 5.8.4.6-1 | Provided for info Huawei: suggest to keep the original wording. |
| CPR 14.1.4-1-10 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable network energy saving methods targeting different scenarios (e.g. different working time) based on respective information regarding predicted energy consumption, carbon equivalent emissions per scenario, and/or the current network traffic conditions. NOTE: This requirement describes scenarios where the user experience could be impacted. | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving Huawei: if merge PR 5.8.6.6-1, PR 5.8.4.6-1, we prefer this one. |
| Orig PR | Subject to operator’s policy and regulatory requirementsregulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related with to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Provided for Info |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party. | PR 6.37.6-1 | Provided for Info |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party | PR 6.37.6-2 | Provided for Info |
| CPR 14.1.4-1-11 | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to expose to an authorised third-party the network energy consumption information and carbon equivalent emissions (e.g. derived through monitoring and/or estimation) related to 3GPP services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 PR 6.37.6-1 PR 6.37.6-2 | Network (Sensing, AI, computing) related Energy Consumption Huawei: not clear how to expose the energy consumption for service. Need clarify. Huawei: not clear how to monitor the energy consumption for service. Need clarify. Nokia: prefer to avoid the word monitor, which may not always be achievable (prefer original CPR) |
| CPR 14.1.4-1-12 | The 6G network shall be able to collect the information related to energy supply (e.g. energy capacity, energy consumed by the SHE) from the energy management system, which is outside of 3GPP network. | PR 6.24.6-1 | ZTE: According to the use case of 6.24, revise the “energy related data” to “energy related information”: Huawei: need to clarify what is the energy related information of SHE.what the relationship between SHE and energy management system?. Align the handling on SHE. E.g., clarifying Service Hosting Environment (excluding RAN) [InterDigital – new CPR wording for clarity and completeness. Receiving data from outside 3GPP is not in scope for us unless it can be correlated for use] |
| CPR 14.1.4-1-13 | The 6G network shall be able to support mechanisms to select computing resources in the SHE (excluding RAN) considering energy related information from authorized 3rd party (e.g. energy management system). | PR 6.24.6-3 | According to the use case of 6.24, revise it as: Huawei: clarify what is the energy supplier, and what is the relationship to 3GPP. ZTE: offline discussed with Softbank, and revised the CPR to resolve ZTE comment. |
| CPR 14.1.4-1-14 | Subject to operator’s policy, the 6G network shall provide 3GPP services (e.g. sensing, AI, computing) considering energy consumption information and carbon equivalent emissions (e.g. related to network resources). | PR 5.8.6.6-3 | Huawei: wording suggestion. |

* * * End of Changes * * *