13GPP TSG-SA WG1 Meeting #113 	S1-261323

9-13 February 2026, Goa, India	(revision of S1-261320. S1-2611311, S1-261239, S1-26110)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.3-1 (Resilience) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.3-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254092 (China Telecom)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

- S1-254159 (NTT DOCOMO)

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Added PRs 5.6.1.2-1 and 5.6.3.6-1 from Table 14.1.7-2: OAM

This revision captures the discussions in the intial CPR discussions. Changes from the draft version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

- With the “greening” of PRs 5.9.4.2-1, 5.9.5.6-1 and 5.9.5.6-2, removed proposals that included them to merge. That left PR 5.6.2.6-1 to be discussed/consolidate as a separate CPR.

- Removed PRs 5.6.2.6-1 and PR 5.6.2.6-2. They will be addressed in Table 14.1.14-2 – Localized network.

This revision adds in CPRs from Table Table 14.1.14-5 Ind/Vertical) to be discussed/consolidated into this Table.

This revision captures the (to date) unagreed CPRs for this table. CPRs that were “all greem” (aka agreeable) were moved to S1-261206.

This revision includes changes proposed to PR 5.6.2.6-1.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.3-1: Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.3-1-8 | Subject to operator’s policy and regulatory requirements, 6G network should support suitable means to minimize the degradation of services in case of abnormal transmission over the user plane path (e.g.between the public network and edge network). | PR 5.6.1.2-2 | Edge resilience From S1-254250 [QC] clarify “stablity”…change to resilience? |
| CPR 14.1.3-1-9 | Subject to operator’s policy and regulatory requirements, the 6G system shall support a means for an MNO to exchange information with a DSO to improve availability of regulated services (e.g. PWS [62], emergency call [58]), e.g. in case of power outage. | PR 11.18.6-1 | Was CPR 14.1.14-5-3 (Moved from Table 14.1.14-5 Ind/Vertical) |
| CPR 14.1.3-1-10 | Subject to operator’s policy and service level agreements, the 6G system shall enable operators to provision network services on-demand, e.g. in response to an urgent event (e.g. disaster, emergency or DDoS event), in a given area during a specific time period. NOTE: The enabled functionalities can include for example, - prioritising voice service in a residential community, or small data transfer service in an IoT based farmland. - capability to manage local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 | Localised Network? provision network services on-demand |
| CPR 14.1.3-1-11 | Subject to operator’s policy and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand, but is available from the PLMN of the same operator | PR 5.6.2.6-2 | Localised Network? authorize a local UE accessing service in PLMN |

* * * End of Changes * * *