3GPP TSG-SA WG1 Meeting #113 	S1-2611353

9-13 February 2026, Goa, India	(revision of S1-261xxx)

Source:	6G Study Rapporteurs

pCR Title:	End of Meeting Thursday - System for approval

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides those CPRs from System & Operational Aspects that were agreeable for inclusion into the TR by the end of the meeting on Thursday.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the those CPRs that were “all green” at the end of the drafting sessions (Wednesday afternoon).

2. Reason for Change

To move the agreeable CPRs into the corresponding tables in the TR.

This TDOC contains those PRs that were deteremined to be agreeable (i.e., “green”) for inclusion in the TR during the closing Plenary sessions.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

### 14.1.1	Basic Services and Capabilities

Table 14.1.1-1: Support for legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.1-1-1 | The 6G system shall be able to support the following services: - Broadcast and Multicast Services, ref TS 22.261 [14], - Emergency Services, ref TS 22.101 [58], - IMS Multimedia Telephony Service, ref TS 22.173 [59], TS 22.228 [138], TS 22.261 [14], - Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312], - Location and positioning services, ref TS 22.071 [61], TS 22.261 [14], - Message service, ref TS 22.262 [57], - Mission Critical Services, i.e. MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Multimedia communication services, ref TS 22.101 [58], TS 22.173 [59], TS 22.261 [14], - Multimedia Priority Service (MPS), ref TS 22.153 [63], - Public Warning System (PWS), ref TS 22.268 [62], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Short Message Service (SMS), ref TS 22.101 [58], and - Other regulatory services, based on regional/national regulatory requirements. NOTE: The following services do not need to be supported by the 6G system: CS related telephony services, e.g. CS Fallback, CS based voice call. | Clause 5.4.2 |  |
| CPR 14.1.1-1-2 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined e.g. in TS 22.011 [60], TS 22.071 [61], TS 22.101 [58], TS 22.104 [64], TS 22.115 [68], TS 22.125 [35], TS 22.137 [6], TS 22.153 [63], TS 22.156 [28], TS 22.173 [59], TS 22.185 [65], TS 22.186 [66], TS 22.228 [138], TS 22.261 [14], TS 22.262 [57], TS 22.263 [67], TS 22.289 [69], TS 22.368 [71], and TS 22.468 [70]. NOTE 1: Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in those requirements. NOTE 2: The requirements in the above standards that mention E-UTRAN, UTRAN, or GERAN are not automatically included. | Clause 5.4.3 |  |
| CPR 14.1.1-1-3 | Subject to operator’s policy, the 6G system shall support mobility procedures between the core network (CN) of the 6G system and a 5G core network with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). | Clause 5.2 |  |
| CPR 14.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures between the core network of the 6G System and the evolved Packet Core Network (EPC) with minimum impact to the user experience (e.g. QoS, QoE). | Clause 5.2 |  |
| CPR 14.1.1-1-5 | Requirements in TS 22.261 [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, shall apply similarly to 6G System with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. NOTE 3: Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in those requirements. | Clause 5.2 |  |
| CPR 14.1.1-1-6 | Requirements in TS 22.261 [14] related to non-3GPP access support in 5GS shall apply to the 6G system. | Clause 5.3 |  |

Table 14.1.1-2: Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.1-2-1 | The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints). | PR 5.7.4.2-2 |  |
| CPR 14.1.1-2-2 | The 6G system shall provide mechanisms to support efficient bandwidth utilisation by a FWA CPE. NOTE: A FWA Customer Premises Equipment (CPE) is used to connect to the network, like any other UE, using a 3GPP access. | PR 5.7.1.2-5 |  |
| CPR 14.1.1-2-3 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and provide operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 |  |
| CPR 14.1.1-2-4 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 5: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 6: This requirement only impacts core network. | PR 5.9.7.6-1 |  |
| CPR 14.1.1-2-5 | The 6G system should support potential enhancement of network slicing, e.g.: - Create and delete a network slice in an optimised manner, - Modify (e.g. reconfigure or change resources) a network slice efficiently, and - Improve the mechanism to select, reselect and access network slice(s). | PR 5.7.5.2-1 |  |
| CPR 14.1.1-2-a | Subject to operator’s policy and regulatory requirements, the 6G network shall support a mechanism to enable home operator to authorize UE to access authorised 6G services from home operator’s selected partner operators when UE only has subscription of home operator. | PR 5.9.11.6-1 |  |
| CPR 14.1.1-2-b | Subject to operator’s policy and regulatory requirements, the 6G network shall support 6G services provided by home operator’s partner operators to a home operator’s subscriber. | PR 5.9.11.6-2 |  |
| CPR 14.1.1-2-c | Subject to operator’s policy and regulatory requirements, the 6G network shall allow home operator to be aware of, to use, and to select the set of 6G services exposed by its partner operators to the home operator subscribers. NOTE: The relationship between partner operators and home operators does not assume roaming agreement between operators. | PR 5.9.11.6-3 |  |
| CPR 14.1.1-2-d | Subject to operator’s policy regulatory requirements and subscriber permission, the 6G network shall support means to provide users with differentiation of QoS based on users’ digital identity information issued by a trusted third party and subscriber’s information. | PR 5.5.9.6-1 |  |

Table 14.1.1-3: Device Support

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.1-3-1 |  |  |  |

Editor’s Note: The title and placement of Table 14.1.1-4 needs to be revisited at the next meeting.

Table 14.1.1-4: Non-AI or Immersive related IMS enhancements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.1-4-1 | The Multimedia Telephony Service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 |  |
| CPR 14.1.1-4-2 | The 6G and IMS systems shall enable the Multimedia Telephony Service to support an IP-CAN in the 6GS. | PR 5.7.2.2-1 |  |
| CPR 14.1.1-4-3 | Subject to operator’s policy, the IMS shall support means to minimise impact on the user experience (e.g. call failure) when a UE is engaged in one IMS session where more than one IMS service is triggered. NOTE: Typical example of such situation can be a IMS media related service (e.g. play tone, play announcement) in conjunction with an IMS data channel based service. | PR.5.7.7.6-1 |  |

* * * Next Change * * *

### 14.1.2	Security and Privacy

Table 14.1.2-1: Security and Privacy

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.2-1-1 |  |  |  |

Table 14.1.2-2: Lawful Interception

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.2-2-1 |  |  |  |

* * * Next Change * * *

### 14.1.3	Resilience

Table 14.1.3-1: Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.3-1-1 |  |  |  |

* * * Next Change * * *

### 14.1.4	Energy-related aspects

Table 14.1.4-1: Energy-related aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.4-1-x | The 6G network shall be able to collect the information related to energy supply (e.g. energy capacity, energy consumed by the SHE) from the energy management system, which is outside of 3GPP network. | PR 6.24.6-1 |  |

* * * Next Change * * *

### 14.1.5	Data Collection and Consumption

Table 14.1.5-1: Data Collection and Consumption

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.5-1-1 |  |  |  |

Table 14.1.5-2: Data Collection and Consumption - Others

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.5-2-a | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide secure means to store the 6G system data for a UE, e.g. UAV or a UAM aircraft upon request (e.g. including service type, storage duration, time expiry) and expose the information derived from stored 6G system data to an authorized 3rd party application. NOTE: The 6G system data can be the processing data derived from a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. NOTE: Sensing data is not exposed, only sensing results. Some exceptions can apply e.g. regulatory and public safety scenarios. | PR 11.7.6-2 PR 11.7.6-1 |  |

* * * Next Change * * *

### 14.1.6	Charging

Table 14.1.6-1: Charging

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.6-1-TBD | The 6G network in conjunction of IMS shall support charging information collection for the use of Intelligent Communication Assistant, incoming call answering and immersive calling (e.g. when initiating and terminating usage) based on mechanisms (e.g. AI capabilities). | PR 6.11.6-2 PR 6.22.6-2 PR 9.10.6-3 |  |

* * * Next Change * * *

### 14.1.7	Other operational aspects

Table 14.1.7-1: NDT

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.7-1-1 |  |  |  |

Table 14.1.7-2: OAM

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.7-2-1 |  |  |  |

* * * End of Changes * * *