3GPP TSG SA WG 1 Meeting #113 	S1-261355

9-13 February 2026, Goa, India	(revision of S1-261263)

Source:	Nokia (ISAC KPI moderator)

pCR Title:	Pseudo-CR on consolidated ISAC KPI table

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.4

Document for:	Approval

Contact:	hideaki dot takahashi at nokia dot com

Abstract: This pCR provides the updated ISAC KPI table to reflect the email discussion outcome.

1. Introduction

An email discussion took place before SA1 #113 meeting to categorize ISAC KPI tables, based on Rel-19 ISAC table captured in TS 22.137.

2. Reason for Change

To update the consolidated ISAC KPI table by reflecting KPI categorization discussed by email in S1-261028.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v1.1.0.

* * * First Change * * * *

### 14.2.2	ISAC

The 6G system shall be able to provide sensing with the following performance requirements:

NOTE 1: The definitions of the terms used in the following KPI tables are defined in [6].

Table 14.2.2-1: Consolidated performance requirements for Integrated Sensing and Communication.

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safety Assistance for vulnerable pedestrians (Use case 7.3) | Outdoor (Crossing) | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤ [500] | ≤ [0.1] | ≤ [5] | ≤ [5] |
| High topology mapping (Use case 7.4) | Outdoor | N/A | 0.10 | 0.10 | - |  | [0.4] | - | 50 | ≤ 0.2 | ≤10 | <1 |
| Low-altitude UAV supervision: UAV intrusion detection (Use case 7.5) | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 |
| Environment object reconstruction (Use case 7.6) | Outdoor (Building) | 95 | [0.5-5] | [0.5-5] | N/A | N/A | [0.5] | N/A | [10,000-600,000] | [10-60] | [1-5] | [1-5] |
|  | Outdoor (Vehicle) | 95 | 0.5 | 0.5 | 1.5 | N/A | [0.5] | [0.5] | 100 | 0.1 | 5 | 5 |
| Road digitalization (Use case 7.7) | Outdoor Sensing target density: ≤750 vehicle Per 1000 m x 24 m; ≤100 pedestrian per 1000 m x 24 m (NOTE1) | 95 | 0.5 (vehicle) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
|  | Outdoor Sensing target density: ≤72 vehicles per 1000 m x 24 m (NOTE2) | 95 | 3 | N/A | 3 | N/A | 1 | 1 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
| Ship detection and tracking (Use case 7.9) | Detection of ship on the near shore waters and offshore waters | 95 | ≤100 | NA | NA | NA | ≤100 | NA | 5000 | 1 | 5 | 5 |
|  | Detection of ship on the harbour entrances, harbour approaches, and port. and coastal waters | 95 | ≤10 | NA | NA | NA | ≤10 | NA | 5000 | 1 | 5 | 5 |
|  | Detection of ship on the inland waterway (e.g. river, lake) | 95 | ≤2~10 | NA | NA | NA | ≤2 | NA | 5000 | 1 | 5 | 5 |
| Enhanced XR navigation (Use case 7.13) | 10 m2 | 99.9 | ≤0.5 | 0.5 | 0.5 | N/A | 0.1- 1 | 0.5 | ≤100 | 0.1 - 1 | 1 | 1 |
| Collaborative Robots (Use case 7.14) | Indoor/Outdoor | [95] | [≤ 0.1] | [≤ 0.1] |  |  |  |  | N/A |  |  |  |
| Infrastructure collapse monitoring (Use case 7.15) | Outdoor (e.g. Detecting sudden collapse on infrastructure such as highway, railway, road, flyover, rural areas, farmland) | [95] | [4] | [N/A] | [N/A] | [N/A] | [4] | [N/A] | TBD | [ 1] | [1~2] | [1~2] |
| UAV takeoff and landing (Use case 7.16) | Outdoor | [95] | [0.1] | [0.1] | [1] | [1] | [0.5] | [1] | [100-500] | [0.2] | [5] | [5] |
| Safe and economic UAV transport (Use case 7.18) | 99.999 | 99.99 | ≤ 50 | 50 | 5 | 5 | 10 | 0.5 | ≤ 100 | 0.1 - 1 | 0.1 | 1 |
| Structural health monitoring (Use case 7.22) | Outdoor (e.g. detection of corner reflectors on bridges, buildings in urban scenario) | 95 | 0.1 | 0.1 | N/A | N/A | N/A | N/A | 5000 | 60 | 5 | 5 |
| UAV detection, classification and counting (Use case 7.23) | outdoor | 95 | 1 | 1 | 1 | 1 | [0.3 – 1] | 1 | N/A | ≤1000 | ≤1 | 2 |
| Hand Gestures Recognition (Use case 7.24) | Indoor (Factory Environment) | 99 | 0.1 | 0.1 | n/a | n/a | [0.3 to 0.5] | n/a | ≤ 20 | ≤ 0.02 | ≤ 1 | ≤ 10 |
| Head Gestures Recognition (Use case 7.24) | Indoor (Factory Environment) | 99 | 0.1 | 0.1 | [0.3] | [0.3] | [0.01] | [0.3] | ≤ 20 | ≤ 0.02 | ≤ 1 | ≤ 10 |
| Body and Limb Gestures Recognition (Use case 7.24) | Indoor (Factory Environment) | 99 | 0.1 | [0.1 | n/a | n/a | 0.1 | n/a | ≤ 20 | ≤ 0.02 | ≤ 1 | ≤ 10 |
| Sensing of Surroundings (Use case 7.24) | Indoor (Factory Environment) | 99 | 0.1 | n/a | 0.1 | n/a | [0.5 to 1] | 1.5 | ≤100 | 0.1 | < 10 | < 10 |
| Detection of Pathways, Openings (Use case 7.27) | Indoor (Factory Environment) | 99 | 0.25 | 0.25 | 0.1 | n/a | [0.75] | 1.5 | ≤100 | 0.1 | < 10 | < 10 |
| NOTE 1: One crossroad area is approximated by 1000 m dual three-lane carriageway (24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). |  |  |  |  |  |  |  |  |  |  |  |  |

Table 14.2.2-3: Performance requirements for  combined communication, sensing and positioning services

|  | Communication KPIs |  |  |  | Positioning KPIs | Sensing KPI |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | User experienced data rate | E2E latency | Communication Service availability | Connection density | Location accuracy | Sensing accuracy |
| Combined communication, sensing and positioning services (UC 7.x Network assisted smart transportation) | [1-10 Mb/s] | [20 ms] | 99.99% (note 1) | 104 devices/km2 | [1m] * [1m] * [1m] with 90% probability (note 2) | category 2 or 3 (notes 2, 3) |
| NOTE 1: within service volume NOTE 2: within 99% of the service volume NOTE 3: Category 2 or 3 in Table 6.2-1 in TS 22.137 [6] |  |  |  |  |  |  |

### 14.2.X	Other Use Cases

Table 14.2.X-1: Additional performance requirements for sensing service category 12 and 13 in digital twin of environment.

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS availability: target value (%) | CS reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area | Remarks |
| 99.999 | 4 h | < 20 ms | Sensing Result: 1 Mbit/s Sensing Data: 2.5 Gbit/s | 15 kByte 50 kByte | 100 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Sensing Information: Robot third party application |
| 99.999 | 4 h | 5 ms to 10 ms | 10 Mbit/s per video camera stream | 4 MByte | 33 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Video Sensing: (Robot third party application) |
| 99.999 | 4 h | 5 ms to 10 ms | 10 Mbit/s per stream | 1 MByte to 5 MByte | 100 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Application data derived from aggregated sensing results: (third party application Robots) |
| NOTE: References are for communication between robot and third party application [373] and for video sensing [372]. |  |  |  |  |  |  |  |  |  |  |