3GPP TSG SA WG 1 Meeting #113	S1-261363

9-13 February 2026, Goa, India	(revision of S1-261146, 1127, 1073)

Source:	Nokia (Moderator)

pCR Title:	Consolidation of KPI requirements on Massive Communication

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.7

Document for:	Approval

Contact:	Feifei Lou, feifei.lou@nokia.com

Abstract: Update the consolidation of massive communication KPI table based on TR 22.870 v1.1.0.

1. Introduction

S1-254322 already updated the table based on the approved S1-254423, but was not treated in SA1#112. S1-260045 was for information in SA1#112 ad hoc e-meeting. This contribution has aligned with the editorial changes in the latest TR.

2. Summary of Changes

The following changes are applied.

- Add Huawei’s proposal as an alternative.

1146:

- Put bit rate for Smart Grid (monitor and control) numbers in square brackets.

- Add NOTE 6 and 7.

1363:

- Clean up of the agreed removals.

3. Conclusions

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

* * * First Change * * * *

### 14.2.5	Massive Communication

Table 14.2.5-1: KPI for M-IoT

| Profile | Bit rate downlink (Mbit/s) | Bit rate uplink (Mbit/s) | End-to-end latency: maximum (ms) (NOTE 4) | Payload size | # of UEs Connection (UEs/km2) | Communication service availability: target value (NOTE 4) | Transfer Interval (sec) (NOTE 5) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Smart Grid (monitor and control) (UC 10.3) | Peak [5 to 10] (NOTE 6) | Peak [5 to 10] (NOTE 6) | 100 (NOTE 2) | 0.5Kbyte (NOTE 1) | 10 000 | > 99.99 | 60 |
| Smart Grid (software download) (UC 10.3) | Peak [5 to 10] (NOTE 6, 7) |  | - | Up to 20MB (NOTE 3) | 10 000 | > 99.99 | - |
| NOTE 1: Typical message sizes are 500-byte payload meter readings and control. NOTE 2: Reference for requirement of 100ms EPRI report “Falling Conductor Protection” [416]. NOTE 3: up to 20MB for Firmware upgrade. NOTE 4: As defined in TS 22.261 [14] NOTE 5: As defined in TS 22.104 [64] NOTE 6: The bit rate performance requirement will need to be confirmed by RAN working groups. NOTE 7: The peak 5 Mbit/s would allow all sensors according to the target density to be updated in 40 days given 20% network load for software download. |  |  |  |  |  |  |  |

* * * End of Changes * * * *