3GPP TSG SA WG 1 Meeting #113 	S1-261375

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Proposed CPRs for Agreement – Satellite Comms/Immersive Comms

Draft Spec:	3GPP TR 22.870 v1.0.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides those CPRs that were agreeable for inclusion into the TR at the end of the Thursday’s plenary meeting session.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the those CPRs that were “all green” at the end of the Thursday plenary session.

2. Reason for Change

To move the agreeable CPRs into the corresponding tables in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * * *

### 14.1.10	Integrated Sensing and Communication (ISAC)

Table 14.1.10-1: ISAC

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.10-1-1 | Subject to operator’s policy, the 6G Network shall provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics to be counted. | PR 7.23.6-1 |  |
| CPR 14.1.10-1-2 | Subject to operator’s policy and regulatory requirements, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 |  |
| CPR 14.1.10-1-3 | Subject to operator’s policy and regulatory requirements, the 6G system shall support mechanisms to provide sensing service with a given sensing target density. | PR 7.7.6-1 |  |
| CPR 14.1.10-1-4 | The 6G network shall be able to provide a means to securely expose the feasibility to deliver the sensing result within delivery time window, as requested by an authorized third party, NOTE: The delivery time window depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.8.6-3 |  |
| CPR 14.1.10-1-5 | Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network shall support the ability to derive a sensing result based on the stored sensing data and ensure that only authorised entities are able to access sensing results. | PR 7.11.6-1 PR 7.12.6-1 |  |
| CPR 14.1.10-1-6 | The 6G Network shall provide suitable mechanisms for the exposure of sensing results together with other types of traffic in coordinated manner (e.g. audio, video, haptics) to the sensing service consumer. | PR 7.14.6-2 |  |
| CPR 14.1.10-1-7 | Subject to operator’s policy, regulatory requirements, the 6G network shall be able to provide sensing results to a UE or an application on a UE, where the UE is authorised by the network operator to use the sensing result for a specific service (e.g. to combine sensing result). | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 |  |
| CPR 14.1.10-1-8 | The 6G system shall be able to support energy-efficient sensing operations (e.g. efficient use of sensing resources). | PR 7.5.6-2 PR 7.8.6-2 |  |
| CPR 14.1.10-1-9 | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to provide means for a mobile network operator to monitor and validate the sensing result against the assistance information (e.g. real location of the sensing target) if available from an authorized 3rd party. | PR.7.26.6-1 |  |
| CPR 14.1.10-1-10 | Subject to operator’s policy and regulatory requirements, the 6G network shall allow mobile operators to provide sensing services using a shared RAN (supporting 3GPP (e.g. 5G or 6G) wireless sensing). | PR 5.7.10.6-1 |  |
| CPR 14.1.10-1-11 | Subject to operator’s policy and regulatory requirements, the 6G system shall provide a sensing service to predict the characteristics of sensing targets (e.g. shape and size of the vehicle). | PR 7.6.6-2 |  |

* * * End of Changes * * *