3GPP TSG-SA WG1 Meeting #113 	S1-261389

9-13 February 2026, Goa, India	(revision of S1-261304, S1-261301, 261117, S1-260112)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.1-2 (Enhancements to legacy services & capabilities) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the Table 14.1.1-2 for continued consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-260112 was the final end of discussion status during SA1 #112 Ad Hoc-2. It is provided here for continued discussions.

- Green indicates there was consensus in SA1 #112 to include the CPR for inclusion into the TR.

- Yellow indicates that there the CPR was discussed, and some additional work is needed.

- Red indicates that this proposal was discussed, and it was decided to not pursue.

NOTE: below the proposed consolidation table, is the table of the removed CPRs (with reasons). It is provided here for information & convenience.

From S1-250112, original PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up Table numbering (to align w/TR) and CPR numbering

- Moved PR 8.21.6-1 to Table 14.1.11-3: Other aspects

- Moved PR 5.5.10.2-1 to Table 14.1.2-1: Security and Privacy to be discussed there.

- Added PR 5.9.1.3-1 from Table 14.1.11-3: Other aspects (Ubiquitous)

- Removed PRs on 6G LAN/Localised Network. Will be discussed in Industry/Vertical consolidatiaon.

- Added proposal to PR 8.20.6-1 from S1-261055 (CATT).

This revision captures changes made during the discussions in the meeting. Changes from the last version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

- Removed PRs 5.7.4.2-1 (NW sharing) and 5.9.1.3-1 (Femtocells) from table and from consolidation). It was decided that these PRs are already covered in 5G.

- Removed text after “End of Changes”.

TDOC 1340 contains the (to date) unagreed CPRs for this table. CPRs that were “all green” (aka agreeable) were moved to a separate pCR.

TDOC 1389 contains the agreed (aka green) CPRs for this table. Changes accepted, comments and green coloring removed.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.1-2: Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.1-2-x | Subject to regulatory requirements, the 6G system shall provide means for network operators to optimise network capabilities to enhance FWA service in the relevant bands, based on device characteristics (e.g. FWA, stationary devices) and service needs (e.g. bandwidth, latency). | PR 5.7.1.2-1 PR 5.7.1.2-2 |  |
| CPR 14.1.1-2-y | The 6G system shall support means for the network to be aware of user device and service characteristics (e.g. support stationary devices, data rate, latency) to provide FWA services. NOTE: the above may depend on information (including predicted changes of service characteristics) provided by the application or an authorised 3rd party service provider. | PR 5.7.1.2-3 PR 5.9.8.2-1 PR 5.9.8.2-2 |  |
| CPR 14.1.1-2-z | Subject to operator’s policy, the 6G network shall be able to provide a suitable means to inform UEs of communication service availability for UEs to determine whether to use 3GPP access or non-3GPP access. NOTE: This requirement is intended for first network connection attempt only and is not applicable for ongoing sessions. | Clause 5.2 |  |

* * * End of Changes * * *