3GPP TSG SA WG 1 Meeting #113 	S1-261394

9-13 February 2026, Goa, India	(revision of S1-261346, 1337, 1324, 1304, 1041)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.8-5 AI model training and inferencing

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.3

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This table contains the CPR which are fully or partly marked in yellow during the drafting session and for further discussion.

* * * First Change (All new CPRs) * * * *

Table 14.1.8-5 – AI model training and inferencing

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.8-5-7 | Subject to operator policies, agreement with an authorized third party, and subscriber permission the 6G network shall provide communication service (between UEs and SHEs) and provide suitable means to associate a group of UEs in a given location with the operator’s Service Hosting Environment, taking into account UE mobility patterns and the characteristics of the data provided by UEs (application data), for them (i.e. UEs and SHE) to perform distributed AI model inferencing. NOTE 1: The purpose is to minimize communication overhead and improve overall performance (e.g., inferencing latency, inferencing accuracy). NOTE 2: The application data is used for inference, which can include sensors data on UE. Those application data characteristics can include sensors types (e.g. camera, infrared, radar, proximity sensors, LiDARs), and other related information e.g. UE sensors’ sensing coverage, sensing data-related location and time, sensor resolution, etc.). | PR 6.58.6-1 |  |