3GPP TSG SA WG 1 Meeting #113 	S1-261396

9-13 February 2026, Goa, India	                                                      (revision of S1-261348)

(revision of S1-261342)

(revision of S1-261305)

(revision of S1-261129)

(revision of S1-261018)

Source:	Moderator (China Unicom)

pCR Title:	Consolidation of KPI Requirements with Updated Value on AI Section

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1

Document for:	Approval

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Prepare and propose a way for consolidation on KPI requirements on AI section based on agreed table header, with updated values.

S1-261305 reflects the results of the discussions on 11 Feb. YELLOW and RED parts need further discussion.

S1-261348 reflects the results of the discussions on 12 Feb. YELLOW parts need further discussion.

S1-261396 reflects the results of the discussions on 13 Feb.

1. Introduction

Based on the meeting preparations, this document only provides the consolidation way forward and discussion regarding the AI KPIs, based on TR 22.870 and do not include performance requirements with editor’s note.

2. Reason for Change

To provide communication performance requirements contribution and reflect key points regarding the performance requirements of AI sections.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to new version of 3GPP TR 22.870.

# Discussion Part

Besides the basic template consolidation, the following comments need to be considered：

General:

CM1: Table y.1-2 the “joint e2e latency” is probably to be properly defined or decoupled?

CM2: Table y.1-2 Joint E2E latency value needs to be divided into a communication value and a compute value.

CM3: Table y.1-1 primarily consists of communication KPIs. Reflect the table name.

CM4: The latency title in table y.1-2 need to align with “Max allowed end-to-end latency” but also need to highlight the difference from existing KPIs.

CM5: Is the table y.1-2 related with computing service?

CM6: A definition needed for Joint E2E latency.

CM7: “Joint e2e latency” is more from service level of SA1.

UC 6.48 Use case on service robot for power grid

The value and note 3 updated based on approved S1-254342.

The parts marked in blue indicate that after discussion and modification, no further feedback was received.

The parts marked in yellow indicate that comment received before or during meeting without discussion, or still no further way forward, or confirmed.

The unmarked parts received no comment before meeting, or comment was solved.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

……

[y]	https://www.thinkwithgoogle.com/_qs/documents/2340/bc22e_The_Need_for_Mobile_Speed_-_FINAL_1.pdf

* * * Next Change * * * *

## 14.2	Consolidated Performance Requirements

Editor’s Note: This clause is FFS and was based on S1-254015. The Tables (and titles) will be updated as the KPI consolidation is completed.

### 14.2.1	AI

Table 14.2.1-1: Communication performance requirements for burst traffic

| Use Cases | Burst size | Max Allowed end-to-end latency for a burst (note B-0) | Service bit rate: user-experienced data rate | transmission latency of a packet | Average packet size | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement (UC 9.3 A) | UL/DL: [20 MB] (note A-1) | UL/DL: [750ms] (note A-2) | UL/DL: [200 Mbps] | N/A | > 800B | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement (UC 9.3 A) | UL: [1-3 MB] (note A-1) | UL: [50ms] (note A-3) | UL: [100 – 400 Mbps] | N/A | > 800B | Stationary or Pedestrian | Countrywide |
| Image based GenAI app Nokia: Image based GenAI app (Text to image generation) (UC 6.26 B) | UL: 400 KB | UL: 50ms | UL: 64 Mbps | 20ms | > 800B | Stationary or Pedestrian | Countrywide |
| Video based GenAI app Nokia: Video based GenAI app (visual assistant) (UC 6.26 B) | UL: 20 MB | UL: 400ms | UL: 400 Mbps | 20ms | > 800B | Stationary or Pedestrian | Countrywide |
| Chatbot (UC 6.26 B) | UL: 0.5 KB | UL: 20ms | UL: 200 Kbps | 30ms | < 800B | Stationary or Pedestrian | Countrywide |
| NOTE A-1: Assuming 6 x 4K raw pictures and compression ratio in [21] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [23] for gaming enhancement. NOTE A-2: 750ms is derived from the round-trip time of 3 s (based on users' patience statistics for website loading time as shown in [22][y]) minus around 1.5 s for processing in the cloud and downloading, divided equally between UL and DL (photo enhancement implies to send back the enhanced photo which is assumed to have at least the same size). NOTE A-3: 50ms uplink latency is derived from [24]. NOTE B-0: Max allowed end to end latency for a burst: max latency for sending out the whole packets within a burst. |  |  |  |  |  |  |  |

Table 14.1.1-2: AI service performance requirements (including communication and beyond communication aspects)

| Use Cases | Traffic type | Data frame size (byte) | Transfer interval | Service bit rate: user-experienced data rate |  |  |  | Reliability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Max Allowed end-to-end latency （note 3） (i) | expected Beyond-communication latency (note 2) (ii) | expected Total value of the service latency (note 1) |  |
| Service robot (UC 6.49 C/ UC 6.10) (note A-3) | UL camera data (note A-1) | [<1000] | [10ms] | 80-400Mbps note 5 (note A-2) | [30ms - 50ms] | [50ms-230ms] | [130ms-290ms] (note A-3) (note 6) | [99.9 %] |
| Service robots with enhanced capabilities (UC 6.48 B) | UL sensor data (without LiDAR) (note B-1) | 1250-12500 | 10ms | UL: 1-10 Mbps | [30ms - 50ms] | Up to [80ms] (note 4) | 90-140ms (note B-4) | 99.99% |
|  | UL LiDAR | 345600 (note B-3) | 100ms | UL: 27.6 Mbps (note B-3) |  |  |  | 99.99% |
|  | Control command (high level task, action plan, etc.) (note B-2) | 625-12500 | 50ms | DL: 0.1-2 Mbps |  |  |  | 99.99% |
| NOTE 1: The expected total value of the service latency should be seen as informative and only serves the purpose to indicate what type of performance would be needed. It will be up to deployments of the compute, available AI models needed by the application. The expected total value refers to the maximum allowed service latency for services provided by the operator, to achieve the overall latency from the application (expected experience). This is calculated as a of i(UL) + ii +i(DL). Here, the Service Latency does not include media pipeline latency inside the Service Robot, e.g. application processor processing captured raw images/videos such as de-mosaicing, noise reduction, tone mapping to certain output, frame compression-related codec algorithm processing delay, etc. This part of internal processing latency is assumed to be less than 10ms. NOTE 2: The expected beyond-communication latency refers to the processing latency of service (e.g. 6G AI Service), which could be provided by the operator or a 3rd party. Here the expected value is captured and should only be seen as informative. NOTE 3: Max Allowed end-to-end latency: Refers to either the uplink or downlink direction. NOTE 4: The target value for this parameter is derived from the total Max-allowed combined communication and non communication latency, considering the portion typically allocated to communication latency. For instance, given a total latency of 150ms and a typical one-way communication latency of 30ms, a non-communication latency of 80ms (calculated as 150ms -10ms - 30ms * 2) is expected. Respectively the expected total service latency ‘s Lower and upper boundary (100ms-150ms), subtracted by media pipeline latency inside the service robot and the expected communication latency (30ms-50ms) results in the indicative beyond-communication Latency. NOTE 5: Lower bound of 80 is an approximation for 81.285Mbps is for 336x336 3x 8bit channel raw stream sent by the UE to a multimodal model encoder at 30fps after internal processing of the camera-captured 1080p frames to fit the input size of the model on the server side (or 144.506Mbps for 448x448 channel). In alternative, split processing of the images at the UE can result in encoded tokens: 336x336 images result in 24x24 768 dimensional vectors of floats (2 bytes each, uncompressible), resulting in 212Mbps. 448x448 images result in 32x32 768 dimensional vectors of floats (2 byte each) resulting in 377Mbps. NOTE 6： Whether the robot uses the AI inference result from the 6G network to perform further AI inference locally is out of scope. NOTE A-1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE A-2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE A-3: For physical AI robot interacting with a human user, the robot is expected to mimic the similar basic human brain reaction time including conscious awareness/recognition and decision-making, which ranges from 140ms (mean auditory reaction) to 200ms (visual reaction) [279], and finer object identification requires larger than 200ms [283] and around 300ms [284]. In the target scenarios, robots are expected to have similar perception time as an average human being. NOTE B-1: Refers to the kinematic state, environment perception, manipulation status info except LiDAR to be sent from the service robot to the network to enable effective motion planning, object interaction and navigation. NOTE B-2: Refers to the control command towards service robot, e.g. high-level task, action plans, motion strategy, gripper command, etc. NOTE B-3: the frame size and data rate of LiDAR are based on frame rate 10Hz, 28800 points/frame, 12 bytes for one point. The frame size is calculated by points/frame * bytes per point whereas the data rate is calculated by points/frame * bytes per point * bits per byte * frame rate (i.e. 28800*12*8*10). NOTE B-4: The typical robot control loops require 100-150ms latency [273] for AI inference, communication and control. For example, the communication may take about 40ms while the AI inference may take about 100ms [274] for the service robot. |  |  |  |  |  |  |  |  |

Table 14.2.1-3 Performance requirements for AI for Disability support in IMS

| Profile | User experienced data-rate (note 8) | End to End Communication latency downlink | Beyond communication latency (note 1) | Communication service availability | Overall user density (note 8) | Activity factor (note 8) | UE speed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AI for Disability support for IMS Audio enhancements (note 3) | 0-2 Mbit/s | N/A | [200-400] ms (note 5) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2500] / km2 Urban [1 000] / km2 Rural [10] / km2 (note 4) | [50]% | 5 km/h |
| AI for Disability support for IMS video enhancements (note 2) | 0-2 Mbit/s | 0-50ms (note 7) | 400- 800] ms for video support (note 6) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2500] / km2 Urban [1 000] / km2 Rural [10] / km2 (note 4) | [50]% | 5 km/h |
| NOTE 1: The AI application/compute latency is the maximum time that the application can use for enhancing the media. NOTE 2: The service is offered using IMS for instance an IMS video call where additional information for text in field-of-view from the other side is read out in audio, or text overlay in the video. The KPI for IMS video call is not included in this KPI table. NOTE 3: The audio enhancements are made on the ongoing IMS session (voice or video). This could be in form of further suppressing background noise and enhancing contrast of the voice coming from the remote party. The KPI’s for IMS voice call is not included in this KPI table NOTE 4: It is assumed that 10% of all users are using AI for Disability support NOTE 5: The overall one-way latency needs to be below 400ms for user acceptance as can be seen in [102], this includes latency for a normal IMS voice call and the added AI application/ compute latency NOTE 6: Video support can be “Fast” (e.g., instance sign language interpretation, scene interpretation) and “Slow” (e.g., orientation, object-recognition). The total latency being the sum of the E2E communication (UL) from the other UE, the E2E communication (DL) and compute / AI application should be kept below 900ms [X] NOTE 7: If there will be an additional latency depends on if the implementation i.e., if the “disability” information will be included in the video stream to the UE or sent separately. NOTE 8: The KPI for IMS voice and/or video call is not included in this KPI table. User experienced data-rate and end-to-end latency are meant to be in addition to these KPI’s meant for the AI for Disability support. |  |  |  |  |  |  |  |