3GPP TSG SA WG 1 Meeting #113 	S1-261412

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Proposed CPRs for Agreement – Satellite Comms/Immersive Comms

Draft Spec:	3GPP TR 22.870 v1.0.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides those CPRs that were agreeable for inclusion into the TR at the end of the Thursday’s plenary meeting session.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the those CPRs that were “all green” at the end of the Thursday plenary session.

2. Reason for Change

To move the agreeable CPRs into the corresponding tables in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * * *

### 14.1.10	Integrated Sensing and Communication (ISAC)

Table 14.1.10-1: ISAC

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.10-1-1 | Subject to operator’s policy, the 6G Network shall provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics to be counted. | PR 7.23.6-1 |  |
| CPR 14.1.10-1-2 | Subject to operator’s policy and regulatory requirements, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 |  |
| CPR 14.1.10-1-3 | Subject to operator’s policy and regulatory requirements, the 6G system shall support mechanisms to provide sensing service with a given sensing target density. | PR 7.7.6-1 |  |
|  | Subject to operator’s policy, regulatory requirements, the 6G network shall be able to provide a sensing service that is able to derive predicted location and/or velocity of sensing target(s) as requested by a subscriber or an authorized 3rd party. | PR 7.10.6-1 |  |
|  | Subject to operator’s policy, regulatory requirements, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.10.6-2 |  |
|  | Subject to operator’s policy and regulatory requirements, he 6G system shall be able to prioritize a combination of communication service, sensing service and positioning service for an application on a UE over 3GPP services for other UEs. | PR 7.19.6-2 |  |

* * * End of Changes * * *