3GPP TSG SA WG 1 Meeting #113 	S1-261413

9-13 February 2026, Goa, India	(revision of S1-261399)

Source:	Nokia (ISAC KPI moderator)

pCR Title:	Pseudo-CR on consolidated ISAC KPI table

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.4

Document for:	Approval

Contact:	hideaki dot takahashi at nokia dot com

Abstract: This pCR provides the updated ISAC KPI table to reflect the email discussion outcome.

1. Introduction

An email discussion took place before SA1 #113 meeting to categorize ISAC KPI tables, based on Rel-19 ISAC table captured in TS 22.137.

2. Reason for Change

To update the consolidated ISAC KPI table by reflecting KPI categorization discussed by email in S1-261028.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v1.1.0.

* * * First Change * * * *

### 14.2.2	ISAC

The 6G system shall be able to provide sensing with the following performance requirements:

NOTE 1: The definitions of the terms used in the following KPI tables are defined in [6].

Table 14.2.2-1: Consolidated performance requirements for Integrated Sensing and Communication.

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safety Assistance for vulnerable pedestrians (Use case 7.3) | Outdoor (Crossing) | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤ [500] | ≤ [0.1] | ≤ [5] | ≤ [5] |
| High topology mapping (NOTE 1) (Use case 7.4) | Outdoor | N/A | 0.10 | 0.10 | - |  | [0.4] | - | 50 | ≤ 0.2 | ≤10 | <1 |
| Low-altitude UAV supervision: UAV intrusion detection (Use case 7.5) | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 |
| Environment object reconstruction (Use case 7.6) | Outdoor (Building) (NOTE 2) | 95 | [0.5-5] (NOTE 3) | [0.5-5] (NOTE 3) | N/A | N/A | [0.5] | N/A | [10,000-600,000] | [10-60] | [1-5] | [1-5] |
|  | Outdoor (Vehicle) (NOTE 2) | 95 | 0.5 | 0.5 | 1.5 | N/A | [0.5] | [0.5] | 100 | 0.1 | 5 | 5 |
| Road digitalization (Use case 7.7) | Outdoor Sensing target density: ≤750 vehicle Per 1000 m x 24 m; ≤100 pedestrian per 1000 m x 24 m (NOTE 5) | 95 | 0.5 (vehicle) (NOTE 4) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
|  | Outdoor Sensing target density: ≤72 vehicles per 1000 m x 24 m (NOTE 6) | 95 | 3 (NOTE 4) | N/A | 3 | N/A | 1 | 1 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
| Ship detection and tracking (NOTE 7) (Use case 7.9) | Detection of ship on the near shore waters and offshore waters | 95 | ≤100 (NOTE 8) | NA | NA | NA | ≤100 | NA | 5000 | 1 | 5 | 5 |
|  | Detection of ship on the harbour entrances, harbour approaches, and port. and coastal waters | 95 | ≤10 (NOTE 8) | NA | NA | NA | ≤10 | NA | 5000 | 1 | 5 | 5 |
|  | Detection of ship on the inland waterway (e.g. river, lake) | 95 | ≤2~10 (NOTE 9) | NA | NA | NA | ≤2 | NA | 5000 | 1 | 5 | 5 |
| Enhanced XR navigation (Use case 7.13) | 10 m2 | 99.9 (NOTE 11) | ≤0.5 (NOTE 10) | 0.5 (NOTE 10) | 0.5 | N/A | 0.1- 1 (NOTES 10, 11) | 0.5 (NOTE 10) | ≤100 | 0.1 – 1 (NOTE 11) | 1 | 1 |
| Collaborative Robots (Note 12, Note 24) (Use case 7.14) | Indoor/Outdoor | [95] | [≤ 0.1] | [≤ 0.1] |  |  |  |  | N/A |  |  |  |
| Infrastructure collapse monitoring (Use case 7.15) | Outdoor (e.g. Detecting sudden collapse on infrastructure such as highway, railway, road, flyover, rural areas, farmland) | [95] | [4] | [N/A] | [N/A] | [N/A] | [4] | [N/A] | TBD | [ 1] | [1~2] | [1~2] |
| UAV takeoff and landing (Note 24) (Use case 7.16) | Outdoor | [95] | [0.1] (NOTE 13) | [0.1] (NOTE 13) | [1] | [1] | [0.5] | [1] | [100-500] | [0.2] | [5] | [5] |
| Safe and economic UAV transport (Use case 7.18) (Note 18) | Outdoor | 99.99 | ≤ 50 | 50 (NOTE 15) | 5 (NOTE 15) | 5 | 10 (NOTE 15) | 0.5 (NOTE 15) | ≤ 100 | 0.1 - 1 | 0.1 (NOTE 16,17) | 1 (NOTE 17) |
| Structural health monitoring (Use case 7.22) | Outdoor (e.g. detection of corner reflectors on bridges, buildings in urban scenario) | 95 | 0.1 (NOTE 19) | 0.1 | N/A | N/A | N/A | N/A | 5000 | 60 | 5 | 5 |
| UAV detection, classification and counting (Use case 7.23) (Note 21) | outdoor | 95 | 1 (NOTE 20) | 1 (NOTE 20) | 1 (NOTE 20) | 1 (NOTE 20) | [0.3 – 1] (NOTE 20) | 1 (NOTE 20) | ≤1000 (NOTE 20) | ≤1 (NOTE 20) | 2 (NOTE 20) | 2 (NOTE 20) |
| Hand Gestures Recognition (NOTE 23, NOTE 24) (Use case 7.24) | Indoor (Factory Environment) (NOTE 31) | 99 | 0.1 (NOTE 30) | 0.1 (NOTE 30) | n/a | n/a | [0.3 to 0.5] (NOTE 30) | n/a | ≤ 20 | ≤ 0.02 | ≤ 1 (NOTE 22) | ≤ 10 (NOTE 22) |
| Head Gestures Recognition (NOTE 23, NOTE 24) (Use case 7.24) | Indoor (Factory Environment) (NOTE 32) | 99 | 0.1 (NOTE 30) | 0.1 (NOTE 30) | [0.3] | [0.3] | [0.01] (NOTE 30) | [0.3] (NOTE 33) | ≤ 20 | ≤ 0.02 | ≤ 1 (NOTE 22) | ≤ 10 (NOTE 22) |
| Body and Limb Gestures Recognition (NOTE 23, NOTE 24) (Use case 7.24) | Indoor (Factory Environment) | 99 | 0.1 (NOTE 30) | 0.1 (NOTE 30) | n/a | n/a | 0.1 (NOTE 30) | n/a | ≤ 20 | ≤ 0.02 | ≤ 1 (NOTE 22) | ≤ 10 (NOTE 22) |
| Sensing of Surroundings (NOTE 26, NOTE 29) (Use case 7.24) | Indoor (Factory Environment) | 99 | 0.1 (NOTE 27) | n/a | 0.1 | n/a | [0.5 to 1] (NOTE 27) | 1.5 | ≤100 | 0.1 | < 10 (NOTE 25) | < 10 (NOTE 25) |
| Detection of Pathways, Openings (NOTE 26, NOTE 29) (Use case 7.27) | Indoor (Factory Environment) | 99 | 0.25 (NOTE 27, NOTE 28) | 0.25 (NOTE 27, NOTE 28) | 0.1 | n/a | [0.75] (NOTE 27) | 1.5 | ≤100 | 0.1 | < 10 (NOTE 25) | < 10 (NOTE 25) |
| NOTE 1: The accuracy KPIs in Scenario “High Topology mapping” are extrapolated from [323] Table C2 as considered for autonomous navigation and remote driving. NOTE 2: A sensing target in Scenario “Environment object reconstruction” refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically NOTE 3: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values are needed to measure and provide the flexibility of reconstruction accuracy. NOTE 4: Vehicle, with the assumptions in Table 7.7.1-1. NOTE 5: Sensing target density for vehicles ≤ 750 per 1000 m x 24 m; sensing target density for pedestrians ≤100 per 1000 m x 24 m. Sensing target density is described in Clause 3.1 of the present document. NOTE 6: Sensing target density for vehicles ≤ 72 per 1000 m x 24 m. Sensing target density is described in Clause 3.1 of the present document. NOTE 7: The typical size (Length x Width x Height) of ship is 270 m x 30 m x 30 m in both sensing service category#1 and #2, and 100 m x 15 m x 15 m in sensing service category #3, according to reference [91] NOTE 8: The KPI values for detection of ship on these sensing service categories are referred to [192]. NOTE 9: The KPI values for detection of ship on this sensing service category are referred to [193]. NOTE 10: This positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the user NOTE 11: The KPIs are based on the Hazard Prevention in Industrial Environments for “Prediction of workers’ and machines’ actions” and “Detection of worker’s location” in [94] NOTE 12: Related usecases are captured in clause 7.2 of 3GPP TR 22.837 [9] and clause 6.2 of 3GPP TS 22.137 [6]. NOTE 13: The KPI values for accuracy of positioning are centimetre-level sourced from reference [199], [200] NOTE 14: The KPI values of this use case are effective for sensing in specific area for UAV takeoff and landing. NOTE 15: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV and validate the reported location of the UAV itself. NOTE 16: Safety impact of missed detection is substantially higher compared to the false alarm, here fore the Missed Detection is assigned a more stringent value compared to the False Alarm. NOTE 17: To enable safe operation of BVLOS UAV flights in mixed airspace (manned & unmanned) and in high risk areas (e.g. above cities) the value for missed detection of 0.1% corresponds to the lower value of the missed detection range specified in Table 6.2-1 of TS 22.137 [6] for sensing service category 2. NOTE 18: Sensing service availability [%] is 99.999 NOTE 19: The typical length of corner reflector is 0.5m [327]. NOTE 20: Most of the requirements presented here are related to the sensing category 3 in [6] NOTE 21: Detection, classification of UAV features such as propellers may require more stringent velocity and range resolution due to the dimensions of propellers with respect to the UAVs. Some propeller diameters to UAV body ratio could be between ½ to 1/3 [330], therefore, the range resolution values are modified accordingly. NOTE 22: Depending on the actual application to be controlled by the gestures, Missed detection and False alarm may have different KPI values from < 1% to 10%. NOTE 23: References for hand gesture recognition are [331][332], for facial and head gesture recognition [333], and for body and limb gesture recognition [334][335]. NOTE 24: The relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). NOTE 25: Only valid for non-time-critical applications, < 1% if integrated in time-critical processes. NOTE 26: A maximum sensing range of 10 m is required. NOTE 27: In this table, accuracy of positioning estimate is the horizontal or vertical minimum distance between distinguishable objects, range resolution is the minimum distance between distinguishable objects regarding depth sensing. NOTE 28: Operational practice of autonomous mobile devices (AMRs, AGVs) in industrial settings requires openings on the path to be at least 50 cm wider/taller than the width/height of the devices. The KPI value is half of this additional width/height. NOTE 29: The sensing KPIs are derived from discussions with application experts. Physical realization has been checked. NOTE 30: In this table, accuracy of positioning estimate is the horizontal or vertical minimum distance between distinguishable objects, range resolution is the minimum distance between distinguishable objects regarding depth sensing. NOTE 31: The sensing KPIs for Hand Gestures Recognition are derived from being able to distinguish the position of a hand. NOTE 32: The sensing KPIs for Head Gestures Recognition are derived from different Head Gestures such as shaking, nodding, or turning one’s head. NOTE 33: Detection of intentional movement of one’s head (e.g. intentional shaking or nodding) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 14.2.2-3: Performance requirements for  combined communication, sensing and positioning services

|  | Communication KPIs |  |  |  | Positioning KPIs | Sensing KPI |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | User experienced data rate | E2E latency | Communication Service availability | Connection density | Location accuracy | Sensing accuracy |
| Combined communication, sensing and positioning services (UC 7.x Network assisted smart transportation) | [1-10 Mb/s] | [20 ms] | 99.99% (note 1) | 104 devices/km2 | [1m] * [1m] * [1m] with 90% probability (note 2) | category 2 or 3 (notes 2, 3) |
| NOTE 1: within service volume NOTE 2: within 99% of the service volume NOTE 3: Category 2 or 3 in Table 6.2-1 in TS 22.137 [6] |  |  |  |  |  |  |