3GPP TSG SA WG 1 Meeting #113 	S1-261067

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	Ericsson

pCR Title:	Solving two editor’s notes in 6.38 AI for disability

Draft Spec:	3GPP TR 22.870 1.1.0

Agenda item:	8.1.3

Document for:	Agreement

Contact:	Daniel.lonnblad at ericsson.com

Abstract: This pCR introduce some discussion and explanation about the KPI’s in the table with the intent to solve the Editors notes.

1. Introduction

Two editors notes where added to the use case AI for disability support, Intention is to add some extra information how the KPI’s are estimated.

2. Reason for Change

The first Editor’s Note “The value of Application round trip latency are for FFS”.

The intended use case covers two possible enhancements, audio enhancements and video enhancements. Furthermore, within these groups you can see different enhancements, but we propose to split the kpi table into these two groups. This would clarify the kpi table specifically for the bit rate and latency.

The latency has been further clarified; it is divided into a communication part that is mainly valid for the video enhancements and a compute latency which is what the application needs to deliver the enhancements within to fulfil the expected experience.

If voice should be enhanced in a voice call the overall latency of the voice call including the enhancement should not be larger than 300 to 500ms to not break the overall experience of the voice call. 300-500ms can be seen as a upper bound for voice this is also described in the ITU-T G.114 (05/2003) recommendation that is referenced in the table. This needs to take both the latency in the network (roundtrip) and the latency for the enhancement into account. Estimating network latency to be 10ms to 50ms in one direction leaving 200-400ms for the voice enhancement application.

If it instead is disability enhancement application to explain surroundings from a video feed somewhat higher latency can be expected. Latencies up to 1000ms could be accepted dependent on the actual type of enhancements. The example described in KPI table is based on a ordinary IMS video call, the video from the other side is enhanced with text overlay to make it easier to read signs for instance. This means video is captured in the IMS application and the additional information created is sent via a side channel to the UE, this could also be incorporated in the original video flow. This will be dependent on real implementation.

For the second Editor’s Note “The value of Bit rate uplink are FFS”

The example use case for enhancements in a IMS video call has been slightly changed to only do enhancements of the other sides video in a IMS video call. Therefore, no extra video is sent from the UE to the network, only enhancement information from the Network to the UE. The Editor’s Note is no longer valid and can be removed.

3. Conclusions

The two Editor Notes can be removed

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 1.1.0.

We also suggest removing the now empty box at the top of the table and the column for bit rate uplink (since it is not used). It is not possible to do with change bars.

* * * First Change * * * *

### 6.38.6	Potential New Requirements needed to support the use case

The following requirement for AI for Disability support are derived from the use case above.

[PR 6.38.6-1] Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support.

NOTE:	The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream i.e. enhancements, clarifications, translations, speech to text, text to speech, etc.), visual aid (modification of the visual stream).

[PR 6.38.6-2] Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system may be able to enhance the IMS emergency communication service with AI capability, to enhance the audio and video stream for disability support.

[PR 6.38.6-3] The 6G system including IMS, shall support following KPIs in Table 6.38.6-1, in Dense Urban, Urban, and Rural indoor/outdoor scenario:

Table 6.38.6-1: Performance requirements for AI Disability support

| Profile |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate downlink |  | End to End Communication latency | AI application/ compute latency (NOTE 1) | Communication service availability | Overall user density | Activity factor | UE speed |
| AI for Disability support Audio enhancements (NOTE 3) | 1-2 Mbit/s |  | N/A | [200-400] ms (NOTE 5) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500] / km2 Urban [1 000] / km2 Rural [10] / km2 (NOTE 4) | [50]% | 5 km/h |
| AI for Disability support video enhancements (NOTE 2 ) | 1-2 Mbit/s |  | 10-50ms | 400- 850] ms for video support (NOTE 6) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500] / km2 Urban [1 000] / km2 Rural [10] / km2 (NOTE 4) | [50]% | 5 km/h |
| NOTE 1: The AI application/compute latency is the maximum time that the application can use for enhancing the media. NOTE 2: The service is offered using IMS for instance an IMS video call where additional information for text in field-of-view from the other side is read out in audio, or text overlay in the video. NOTE 3: The audio enhancements are made on the ongoing IMS session (VoLTE/ViLTE call). This could be in form of further suppressing background noise and enhancing contrast of the voice NOTE 4: It is assumed that 10% of all users are using AI for Disability support NOTE 5: The overall one-way latency needs to be below 400-500ms for user acceptance as can be seen in [Y], this includes latency for a normal IMS voice call and the added AI application/ compute latency NOTE 6: Video support can be “Fast” (e.g., instance sign language interpretation, scene interpretation) and “Slow” (e.g., orientation, object-recognition). The total latency being the sum of E2E communication (DL)and compute / AI application should be kept below 900ms[X] |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

# 2	References

[X] Delay Threshold for Social Interaction in Volumetric eXtended Reality, Carlos Cortes et al., ACM, 2024.

[Y] ITU-T Recommendation G.114 (05/2003), One-way transmission time.

* * * Next Change * * * *

<Proposed change in revision marks>