3GPP TSG SA WG 1 Meeting #113	S1-261104

9-13 February 2026, Goa, India	(revision of S1-261016)

Source:	Thales, Magister solutions, Novamint

pCR Title:	UC 5.10.2 - Diversity of UEs for satellite access -UPDATES

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.2.1 System and Operation Aspects (FS_6G_REQ)

Document for:	Approval

Contact:	nicolas.chuberre@thalesaleniaspace.com

Abstract: this contribution provides additional rows to the table “performance requirements for ubiquitous communication” of the KPI requirements on ubiquitous connectivity currently being consolidated.

1. Introduction

The performance requirements for NTN for clause 8 are being consolidated.

This contribution provides additional rows to the table “performance requirements for ubiquitous communication” of the KPI requirements on ubiquitous connectivity.

2. Summary of changes

The update is proposing to add some Performance requirements for satellite access to the Table 14.2.3.2-1: Consolidated performance requirements for ubiquitous communication. They relate to Pedestrian, High speed train connectivity, Drone/vehicular connectivity and Large vessel connectivity use cases (see agreements on UC 5.10.2 in previous SA1 meetings).

The proposed values are based on on system level simulations and assume the following:

- Pedestrian scenarios; UE with omni directional antenna, S band service link, 30 MHz channel bandwidth on both UL and DL, VLEO-300 km, Frequency Reuse Factor of 3, 90° center beam elevation angle (NOTE 1)

- All other scenarios: UE with directional antenna, Ka band service link, 400 MHz channel bandwidth on both UL and DL, VLEO-300 km, Frequency Reuse Factor of 3, 90° center beam elevation angle

- User equipment performances updated from TR 38.821 (NOTE 2)

- Satellite access node performances as proposed for 6G in RAN1#123, document R1-2509055, “NTN Characteristics for the Evaluation Assumptions for 6GR air interface” (NOTE 2)

- Propagation model: 3GPP TR 38.811

NOTE 1: S-band VLEO-300 km payload assumes modified EIRP density of 28 dBW/MHz.

NOTE 2:

- UE with directional antenna refers to a VSAT with either a 20 x 20 cm antenna or a 60 cm aperture antenna depending on the scenario. The 20 x 20 cm antenna based on automotive VSAT assumptions, assumes 34 dBi directional antenna gain, antenna temperature 150 K, noise figure 4 dB, 38.5 dBm Tx power.

- UE with Omnidirectional antenna assumes 4x4 elements.

3. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

* * * First Change * * * *

## 14.2.3	Ubiquitous Connectivity

…

### 14.2.3.2	Consolidated performance requirements for ubiquitous communication

Table 14.2.3.2-1: Consolidated performance requirements for ubiquitous communication

| Scenario | Experienced data rate | Area traffic capacity (note 1) | Overall user density | Activity factor | UE speed | UE type | Reliability | Service availability | End-to-end latency | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite access |  |  |  |  |  |  |  |  |  |  |
| Disaster relief (note 2) (UC 8.6) | N/A | N/A | Up to [100] devices/km2 for SMS only | N/A | 3 km/h | Handheld | [99,9-99,999] % | Up to 99,9 % | Up to [600] ms | Peak data rate DL: Up to [20] Mbit/s UL: Up to [2] Mbit/s |
| Global video service in remote area (note 5) (UC 8.8) | High video resolution DL: [2-5] Mbit/s UL: [1] Mbit/s Low video resolution DL: [2] Mbit/s UL: [250] kbit/s | DL: Up to [10] kbit/s/km2 (note 6) | [< 0,1] person/km2 | 2% | N/A | Handheld | N/A | N/A | N/A | - |
| Global video service in deep rural area (note 5) (UC 8.8) | High video resolution DL: [2-10] Mbit/s UL: [2] Mbit/s Low video resolution DL: [2] Mbit/s UL: [250] kbit/s | DL: [1] Mbit/s/km2 (note 7) | [0,1-10] person/km2 | 4% | N/A | Handheld | N/A | N/A | N/A | - |
| Immersive media service via AAM/UAM (direction: from network to user, uncompressed) (UC 11.5) | DL: [1,3] Gbit/s (note 9) (note 11) | DL: TBD | - | - | Up to 260 km/h (note 14) | UAM mounted | - | - | - | Service interruption time [10] ms for 60 fps, [20] ms for 30 fps (note 8) |
| Immersive media service via AAM/UAM (direction: from network to user, compressed) (UC 11.5) | DL: [25] Mbit/s (note 10) (note 11) | DL: TBD | - | - | Up to 260 km/h (note 14) | UAM mounted | - | - | - | Service interruption time [10] ms for 60 fps, [20] ms for 30 fps (note 8) |
| Airplanes connectivity (note 12) (UC 11.6) | DL: 6 Gbit/s/plane UL: 3 Gbit/s/plane | - | - | - | Up to [1500] km/h | Airplane mounted (note 13) | - | - | - | - |
| Pedestrian (note 15) | DL: 5 Mbit/s UL: 2 Mbits | DL: 0.1 Mbit/s:km² UL: 0.05 Mbit/s:km² | 2/km2 | 1% | Pedestrian | Handheld | N/A | N/A | N/A | N/A |
| High speed train connectivity (note 16) | DL: 40 Mbit/s UL: 40 Mbits per wagon | DL: 1 Mbit/s:km² UL: 1 Mbit/s:km² | 0.5/km2 | 5% | Up to 500 km/h | Train mounted | N/A | N/A | N/A | N/A |
| Drone/vehicular connectivity (note 16) | DL: 40 Mbit/s UL: 40 Mbits per drone or vehicle | DL: 1 Mbit/s:km² UL: 1 Mbit/s:km² | 1/km2 | 2.5% | Up to 250 km/h | Drone/vehicle mounted | N/A | N/A | N/A | N/A |
| Large vessel connectivity (note 16) | DL: 60 Mbit/s UL: 60 Mbits Per vessel | DL: 1.5 Mbit/s:km² UL: 1.5 Mbit/s:km² | 0.25/km2 | 10% | Up to 50 km/h | Vessel mounted | N/A | N/A | N/A | N/A |
| Terrestrial access |  |  |  |  |  |  |  |  |  |  |
| Ubiquitous and Resilient Network (note 3) (UC 8.2) | DL: [0,1-25] Mbit/s UL: [2] Mbit/s (note 4) | N/A | [0,1] devices/m2 | N/A | up to [120] km/h | Handheld | [99.9-99.999] % | [98.5] % | [10-100] ms (note 4) [10-500] ms | Coverage [99.9] % |
| NOTE 1: Area capacity is averaged over a satellite beam. NOTE 2: Positioning accuracy is low (≈ [100] m). Positioning service availability is up to [99] %. Positioning service latency is [1] s. NOTE 3: Location accuracy is low (≈ [10] m). Positioning availability is [99] %. Positioning latency is [1] s. NOTE 4: When using base station on board HAPS for NTN, end-to-end latency should be up to 10 ms (operating at 20 km altitude) and user experienced data rate should be up to [500] Mb/s for DL and [50] Mb/s for UL to enable eMBB service [374]. NOTE 5: Reduced video quality is acceptable in the remote areas and deep rural area, see chapter 3.9.1.3 and 3.9.1.5 in [31]. NOTE 6: The DL area traffic capacity corresponds to an average user density of 0,1 person/km2. NOTE 7: The DL area traffic capacity corresponds to an average user density of 5 persons/km2 equally shared by two operators. The area traffic is here split between two operators, with [2] Mbit/s/km2 in total. NOTE 8: It is assumed that the interruption time is less than a single frame (16ms for 60 fps, 32 ms for 30 fps) plus a margin of an order of millisecond (e.g. for other processing time). NOTE 9: Uncompressed (0.8 M points per frame, total 300 frames are considered: i.e. 43.2 Mbits/s, 30 fps) NOTE 10: Compressed (Video-based Point Cloud Compression (V-PCC), bpip (bits per input points) total 1.14 (Color 0.9, Geometry 0,11, Occupancy 0.13) are considered: 25 Mbits/s) NOTE 11: It is assumed that the number of passengers using immersive media service per UAM is up to 2. NOTE 12: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 2:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple satellite links NOTE 13: This airplane mounted UE is operated as a mobile base station relay, providing connectivity to normal UEs used by passengers. These KPIs enable the case of XR UEs for applications in highly dynamic environments, where UEs offload processing to an edge-cloud. NOTE 14: The number is based on [408], [409]. NOTE 15: The values are assuming channel bandwidth of 30 MHz per direction for UE with omnidirectional antenna NOTE 16: The values are assuming channel bandwidth of 400 MHz per direction for UE with directional antenna |  |  |  |  |  |  |  |  |  |  |

[CPR 14.2.3.2-1]: The 6G system with satellite access, shall support SMS delivery to UEs with up to [1000] devices/km2 density.

[CPR 14.2.3.2-2] The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) at the altitudes from 0 to 3 km via satellite access and/or terrestrial access.

* * * End of changes * * * *