3GPP TSG-SA WG1 Meeting #113 	draft S1-261112

9-13 February 2026, Goa, India	(revision of S1-261093)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.11-2 (Satellite-based positioning) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.5

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.11-2: for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting. As agreed in SA1 #112 Ad Hoc-e, Table14.1.11-2 will include CPRs about determining position, velocity and/or time based on satellite-based positioning capability.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254412, the output of the initial consolidation discussions for Ubiquitous Connectivity (SA1 #112)

  - In that document, this table did not exist and there were no proposed CPRs for inclusion.

- S1-254120  and S1-254121 (CATT)

- S1-254163 (ZTE Corporation, CSCN)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Removed PR 8.16.6-1 (position integrity). It will be addressed in Table 14.1.2-1: Security and Privacy

- Merged comments from S1-251054 (CATT/China Telecom), but did not move any PRs across tables to retain stability/integrity of tables for discussions.

NOTE: Please check that PRs “moved to the KPI discussions” (i.e., below the “End of Changes” do not have any PR-related text that should be captured in the CPR Table (e.g., Positioning Integrity and Time Synchronisation).

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-2: Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-2-1 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine and provide the UE location meeting the regulatory requirements for both sparse and dense satellite constellations. | PR 8.3.6-4 PR 8.6.6-1 | UE Location Regulatory Support CATT proposal in S1-254121 was marked to be not pursued in S1-254412. |
| Alt CPR 14.1.11-2-1 (Qualcomm) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location, based on 3GP positioning technologies for UE using only satellite access. | PR 8.3.6-4 PR 8.6.6-1 PR 8.5.6-2 | UE Location Regulatory Support QC: suggest some edits (regulatory reqs is redundant), and prefer to re-merge with 8.5.6.2 |
| Alt CPR 14.1.11-2-x | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location, based on 3GPP positioning technologies for UE using only satellite access. | PR 8.3.6-4 PR 8.5.6-2 | UE Location CATT (new) from S1-261054 |
| New CPR 14.1.11-2-4 (CATT) | Subject to operator’s policy, regulatory requirements, and subscriber permission, the 6G system with satellite access shall be able to determine the UE location based on 3GPP positioning technologies for UE only using satellite access. | PR 8.5.6-2 | Determine UE location using satellite-based positioning QC: prefer to merge with CPR 2-1 |
| Orig PR | The 6G system with satellite access shall be able to provide the location service meeting the regulatory requirements for both sparse satellite constellation and the dense satellite constellation. | PR 8.3.6-4 | Provided for info |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to determine the UE location. | PR 8.5.6-2 | Provided for info |
| Orig PR | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | Provided for info |
| Alt CPR 14.1.11-2-2 | Qualcomm proposal The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine its position using 3GPP positioning technology. CATT proposal The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices I did not see a difference in the two proposals. |
| Alt CPR 14.1.11-2-3 (Qualcomm) | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of 3GPP satellite and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| CPR 14.1.11-2-5 (Hytera) | The 6G system with satellite access shall enable an MCX UE to use the 6G positioning services to obtain its position, including support for network-assisted GNSS via satellite access, on request, triggered by an event, or periodically. | PR 11.13.6-4 | MCX positioning with support of A-GNSS via satellite access [moving from industrial part]. QC: clarify “on request”, by WHO? |
| New CPR 14.1.11-2-x | The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS). | PR 8.5.6-1 | Proposed by Airbus to separate functional requirement from KPI PR |
| New CPR 14.1.11-2-y | The 6G system with satellite access shall be able to provide a positioning service supported by single 3GPP satellite positioning technologies or the combination of 3GPP satellite and non-3GPP positioning technologies(e.g. Network-based Assisted GNSS). | PR 8.5.6 -1 PR 8.11.6-1 | Positioning service From CATT (S1-261054) |

* * * End of Changes * * *

Note (provided for information): the following initial table entries in S1-254020 were moved to the KPI discussions.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.11-2-5 | The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the KPIs in Table 8.5.6-1: Performance requirements for satellite-based positioning services | PR 8.5.6-1 | KPI |
| Alt 14.1.11-2-5 (CATT( | The 6G system with satellite access shall be able to provide different positioning service to different typoes of UE (e.g. energy constrained device, UAV, airplane) using satellite access, supported by single satellite based positioning method or hybrid 3GPP methods. | PR 8.5.6-1 PR 8.7.6-1 PR 8.10.6-1 | KPI Positioning Service based on satellite-based positioning technology |
| 14.1.11-2-6 | The 6G system with satellite access shall be able to provide positioning service following the KPIs in Table 8.7.6-1: Performance requirements for satellite-based positioning services for energy constrained devices from [338] | PR 8.7.6-2 | KPI Energy constrained devices |
| 14.1.11-2-7 | The 6G system with terrestrial and satellite access shall be able to provide positioning service for UE type e.g. Airplane Mounted and UAV Mounted with high availability, by using 3GPP positioning technologies and fulfilling the KPIs in Table 8.10.6-1: Performance requirements for positioning services based on hybrid TN and NTN positioning technologies. | PR 8.10.6-1 | KPI |
| 14.1.11-2-8 | The 6G system with satellite access shall be able to provide positioning service with the KPIs from Table 8.11.6-1: Performance requirements for positioning services with satellite access | PR 8.11.6-2 | KPI |