3GPP TSG SA WG 1 Meeting #113 	S1- 261201

9-13 February 2026, Goa, India	(revision of S1-261200, S1-261042)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.8-6 AI traffic characteristics

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.3

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254490. This table was not discussed during SA1 #112 but addressed the comments from SA1 #112 from companies’ emails and draft on wording proposals.

Based on the outcome of discussion in SA1 #112 Ad hoc meeting, the following wording is changed:

User consent – subscriber permission

Based on operator policy – subject to operator’s policy

This pCR proposes to update Table 14.1.8-6 (AI traffic characteristics) in TR 22.870 with CPRs for inclusion into the draft TR.

* * * First Change(All New CPRs) * * * *

Table 14.1.8-6 – AI traffic characteristics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.8-6-1 | Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission , 6G system shall be to support network awareness of the burst characteristics (e.g. Burst Data Rate) in traffic and provide mechanisms to optimize resource efficiency and assure user experience when handling such traffic. NOTE: Improved coordination between application on the UE and the 6G network is expected, considering GenAI traffic is usually encrypted end-to-end. For example, the application on the UE could inform the 6G network of the type of GenAI traffic (e.g., image-based, video-based, chatbot) and/or characteristics of the traffic (e.g., burst), for the 6G network to consider appropriate mechanisms to provide communication service. QC: Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission, 6G system shall provide information about AI traffic characteristics (e.g. Burst Data Rate) to the network and support mechanisms to optimize resource efficiency and assure user experience when handling such traffic. Ericsson: Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission, 6G system shall be able to support AI application traffic (eg burst Data Rate) | PR 6.26.6-2 | QC: Communication Service Optimization based on Traffic characteristics Awareness in the Network QC propose to delete the note [Ericsson] Not sure what is meant with awareness, seems to imply a solution. Optimize resource efficiency I assume we always do. Xiaomi: GenAI change to AI service ZTE: remove the NOTE Is the Burst Data Rate a term? |
| CPR 14.1.8-6-2 | Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission , 6G network shall be able to dynamically adjust the QoS configurations to changes in traffic characteristics (e.g payload type changes from text to image, or the pay load type is predicted/notified to be changed during a session). Nokia: Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission, 6G network shall be able to dynamically adjust the QoS to changes in traffic characteristics (e.g. when the payload type changes from text to image, or predicted or notified payload type changed during a session). for UE or group of UEs. Ericsson: Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission, 6G network shall be able to adjust QoS level based on need from application (e.g., payload type change from text to image). | PR 6.26.6-3 PR 6.61.6-1 | suggest to merge CPR 7 into this [Ericsson]: Proposal to simplify requirement Xiaomi: understanding is that any changes in the traffic will be indicated to the network explicitly, not requiring the Nw to examine delivered packets |
| - | Subject to operator’s policy, regulatory requirements, subscriber permission and agreement with authorised 3rd party, 6G network shall be able to adjust the QoS configurations dynamically when the payload type changes (e.g. from text to image), or the pay load type is predicted/notified to be changed during a session. | PR 6.26.6-3 |  |
| - | Subject to operator’s policy, the 6G system shall support the dynamic changes in QoS, when the traffic characteristic dynamically changes or is predicted to change, upon request by an authorised third party | PR 6.61.6-1 |  |
| ZTE: CPR 14.1.8-6-2 | Subject to operator’s policy, agreement with authorized 3rd party and subscriber permission , 6G network shall be able to dynamically adjust the QoS configurations for a UE or a group of UEs, in response to changes in traffic characteristics (e.g payload type changes from text to image, or the pay load type is predicted/notified to be changed during a session,etc. ), upon request by an authorized third party. | PR 6.26.6-3 PR 6.52.6-3 PR 6.61.6-1 | ZTE: merge CPR 2, 4 dynamically adjust QoS based on traffic characteristics changes. |
| CPR 14.1.8-6-3 | There is proposal to merge CPR 3 and 4 into 2. There is a need for more time on this part. HW: CPR3 and 4 expand the scope. The 6G network shall be able provide a suitable means with a very high efficiency and reliability, required by trusted third party (e.g., application server), to accommodate the dynamic changes of traffic demand and QoS characteristics in a group of trusted third parties (e.g., multiple applications running in multiple robots that are UEs). NOTE 1: The dynamic changes of traffic demand and QoS characteristics might be caused e.g., by using different levels of data/sensor fusion within each robot of the robot group. The changes of traffic demand include changes of AI-related traffic type (e.g., raw data, pre-processed data, AI/ML model, inference result). NOTE 2: Each robot can have multiple applications. There can be multiple intra-robot AI-related operation sessions between robot’s applications layer (e.g., “robot sensing part”, “robot processing part”, “robot actuating part”) and communications layer (e.g., “robot communication part”). Also, there can be many more inter-robot AI-related communication sessions, each of which might have different QoS characteristics and traffic demand characteristics. Xiaomi: The 6G network shall be able provide a suitable means with a very high efficiency and reliability, as indicated as required by trusted third party (e.g., application server), to accommodate the dynamic changes of traffic demand and QoS characteristics in a group of trusted third parties (e.g., multiple applications running in multiple UEs). | PR 6.52.6-1 | 3rd party AI agent, communication, QoS [Ericsson]: Need to simplify, words like “very high efficiency” not needed. What is really the addition from 14.1.8-6-2? Will there be other mechanisms due to the fact that there are more than one UE and application involved? It is still the application that would require change of QoS to the UE’s? Xiaomi: 3rd party will indicate required efficiency and reliability. Not sure if we need to capture in the Requirement that this is for “multiple Robots”, multiple UEs maybe sufficient. Robot aspect is captured in the Note. ZTE: How to understand: very high? Is it KPI related? |
| CPR 14.1.8-6-4 | Subject to operator’s policy, the 6G network shall be able to support dynamic QoS needed for a group of UEs (e.g., multiple robots or third party AI agents) when traffic characteristics change is predicted to occur or has occurred (e.g. change between AI/ML model transfer and AI/ML inference result transfer). | PR 6.52.6-3 | 3rd party AI agent, communication, QoS [Ericsson] Is this not covered by above requirements? ZTE: propose to merge with CPR 14.1.8-6-2 |
| QC: CPR 14.1.8-6-3 | Subject to operator’s policy and subscriber permission, 6G network shall be able to accommodate the dynamic changes of traffic demand and QoS characteristics in a group of authorized third parties applications (e.g., multiple applications running in multiple robots with UEs) | PR 6.52.6-1 PR 6.52.6-3 | Proposed merged CPR 3, 4 Nokia: ok with merge with proposed changes Xiaomi: Generally okay to consider combining. Prefer to delete robot only keep UE. |
| CPR 14.1.8-6-5 | Subject to operator’s policy, agreement with authorized 3rd party, regulatory requirements and subscriber permission, 6G system shall support 3rd party application to provide AI traffic characteristics information including expected communication performance requirement to 6G network, in order to efficiently support communication of the AI traffic. ZTE: Subject to operator’s policy, agreement with authorized 3rd party, regulatory requirements and subscriber permission, 6G system shall support 3rd party application to provide AI traffic characteristics information, and the expected communication service performance requirements to 6G network, in order to efficiently support communication of the AI traffic. | PR 6.26.6-5 PR 6.28.6-1 | Exposure ZTE: propose to merge PR 6.28.6-1 and PR 6.26.6-5. Nokia: consider merge PR 6.28.6-1 into CPR 14.1.8-6-5 |
| - | Subject to operator policy, 6G network shall support means for an authorized third party to provide information about expected communication performance requirement, for the 6G network to provide data transfer accordingly for the application to enhance user experience. NOTE: For example, for AI inference application based on video, such information can refer to max allowed packet error rate and average packet error rate on a per video frame basis. The max allowed packet error rate characterizes the upper bound of packer error rate of each frame to achieve successful source decoding. The average packet error rate is averaged over a number of video frames related to providing acceptable user experience. | PR 6.28.6-1 | Performance |
| CPR 14.1.8-6-6 | Subject to operator’s policy, the 6G network shall be able to provide communication service for AI applications with various traffic characteristics (e.g. modality, tolerated error rate, priority, generation/arrival rate of data) efficiently for the AI applications to guarantee user experience. | PR 6.59.6-1 | Service providing [Ericsson] Is this QoS traffic or not, if it is best effort this is up to implementation to decide if adjustments are needed Nokia: consider merge into CPR 14.1.8-6-5 |
| Xiaomi: CPR 14.1.8-6-6 | Subject to operator’s policy, the 6G network shall be able to efficiently provide communication service for AI applications with various traffic characteristics (e.g. modality, tolerated error rate, priority, generation/arrival rate of data) . | PR 6.59.6-1 | Service providing Xm: unless we are intending to provide details of user experience requirements it seems better to leave it to AI service requirement? |
| Nokia: CPR 14.1.8-6-5 | Subject to operator’s policy, regulatory requirements and subscriber permission, 6G system shall support 3rd party application to provide traffic characteristics information (e.g. modality, tolerated error rate, priority, generation/arrival rate of data, burst data traffic) and expected communication performance requirements to 6G network, in order to efficiently support communication of the e.g. AI traffic. | PR 6.26.6-5 PR 6.28.6-1 PR 6.59.6-1 | Exposure Merge of CPR 5, 6 |
| CPR 14.1.8-6-7 | Subject to operator’s policy, the 6G system shall support the dynamic changes in QoS, when the traffic characteristic dynamically changes or is predicted to change, upon request by an authorized third party. | PR 6.61.6-1 | [Ericsson] Can this be merged with CPR 14.1.8-6-2? i.e applications or trusted 3rd party Nokia: consider merge into CPR 14.1.8-6-5 |
| Xiaomi: CPR 14.1.8-6-7 | Subject to operator’s policy, the 6G system shall support the dynamic changes in QoS, when the AI service traffic characteristic dynamically changes or is predicted to change, upon request by an authorized third party. | PR 6.61.6-1 | Xm: Understand that the difference compared to CPR 14.1.8-6-2, is the predicted aspect, otherwise this seems similar. ZTE: propose to merge with CPR 14.1.8-6-2 |