TSG-SA WG1 #36	S1-070500

Madrid, Spain, 23 - 27 Apr 2007	Agenda Item: 8.14

Title: TR 22.942 Overall System Requirements (Section 5)

Source:	 ZTE, China Mobile

Contact:	huang.zheng2@zte.com.cn

wubin@chinamobile.com

It is proposed to include the following text in section 5 of TR22.942.

# 5	Requirements

## 5.1	Suggested High Level Requirements

The VASM shall be implemented without influencing the existing SMS service.

The VASM shall be implemented without depending on the terminal’s capability.

Users shall be able to register, activate, deactivate, withdraw and reconfigure VASM via the UE, or web portals.

## 5.2	Overall System Requirements

### 5.2.1 Management of Service Information

-	Capabilities provided to the user

The VASM shall be able to support a request from an application to query/change the choice of services for a subscriber.

The VASM shall be able to support a request from an application to query/change the subscriber’s preferences for a certain service, for example:

i) To add or delete or modify a subscriber’s filtering conditions by which VASM can refuse some of the subscriber’s incoming messages.

ii) To modify a subscriber’s signature that will be appended to a SMS sent from the subscriber.

iii) To modify a subscriber’s forwarding address that substitutes for the subscriber’s original receiving address.

-	Capabilities provided to the network

The VASM shall be able to support to store the subscription information.

The VASM shall be able to support a request to query/change the subscriber’s information, for example:

i) To get the detail information about the subscriber’s service.

ii) To add or delete a subscriber’s service information.

The VASM shall be able to support a request to query the handling results of the subscription service.

### 5.2.2 SM Processing and Forwarding

The VASM shall be able to deal with the content of a SM, for example:

i) To insert content into the original SM and form a new SM (e.g. append the signature to the SM).

ii) To compile a SM by containing VASP’s information (e.g. construct a delivery report).

iii) To search in a SM for certain words as the filtering conditions.

The VASM shall be able to convert the format of a SM into other formats (e.g. email, wap message, etc).

The VASM shall be able to send the content of a SM to a reachable address, for example:

i) To send an email to email severs after converting the SM into an email.

ii) To send a wap message to a WAP Push Gateway after converting the SM into a WAP message.

iii) To send a SM to a subscriber by sending a SM to SMSC.

iv) To send a SM to VASP, such as delivery report message.

### 5.2.3 Management and Control of Network Based Repository

VASM shall be able to provide following capabilities to support network based repository:

-	The VASM shall allow configuring in such a way that certain sent or received messages of a particular user can be stored persistently in a network based repository.

-	The VASM shall be able to support a request from SMSC to persistently store a sent message in a network based repository at the time of sending.

-	The VASM shall be able to support a request from an application to upload certain messages into a network based repository for persistent storage.

-	The VASM shall be able to support a request from an application to retrieve/delete certain messages stored in a network based repository

-	The VASM shall be able to support a request from an application to view the list of messages and related attributes (e.g. sender, recipient, date/time, etc) in a network based repository

### 5.2.4 Addressing

The VASM shall support different addressing formats to identify the sender and recipient; it shall support the capability for both MSISDN [4] and e-mail addressing schemes [5] to be used.

The VASM shall be possible to submit one message to multiple recipients.

The VASM shall be able to support the request to hide the sender’s address from the recipient.