TSG-SA WG1 #36	S1-070501

Madrid, Spain, 23 - 27 Apr 2007	Agenda Item: 8.14

Title: TR 22.942 Requirements for Service priority and Interaction (Section 6)

Source:	 ZTE, China Mobile

Contact:   huang.zheng2@zte.com.cn

wubin@chinamobile.com

It is proposed to include the following text in section 6 of TR22.942.

# 6 Requirements for Service priority and Interaction

The different rule of VASM service priority and interaction should be provided by carrier.

The VASM shall be classified by priority and shall be triggered according to priority.

The VASM should provide the capability to preset the certain priority.

Following table summarizes the service interactions between pairs of services as identified in section 3.

|  |  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SMForwarding | SMFiltering | SMReceipt | SMNS | SMTMD |
| 1 | SMForwarding |  | Y1-B | N | Y1-D | X |
| 2 | SMFiltering | Y2-A |  | Y2-C | Y2-D | X |
| 3 | SMReceipt | N | Y3-B |  | N | X |
| 4 | SMNS | Y4-A | Y4-B | N |  | X |
| 5 | SMTMD | X | X | X | X |  |

Service Interactions Table

Note:

SMForwarding -- Indicates ‘Short Message Forwarding’.

SMFiltering   -- Indicates ‘Short Message Filtering’.

SMReceipt    -- Indicates ‘Short Message Receipt’.

SMNS        -- Indicates ‘Short Message Network Storage’.

SMTMD      -- Indicates ‘Short Message to Multiple Destinations’.

YN-C   -- Indicates interaction or confliction may exist between the corresponding services pair, detailed description will be given below.

N     -- Indicates interaction or confliction may not exist between the corresponding services pair.

X     -- Indicates simultaneous activation of the services pair is not allowed.

Explanation:

Y1-B (Y2-A) -- SMFiltering has the higher priority.  The messages filtered shall not be transferred.

Y1-D (Y4-A) -- SMForwarding has the higher priority than SMNS subscribed by called.  The messages transferred shall not trigger SMNS subscribed by original called.

Y2-C (Y3-B) -- SMFiltering has the higher priority than SMReceipt subscribed by called.  The messages filtered shall not trigger SMReceipt subscribed by called.

Y2-D (Y4-B) -- SMFiltering has the higher priority than SMNS subscribed by called.  The messages filtered shall not trigger SMNS subscribed by called.