3GPP TSG-SA WG1 Meeting #66	S1-141417

Sapporo, Japan, 12 – 16 May 2014	(revision of S1-141197)

Title:	EPS priority management

Agenda Item:	7.2

Source:	Alcatel-Lucent

Contact:	Betsy Covell (betsy.covell@alcatel-lucent.com)

Abstract:

In order to support different priorities of MCPTT calls, requirements on the 3GPP EPS are needed in addition to application level requirements.  Currently EPS systems provide 16 priority levels.  The network operator manages these levels according to the needs of the various customers and applications they support.

The following reference and EPS requirements are proposed to be included in TS 22.179.

****First Change****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

****Second Change****

## 4.6	PTT Priority Requirements

The MCPTT application manages the MCPTT priorities and issues requests to the EPS for prioritized services.  The EPS maps the MCPTT priorities into the  available priority levels defined in TS 23.303 [x].  The following requirements support the EPS management of MCPTT priorities.

It shall be possible to have multiple MCPTT Application priorities which are mapped to available EPS priority levels, based on network operator policy.

It shall be possible for the MCPTT Client on the UE to signal a different statuses to the MCPTT Application relative to a specific group, including at least “Public Safety Emergency” and “Normal”.