3GPP TSG-SA WG1 Meeting #66	S1-141101

# Sapporo, Japan, 12-16 May 2014														(revision of S1-14xxxx)

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.228 | CR | 0197 | rev | - | Current version: | 12.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use case for end to end WebRTC bearer support over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eWebRTCi |  |  |  |  |  |  | Date: |  | 2014-05-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Illustrate the capability to allow WebRTC IMS clients to use WEbRTC bearer and security protocols end to end over IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A use case illustrating use of WebRTC bearer protocols and security mechanisms between WebRTC IMS clients. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No use case for end to end WebRTC bearer paths or end to end security between WebRTC IMS clients. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.1, New clause H.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** Start First Change ****

# H.1	General

The following use cases have been provided to describe WebRTC client access to IMS.

1.	WebRTC-based application supports broad IMS client capabilities

2.	User gets his/her IMS service via third-party WebRTC-based application

3.	User gets third-party IMS service via WebRTC-based application

4.	Anonymous user gets IMS service via third-party WebRTC-based application

5.	End to end WebRTC bearer support over IMS

**** End First Change ****

**** Start Last Change ****

# H.X	End to end WebRTC bearer support over IMS

## H.X.1	Description

Jane is doing her taxes using an online tool and encounters a complexity with which she would like some help.  Her tax tool website includes a big red HELP button which connects her to a tax advisor associated with the tool.  Associated with the HELP button is a link to a third-party WebRTC-based application that offers access to communications services facilitated by an IMS operator. The third-party WebRTC-based application provides a specialized application to the user such as a customer services portal for a tax advisor. In addition to being able to utilize IMS capabilities, the tax advisor has an agreement with their IMS operator to use a WebRTC based codec as its preferred audio codec and, as an enterprise service,  to provide secure chat to communicate with WebRTC IMS client users. The IMS operator is able to convey the audio and chat session without bearer level protocol conversion which might interfere with quality and maintains the end to end security between Jane’s device and third party WebRTC-based application. Figure H.1 illustrates the configuration allowing end to end communication between WebRTC IMS clients.

Figure H.1 End to end WebRTC IMS communication

## H.X.2	Pre-conditions

Jane has a WebRTC capable browser on a device that supports a WebRTC codec and an end to end WebRTC security protocol.

Her tax advisor has a subscription with an IMS operator and provides a web site that provides access to a WebRTC-based portal to the IMS; the IMS does not support the WebRTC codec.

The tax advisor has a business relationship with the IMS operator that allows the tax advisor to incorporate access to specialized non-IMS capabilities (e.g., use of WebRTC codec, secure chat) into the features provided by the tax advisor’s WebRTC-based application.

The tax advisor’s web site provides customers the ability to select from various options (e.g., voice, chat, video, secure/unsecure channels) when they request online HELP.

The tax advisor has their WebRTC browser up and is registered with IMS and is able initiate services or to accept incoming sessions.

## H.X.3	Service flows

Jane accesses the tax advisor web site from her browser by clicking on the HELP button, which executes the WebRTC-based application along with the WebRTC IMS client functions needed for access to IMS.

Jane engages in an unsecured chat session with her tax advisor to resolve a question on completing her tax forms.  This chat session is hosted by IMS and does not provide end to end security.

While continuing the chat session, Jane opens an audio connection with her tax advisor as the discussion becomes more complex.  Since both endpoints are WebRTC IMS clients supporting the same WebRTC codec, and the tax advisor’s subscription allows it, the bearer path is conveyed without protocol conversion, allowing Jane and the tax advisor to experience very high quality audio.

While discussing the tax issue, Jane and her tax advisor decide it would be helpful to share figures, calculations, and personal tax information.  To do this securely, they select a secure chat line that utilizes the WebRTC end to end security.

Jane is satisfied with the quality and security of her exchange with her tax advisor.

## H.X.4	Post-conditions

After concluding the session with Jane, the tax advisor’s browser window remains available to initiate other services or to accept incoming sessions, if supported.

**** End Last Change ****