3GPP TSG-SA WG1 Meeting #69	S1-150144

Sanya, P.R. China, 2–6 February 2015                                                                  (revision of S1-14xxxx)

Title:				V2I with Service RSU

Agenda Item:	5

Source:			Huawei, Hisilicon

Document for:	Discussion and Approval

Contact:			Laurence Meriau laurence.meriau@huawei.com; Shi Yi wn.shiyi@huawei.com

# Introduction

This contribution clarifies that from 3GPP perspective, i.e. for the transport of V2X application service, the V2I service between vehicles and a “Services RSU” (like traffic lights RSU) is similar to V2V service between two vehicles.

# Discussion

ETSI TR102.638 [1] has defined two different types of Road Side Units (RSU) to support 2 types of V2I services that we call here “Service RSU” and “Access RSU”.

- V2I between vehicle and “Service” RSU is similar to V2V between 2 vehicles:

“Service” RSU can be for example a unit in a traffic light (See [1] ETSI TR 102.638 Annex C2.2) broadcasting timing data associated to its current state (e.g. time remaining before switching green, amber, red) or a unit in a street panel ([1] ETSI TR 102.638 Annex C2.1) to continuously broadcast some speed limit information at a given frequency to warn the driver of not violating the regulation, as the following:

Fig 1. Service RSU in the street lights to advice of the light status

Fig2. Service RSU in a street panel to advice regularly the speed limit

Both “Service RSU” and OBU broadcast CAM / DEMN messages in a similar way. “Service RSU” broadcasts CAM (Cooperative Awareness Message) and DENM (Decentralized Environmental Notification Message) like what an OBU (On Board Unit) would do for V2V service in a vehicle. They also have similar traffic models and service requirements like message size, frequent transmission, short range communication, low latency (<100ms) and high reliability.

The difference is that the “Service RSU” has a fixed location while a vehicle is usually mobile. But support mobile vehicles service allows support of fixed RSU services. So both can be considered together with requirements based on ETSI specification.

Those devices (“Service RSU” and OBU in vehicles) can be considered with similar requirements in the SA1 V2X study.

- “Access RSU” instead provides access to internet for vehicles for example to access to the personal data  (TR 102.638 Annex C3.9):

Fig3. “Access RSU” allowing cars to access to internet

“Access RSU” provides internet access to servers for vehicles. The data exchanged between the vehicle and the “Access RSU” is  IP data, They are not CAM / DENM data..

The differences between “Service RSU” and “Access RSU” are described in the following table:

Table1 comparison of “Service RSU” and “Access RSU”

| Items | “Service” RSU | “Access” RSU |
| --- | --- | --- |
| Provided service | Broadcasting CAM/DENM itself | Providing access to internet, not broadcasting CAM/DENM |
| Communication mode | Broadcasting | P2P between vehicle and backend server |
| Traffic model | Frequent transmission (1~10Hz), pre-defined packet size for each transmission | Infrequent transmission, can be big packet size |
| Service requirement | Low latency, high reliability, short range communication | Large coverage, service continuity |

# Conclusion

The contribution clarifies the difference between “Service RSU” and “Access RSU”.

The “Service RSU” and “OBU in vehicle” have the same traffic model and service requirements according to ETSI ITS TC study. So both V2V/V2I for Access RSU can be considered together regarding requirements.

The “Access RSU” is different and can be studied separately to allow access to internet.

# Proposal

As V2I with Service RSU has similar requirements on services and messages to V2V, it is proposed that SA1 considers merging V2V and V2I agreeable requirements into a single CR.

# References

[1] ETSI TR 102 638 V1.1.1 Technical Report Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Definitions, 2009.06