3GPP TSG-SA WG1 Meeting #71 ad hoc on SMARTER (5G)	S1-153168

Vancouver, Canada, 19 – 21, October, 2015               (Revision of S1-153108)

Title:		SMARTER Use Case on 5G Connectivity using satellites

Agenda Item:	3.5	New use cases

Source:		Dish Network

Contact: 	Ojas Choksi (ojas.choksi@dish.com)

Abstract: This submission proposes a new use case for using satellites to deliver 5G connectivity for a wide variety of services

------------- 1st CHANGE -----------

5.x 5G Connectivity Using Satellites

### 5.x.1	Description

One of the key requirements across various industry white papers is ensuring high availability and service reliability via ubiquitous coverage. For a 100% geographic coverage requirement case, satellites are ideal in covering areas that cannot be covered by terrestrial networks. As part of wide area coverage, the ARIB’s 5G white paper [x] and 5GMF’s presentation at the RAN 5G workshop also referenced satellite-terrestrial cooperation as part of the mobile networks of 2020 and beyond.  The ARIB 5G whitepaper called for a high degree of commonality between the terrestrial and satellite radio interfaces.  Similarly, the NGMN white paper [2], as part of its technology candidate analysis, has also listed satellites as an example of an emerging technology that could be relevant as part of 5G.

There are several use cases that can only be served by satellites, and there are other use cases for which satellites provide a more efficient solution.  These use cases are described below:

- Areas where it is not possible to deploy terrestrial towers:  For example, maritime services, coverage on lakes, islands, mountains or other recreational areas that can only be covered by the satellites

- Disaster relief:  During natural disasters or other unforeseen events that entirely disable the terrestrial network, satellites are the only option.

- Emergency response:  Besides wide scale natural disasters, there are specific emergency situations in areas where there is no terrestrial coverage.  For example a public safety uses case of an accident in a power plant.

- Secondary/backup connection (limited in capability) in the event of the primary connection failure or for connected cars

- Connectivity in rural areas that are hard to cover using terrestrial networks

- Connectivity for remotely deployed sensors, e.g. farms, substations, gas pipelines, digital signage, remote road alerts, etc.

- Low bit-rate broadcast services: Satellites can broadcast wide area emergency messages at a more efficient rate than terrestrial networks.

Standards based satellite specifications have been developed across multiple forums.  Typically, these satellite standards are modifications or enhancements of existing terrestrial specifications.  Some examples of this are: GMR (GEO-Mobile Radio Interface) which is an ETSI standard derived from 3GPP GSM/UMTS, xHRPD (Extended Cell High Rate Packet Data) which is a 3GPP2 standard derived from EV-DO Rev A, and LTE over satellite specifications which is another ETSI standard.

### 5.x.2	Potential Service Requirements

The 3GPP system shall support highly available and reliable connectivity using satellites for use cases such as ubiquitous coverage, disaster relief, public safety requirements, emergency response, remote sensor connectivity, broadcast service, etc.

The 3GPP system shall support up to 100% geographic coverage.

The 3GPP system shall support an air-interface with latency of up to 275 ms when satellite connection is involved.

The 3GPP system shall support seamless mobility between terrestrial and satellite based networks with widely varying latencies.

### 5.x.3	Potential Operator Requirements

----------  2nd CHANGE --------

## References

### The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NGMN 5G White Paper v1.0

[3]	China IMT2020 (5G) Promotion Group white paper “5G Vision and Requirements” May 2014.

[4]	3GPP TR 36.913 v12.0.0. “Requirements for further advancements for Evolved Universal Terrestrial Radio Access (E-UTRA), Release 12”

[5]	China IMT2020 (5G) Promotion Group white paper “5G Concept” February 2015.

[6]	4G Americas’ Recommendations on 5G Requirements and Solutions, October 2014.

[7]	FP7 COMBO, “Requirement for converged fixed and mobile networks”, deliverable, D2.4, June, 2014, available at http://ict-combo.eu/data/uploads/pdf-combo-v2/combo_d2.4_wp4_20june2014_tid_v2.0_sec.pdf

[8]	3GPP TS 22.105, “Services and service capabilities”, v 12.1.0, Dec 2014.

[9]	http://www.ravepubs.com/wp-content/uploads/2013/10/Almo-G4-UHD-Presentation-PDF-v1.pdf

[10]	GSMA, “Understanding 5G: perspectives on future technological advancements in mobile”, Dec., 2014.

[11]	Requirements and Current Solutions of Wireless Communication in Industrial Automation, A. Frotzscher et al., IEEE ICC’14 – W8: Workshop on 5G Technologies, 2014

[12]	Design of a Low-Latency, High-Reliability Wireless Communication System for Control Applications, M. Weiner et al., IEEE ICC 2014 – Selected Ares in Communications Symposium, 2014.

[13]	Funding project BMBF by German government on wireless Factory Automation (2014), http://www.bmbf.de/foerderungen/22967.php

[14]	Unplugged but Connected: Design and Implementation of a Truly Wireless Real-Time Sensor/Actuator Interface, G. Scheible et al, IEEE Industrial Electronics Magazine, 2007

[15]	Wireless Sensors in Industrial Time-Critical Environments, Jose Cecilio & Pedro Furtado, Springer, ISBN 978-3-31-02888-8

[16]	WirelessHART versus ISA100.11a: The Format War Hits the Factory Floor, S. Peterson & S. Carlsen, IEEE Industrial Electronics Magazine, Dec 2011.

[17]	Wireless Industrial Monitoring and Control Networks: The Journey So Far and the Road Ahead, P. Zand et al, J. Sens. Actuator Netw. 2012.

[18]	Performance Evaluation of WirelessHART for Factory Automation, S. Petersen & S. Carlson, IEEE Emerging Technologies & Factory Automation, 2009.

[19]	Industrial Wireless Sensor Networks, ON World, www.onworld.com, 2010

[20]	Coexistence of Wireless Systems in Automation Technology, ZVEI - German Electrical and Electronic Manufacturers’ Association, 2009.

[21]	China IMT2020 (5G) Promotion Group. “5G Network Technology Architecture” May 2015

[22]	European Commission’s 5G PPP “5G Vision”, Feb. 2015. www.5g-ppp.eu.

[x]	ARIB 2020 and Beyond Ad Hoc Group White Paper, October 2014. http://www.arib.or.jp/english/20bah-wp-100.pdf