3GPP TSG-SA WG1 Ad-hoc on SMARTER (5G)	S1-153169

Vancouver, Canada, 19-21 October 2015 (revision of S1-153109,S1-153099,S1-153040)

Title:	Discuss grouping of SMARTER requirements

Agenda Item:	3.1

Source:	Huawei

Contact:	Laurence Meriau laurence.meriau@huawei.com

Abstract: SA1 needs to identify requirements for SMARTER, some grouping will help to clarify the main directions to focus on for network evolution.

Note: this is a re-submission of Belgrade S1-152087, with clarifications and improvements due to emails discussions on grouping:

- Criteria used for grouping

- Grouping proposal

- Matrix of requirements showing the detailed reasons for grouping, it distinguishes Verticals (eMBB, mMTC... ) and consider all new use cases agreed in Belgrade

- Matrix to compare different companies grouping proposals (updated)

- Proposals for TR changes: it is proposed to show groups (so the expected SID) at the end of the TR

- Criteria used for Grouping

There is a large variety of use cases in the TR. We used following criteria for Grouping:

Criteria#1:

- we considered each individual requirement

- and created 2 detailed Matrix in section III & IV to distinguish the groups of Verticals and Horizontal:

  - Matrix of section III: the 279 requirements of the TR show commonalities/difference between use cases,

  - Matrix of section IV: gives an overview of some other companies grouping (Vodafone, Alcatel-Lucent, Qualcomm), to help to reach a common SA1 visibility.

This matrix shows naturally certain groups

Criteria#2: We considered SA1 has some benefit to identity each Vertical group when some requirements differ:

- To ensure SA1 floor share the same understanding on needs

- To ensure groups support specific business model: for ex separate V2X from eMBB as this could be 2 separate business: with a MVNO deploying for example V2X, with no MBB business at all;

- To allow flexibility in Stage2:

  - It is expected that if a UE needs support for 2 types of Verticals (like a UE using eMBB service in a Vehicle), then 2 groups of characteristics have to be considered for the slice creation. While if a UE supports only 1 Vertical, there is no need for the operator to consider characteristics of the other Verticals for this UE for the slice supporting the UE.

  - For ex: V2X differs from eMBB due to its high reliability, lower rate, high speed and high positioning accuracy, V2X group also needs to consider the migration of the V2X service e.g. specified in 3GPP R14 while eMBB group does not need to consider this aspect.

- Grouping proposal

- Groups obtained from Matrix for Verticals (eMBB, V2X, Critical Communication, mMTC):

The following table shows Groups for verticals from Matrix of section III.

The column on the right highlights the main differences between this group and others.

| Verticals Summary | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Traffic density | Conn. density | UE speed | Position accuracy | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group#1 eMBB | Very High data rate (Up to 10 Gbps) | Low latency |  |  | High traffic density Tbps/ km2） | High density for UE |  |  | Characterised by very high traffic, high bit rate. No Reliability needed, no accuracy need |
| Group#3 Critical Comm. |  | Realtime low latency | Ultra high Reliability |  | High Density |  |  | Precise position within [10 cm] | Characterised by low latency, ultra high reliability |
| Group#4 mMTC |  |  |  | Coverage enhancement, Efficient resource and signalling to support low power | High Density | Low mobility (for majority of MTC cases except for inventory) |  | High positioning accuracy in both outdoor and indoor (e.g., 0.5m) | Important difference with CC is: No Latency, No reliability |
| Group#2 V2X | Medium Rate (10 of Mbps per device) | Low latency | High Reliability |  | Medium traffic density | Medium connection Density | High Speed(up to 500 KM/h) | High positioning accuracy (e.g. 0.1 meters) | V2X differs from eMBB due to its high reliability , lower rate , high speed and high positioning accuracy |

- Group#1: eMobile Broadband (eMBB)

  - 5.5 Mobile broadband for indoor scenario

  - 5.6 Mobile broadband for hotspots scenario

  - 5.7 On demand networking (support high density of terminals)

  - 5.10 Mobile broadband services with seamless wide area coverage

  - 5.11 Virtual presence

  - 5.30 Connectivity Everywhere

- Group#2: Critical Communication (CC):

  - 5.1  Ultra-reliable communications

  - 5.12 Connectivity for drones

  - 5.13 Industrial control

  - 5.14 Tactile Internet

  - 5.15 Localized real-time control

  - 5.17 Extreme real-time communications and tactile internet

  - 5.18 Remote control

  - 5.44  Cloud Robotics

  - 5.45  Industrial Factory Automation

  - 5.46  Industrial Process Automation

  - 5.50  Low-delay speech coding

  - 5.54  Local UAV Collaboration

- Group#3: mMTC:

  - 5.19 Light weight device configuration

  - 5.20 Wide area sensor monitoring and event driven alarms

  - 5.24  Bio-connectivity

  - 5.25  Wearable Device Communication

  - 5.40  Devices with variable data

  - 5.41  Domestic Home Monitoring

  - 5.42  Low mobility devices

  - 5.43  Materials and inventory management and location tracking

  - 5.59  Massive Internet of Things M2M and device identification

- Group#4: V2X:

  - 5.10  Mobile broadband services with seamless wide-area coverage (500 km/h speed in V2X)

  - 5.29  Higher User Mobility

  - 5.32  Improvement of network capabilities for vehicular case

  - 5.33  Connected vehicles

  - 5.53  Vehicular Internet& Infotainment

B) Horizontal Group#5: Network Operation for foundation for a new architecture:

This groups some “horizontal” requirements that are used to support Verticals.Network operation looks to be an important group to be considered in SMARTER to allow the operator to have framework and the flexibility to select the network functions and capabilities it needs to provide the diverse verticals, while avoiding other unnecessary functionalities and to support performances needed for future services and  reduce the time to market for new services.

A single group is proposed with some particular “families”. A split in independent groups looks to be complex with no clear benefit (see Comparison of companies’ position in Table of section III).

  - 5.2   Network Slicing

  - 5.4   Migration of Services from earlier generation

  - 5.8   Flexible application routing

  - 5.9   Flexibility and scalability

  - 5.16 Coexistence with legacy systems

  - 5.21  IoT Device Initialization

  - 5.22  Subscription security credentials update

  - 5.35  Context Awareness to support network elasticity

  - 5.47  SMARTER Service Continuity

  - 5.49  Network capability exposure

  - 5.51  Network enhancements to support scalability and automation

  - 5.58  Use Case for Green Radio

The following use cases consider the access types:

  - 5.23 Access from less trusted networks

  - 5.26 Best Connection per Traffic Type

  - 5.27 Multi Access network integration

  - 5.28 Multiple RAT connectivity and RAT selection

And some use cases focussed on minimum service capability by the network:

  - 5.3   Lifeline communications / natural disaster

  - 5.31 Temporary Service for Users of Other Operators in Emergency Case

  - 5.48  Provision of essential services for very low-ARPU areas

Some other use cases are to attract new services/create new values

Note: either the service will be defined by 3GPP (like done for Public Safety, MCPTT) or 3GPP will have to consider services defined in other SDOs. SA1should define the relationship with these services, for example considering currently ETSI ISG MEC work on this subjects and how 3GPP can support this work to offer value added with the operator’s network to attract these service. We consider it is important SA1 gives a clear message that future network has to attract new service, that this is a target also for the new architecture discussions.

  - 5.36       In-network caching

  - 5.37       Routing path optimization when server changes

  - 5.38       ICN Based Content Retrieval

  - 5.39       Wireless Briefcase

And use cases for network optimisation:

  - 5.34       Mobility on demand

  - 5.52       Wireless Self-Backhauling

  - 5.55      High Accuracy Enhanced Positioning (ePositioning)

  - 5.56       Broadcasting Support

  - 5.57       Ad-Hoc Broadcasting

Also some verticals use cases impact the Network Operation (see table below for detailed requirements):

  - eMBB

  - V2X

  - mMTC

  - CC

- Matrix of all requirements of the TR

This matrix below list all the TR use cases and all the requirements of each use case (lines) which are put in appropriate column based on their purpose (columns).

When characteristics are similar, different groups are visible and a grouop # and name is addedas well as line summarizing the main characteristics.

Note: in the following tables:

- QoE and Peak data rate includes: “QoE Data Rate” and “Peak Data Rate”

- Latency includes “UE-to-network Latency” and “UE-UE Latency”

- Reliability includes “Reliability”, “Availability”, “Jitter”

- Communication efficiency includes “Communication Efficiency” , “Traffic Freq.”, “Comm. Range” and “Battery”

- Network operation contains: (Slice, provisioning, roaming, broadcast, security, service exposure, legacy service support, LI/Charging, Minimum service, Multiple access)

- Network Optimization contains: Backhaul, Mobility, Relay, Traffic Multiplex, UE-UE Networking, Energy saving)

| Group #1 eMBB | QoE and Peak data rate | Latency | Reliability | Comm efficiency | Traffic density | Connection density | UE speed | Positioning accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eMBB summary | Very High data rate (Up to 10Gbps) | Low latency |  |  | High traffic density Tbps/ km2） | High density for UE/traffic |  |  |
| 5.5 Mobile broadband for indoor scenario | Up to Gbps of level |  |  |  | Tbps/ km2 |  |  |  |
|  | Peak rate: 10 of Gbps |  |  |  |  |  |  |  |
|  |  | very low |  |  |  |  |  |  |
| 5.6 Mobile broadband for hotspots scenario | Up to Gbps of level while the user is moving slowly |  |  |  | Tbps/ km2 |  |  |  |
|  | Peak rate:10 of Gbps while moving slowly |  |  |  |  |  |  |  |
| 5.7 On-demand Networking | DL: 300Mbps; UL: 50Mbps |  |  |  |  | in 200-2500 /km2 |  |  |
|  | DL:50Mbps; UL: 25Mbps |  |  |  |  | in 2000/km2 |  |  |
| 5.10 Mobile broadband services with seamless wide-area coverage | anytime and anywhere, e.g., 100Mbps. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | e.g., 500km/h. |  |
|  |  |  |  |  |  | 500 active UEs simultaneously |  |  |
|  |  | low latency for high speed |  |  |  |  |  |  |
| 5.11 Virtual presence | high bandwidth (bidirectional) | low latency |  |  |  |  |  |  |
| 5.30 Connectivity Everywhere |  |  | aerial object with reliable mobile broadband connectivity. |  |  |  |  |  |
|  |  |  | nautical object with reliable mobile broadband connectivity. |  |  |  |  |  |
|  |  | reliable low-latency connectivity between aerial objects |  |  |  |  |  |  |

| Group #2 Critical Communication | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Connection density | UE speed | Position accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CC summary |  | Realtime /low latency | Ultra high Reliability | Comm. Range:10-20m | High Density |  | ## Precise position within [10 cm] |
| 5.1 Ultra-reliable communications |  |  | limit the duration of service interruption for mission critical traffic |  |  |  |  |
|  | “Substation protection and control”: ~12.5Mbps per MU at 256 samples/cycle | “Substation protection and control”: as low as 1 ms end-to-end; the case “Smart grid system”: less than 8 ms | “Substation protection and control”: Packet loss rate: as low as 1e-04; the case “Smart grid system”: reliably delivered in 8 ms |  |  |  |  |
| 5.12 Connectivity for drones |  | Round trip latency less than [150 ms] |  |  |  |  |  |
|  |  |  | [near 100%.] |  |  |  |  |
|  |  |  | the same level for current aviation Air Traffic Control (ATC) |  |  |  |  |
|  |  |  | Realibity with Priority, Precedence, Preemption (PPP) mechanisms |  |  |  |  |
|  |  |  |  |  |  |  | within [10 cm] |
|  | high speed uplink bandwidth at least [20Mbps], for a flying UE at low altitude of [10-1000] meters |  |  |  |  | high speed as maximum as [300km/h]. |  |
| 5.13 Industrial Control |  | low latency (~1 ms) |  |  |  |  |  |
|  |  |  | high reliability |  |  |  |  |
|  |  |  | high availability |  |  |  |  |
|  | UL:10 of Mbps per device |  |  |  | in a dense environment |  |  |
| 5.14 Tactile Internet |  | low latency (~1 ms) |  |  |  |  |  |
|  |  |  | high reliability |  |  |  |  |
| 5.15 Localized real-time control |  | UE-UE latency: low latency [1-10 ms] | Extremely high reliability |  |  |  |  |
| 5.17 Extreme real-time communications and the tactile internet |  | 0.5ms one-way delay |  |  |  |  |  |
| 5.18 Remote Control |  | Round trip latency less than [150 ms] |  |  |  |  |  |
|  |  |  | near 100% |  |  | 120km/h |  |
|  |  |  | Reliability to be at the same level for current aviation Air Traffic Control (ATC) |  |  |  |  |
|  |  |  | With Priority, Precedence, Preemption (PPP) mechanisms |  |  |  |  |
|  |  |  |  |  |  |  | < [10 cm] in densely populated areas. |
| 5.44 Cloud Robotics | UL:synchronized audio, video and data in real time |  |  |  | high density distribution |  |  |
|  |  | end to end latency lower than [10ms] |  |  |  |  |  |
| 5.45 Industrial Factory Automation |  | cycle times of [1ms to 2ms.] |  |  |  |  |  |
|  |  |  | [below 10usecs.] |  |  |  |  |
|  |  |  | [below 10-9.] |  |  |  |  |
|  |  |  |  | Comm. Range:10-20m |  |  |  |
| 5.46 Industrial Process Automation |  |  |  |  | 10k sensor /10sqkm |  |  |
|  |  |  | Packet loss <10-5 |  |  |  |  |
|  |  | 50-100ms two way |  |  |  |  |  |
|  |  |  |  | Battery: multiple years, transaction rate of one every few seconds |  |  |  |
| 5.50 Low-delay speech coding |  | speech with very low one-way service latency [10 ms] |  |  |  |  |  |
| 5.54 Local UAV Collaboration |  | [10 ms] |  |  |  |  |  |
|  |  |  | Near [100%] |  |  |  |  |
|  |  |  | Priority, Precedence, Preemption (PPP) |  |  |  |  |
|  |  |  |  |  |  |  | < [10 cm] due to multiple UAVs that may need to collaborate in close proximity to one another. |

| Group #3 massive MTC | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Connection density | UE speed | positioning accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mMTC summary |  |  |  | Coverage enhancement, Efficient resource and signalling to support low power | High Density | Majority expected with low mobility (location tracking use case for V2X group) | High positioning accuracy in both outdoor and indoor (e.g., 0.5m) |
| 5.19 Light weight device configuration |  |  |  | support devices (e.g., smart meter) with limited communication requirements and capabilities (e.g., devices without an IMS client). |  |  |  |
|  |  |  |  | support a lighter weight signalling for device configuration (i.e., service parameters) than is currently available in EPS. |  |  |  |
| 5.20 Wide area sensor monitoring and event driven alarms | UL only |  |  | low power devices , infrequent uplink data |  |  |  |
|  |  |  |  | power efficiency (e.g., battery life up to more than 10 years |  |  |  |
|  |  |  |  | efficient data transmission with limited resource and signalling cost |  |  |  |
|  |  |  |  |  | high density massive connections (e.g.1 million connections per square kilometre) |  |  |
|  |  |  |  | coverage enhancement (e.g., 20dB better coverage than Rel 99 GPRS system). |  |  |  |
| 5.24 Bio-connectivity |  |  |  |  |  |  |  |
|  |  |  |  | efficient transfer of data for UEs which only support 3GPP device-to-device communication |  |  |  |
| 5.25 Wearable Device Communication |  |  |  | an UE (e.g wearable device) to access to the 3GPP network directly, or via an authorized UE (e.g. smart phone) when the two UEs are within a short range communication (e.g., less than [TBD]m) |  |  |  |
| 5.40 Devices with variable data |  |  |  | efficient and flexible for both low throughput short data bursts and high throughput data transmissions (e.g., streaming video) from the same device. |  |  |  |
|  |  |  |  | support efficient signaling mechanisms (e.g., signaling is less than payload). |  |  |  |
|  |  |  |  | reduce signalling overhead for security needed for short data burst transmission, without reducing the security protection provided by 4G 3GPP Systems. |  |  |  |
| 5.41 Domestic Home Monitoring |  |  |  | Payload size: messages up to 256 octets per message. |  |  |  |
| 5.42 Low mobility devices |  |  |  | accept information from large numbers of stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). |  |  |  |
|  |  |  |  | support a resource efficient mechanism to provide information to a stationary device |  |  |  |
|  |  |  |  | provide resource efficient support for stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). |  |  |  |
| 5.43 Materials and inventory management and location tracking |  |  |  | a resource efficient mechanism | to accept information from large numbers of locally dense devices, possibly simultaneously. |  |  |
|  |  |  |  | enable sufficient indoor and outdoor coverage (e.g., 20dB better coverage than legacy Rel 99 GPRS system) for a large number of locally dense low power devices. |  |  |  |
|  |  |  |  |  | a mechanism to manage resource (e.g., radio resources) sharing by large numbers of locally dense devices efficiently. |  |  |
|  |  |  |  | with reduced battery consumption. | communication service for high density of devices up to (e.g., 1 million devices per km2) | with high mobility at minimum of 100 km/h |  |
|  |  |  |  |  |  |  | high positioning accuracy in both outdoor and indoor scenarios (e.g., 0.5m) |
| 5.59 Massive Internet of Things M2M and device identification |  |  |  |  | connection densities of [200,000 / km2] |  |  |

| Group #4 V2X (high mobility access) | QoE and Peak data rate | Latency | Reliability | Comm . efficiency | Traffic density | Connection density | UE speed | Position accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X summary | Medium Rate (tens of Mbps per device) | Low Latency | High Reliability |  | Medium traffic density | Medium connection Density | High Speed High Speed (up to 500 KM/h) | High positioning accuracy (e.g. 0.1 meters) |
| 5.10 Mobile broadband services with seamless wide-area coverage |  |  |  |  |  |  | e.g., 500km/h. |  |
| 5.29 Higher User Mobility |  |  |  |  |  |  | support enhanced mobile broadband services in fast moving vehicles (e.g. up to 500 km/h) |  |
|  |  |  |  |  |  |  | support enhanced connectivity services in fast moving airplanes (e.g. up to 1000 km/h) |  |
|  |  |  |  |  |  |  | provide the mobile broadband service in fast moving vehicles with enhanced system experience. |  |
|  |  |  |  |  |  |  | provide the airplanes connectivity service with enhanced system experience. |  |
| 5.32 Improvement of network capabilities for vehicular case | Streaming:[100] Mbps (DL) |  |  |  |  |  |  |  |
|  |  | (end-to-end) is less than [1]s |  |  |  |  |  |  |
|  |  | (over the air) is less than [200] ms |  |  |  |  |  |  |
|  |  |  |  |  |  |  | up to 100 km/h |  |
|  | Video call: > [10] Mbps (DL and UL) |  |  |  |  |  |  |  |
|  |  | (end-to-end) is less than [150]ms |  |  |  |  |  |  |
|  |  | (over the air) is less than [30] ms |  |  |  |  |  |  |
|  |  |  |  |  |  |  | up to 100 km/h |  |
|  | Gaming: > [50-120] Mbps (DL or UL) |  |  |  |  |  |  |  |
|  |  | (end-to-end) is less than [7.5]ms |  |  |  |  |  |  |
|  |  | (over the air) is less than [1.5] ms |  |  |  |  | up to 100 km/h |  |
| 5.33 Connected vehicles |  | 1 millisecond end-to-end latency |  |  |  |  |  |  |
|  |  |  | nearly 100% |  |  |  |  |  |
|  | tens of Mbps per device |  |  |  | in a dense environment |  |  |  |
|  |  |  |  |  |  |  | high mobility (e.g., absolute speed more than 200 km/h while relative speed more than 400 km/h). |  |
|  |  |  |  |  |  |  |  | high positioning accuracy (e.g. 0.1 meters) |
|  |  |  |  |  |  | high density of connections for vehicles (e.g. the number of vehicles can exceed 10000 in scenarios with multiple lanes and multiple levels and types of roads) |  |  |
| 5.53 Vehicular Internet& Infotainment | Internet access: at least [0.5Mb/sec] |  |  |  |  |  |  |  |
|  | Music streaming at least [1Mb/sec] |  |  |  |  |  |  |  |
|  | video streaming at least [5Mb/sec] |  |  |  |  |  |  |  |
|  | UHD streaming at least [15 Mb/sec] |  |  |  |  |  |  |  |
|  |  | no more than [100ms] for internet browsing |  |  |  |  |  |  |
|  |  |  |  |  |  |  | up to 200km/hr. |  |
|  |  |  |  |  |  | up to [2000] vehicles in a given service area [1km2] | ranging from 0km/h (e.g. in a traffic jam) to 200km/h. |  |

Group #5: Network operation

Note: the 1st column also list use cases already existing in Vertical groups tables, because they have requirements that have to be considered for the group on network Operation:

| Use case | Network operation | Network Optimization |
| --- | --- | --- |
| 5.6 Mobile broadband for hotspots scenario |  | flexible and efficient backhaul especially outdoor |
| 5.7 On-demand Networking | adjust the network capacities dynamically |  |
| 5.1 Ultra-reliable communications |  | multiplexing of mission critical traffic and nominal traffic |
| 5.14 Tactile Internet | very difficult to block, modify, or hijack |  |
| 5.15 Localized real-time control |  | self-organized dynamic networking for multi-hop localized network |
| 5.18 Remote Control |  | Seamless connection for fast-moving |
| 5.45 Industrial Factory Automation | sufficiently integrity- and confidentiality |  |
| 5.46 Industrial Process Automation | sufficiently integrity- and confidentiality-protected |  |
| 5.54 Local UAV Collaboration | at the level for current aviation Air Traffic Control (ATC |  |
| 5.33 Connected vehicles | data transmission from one point to multipoint (e.g. multicast and/or broadcast) |  |
| 5.20 Wide area sensor monitoring and event driven alarms | a resource efficient mechanism to provide service parameters and activate groups of low power devices |  |
|  | an appropriate and efficient authentication mechanism for low power devices. |  |
|  | provides appropriate and efficient confidentiality and integrity protection for mobile originated transfer from low power devices. |  |
| 5.24 Bio-connectivity |  | allows the network to be aware of the UEs operating with assistance from a UE-to-network relay. |
| 5.25 Wearable Device Communication |  | service continuity for an UE when it changes from direct access to indirect access via another UE, and vice versa. |
|  | Data transmission in secure manner for an UE, when it’s connected to the network via another UE. |  |
|  | wearable device should have a subscription associated to its own subscriber’s identity (e.g. IMSI) with mobile operator. |  |
| 5.41 Domestic Home Monitoring | support of a UE connected to remote monitoring (that suits home monitoring/control) and includes: Static mobile, Group connection, Normal data service and service for low volume, low frequency updates. |  |
|  | IoT Security is provided only by the connected UE. Further security measures are for implementation at the IoT level applications. |  |
| 5.43 Materials and inventory management and location tracking | efficient authentication for low power devices. |  |
|  | confidentiality and integrity protection for data from low power devices. |  |
|  | efficiently manage one or more devices associated with a service (e.g., inventory tracking service). |  |
|  | support roaming |  |
| 5.59 Massive Internet of Things M2M and device identification | support network servers/applications and devices to identify, address and reach other devices, in a consistent manner independently of how these devices are connected. |  |
| 5.2 Network Slicing | x |  |
| 5.4 Migration of Services from earlier generations | x |  |
| 5.8 Flexible application traffic routing | x |  |
| 5.9 Flexibility and scalability | x |  |
| 5.16 Coexistence with legacy systems | x |  |
|  | x |  |
|  | x |  |
| 5.21 IoT Device Initialization | x |  |
|  | x |  |
| 5.22 Subscription security credentials update | x |  |
|  | x |  |
| 5.35 Context Awareness to support network elasticity | x |  |
| 5.47 SMARTER Service Continuity | x |  |
| 5.49 Network capability exposure | x |  |
| 5.51 Network enhancements to support scalability and automation | x |  |
| 5.58 Use Case for Green Radio |  | [1000] times energy efficiency compared to legacy system. |
| 5.23 Access from less trusted networks | x |  |
| 5.26 Best Connection per Traffic Type | x |  |
| 5.27 Multi Access network integration | x |  |
| 5.28 Multiple RAT connectivity and RAT selection | x |  |
| 5.3 Lifeline communications / natural disaster | maintain an appropriate level of communications security for temporary service. |  |
| 5.31 Temporary Service for Users of Other Operators in Emergency Case |  | support temporary service for users of other than home operators as temporary users in emergency case by serving operator policy |
|  |  | support defining the limited set of necessary communication services and acceptable terminal features for temporary users by serving operator policy |
|  | maintain an appropriate level of communications security for temporary service |  |
| 5.48 Provision of essential services for very low-ARPU areas | x |  |
|  | x |  |
|  |  | minimise as much as possible the traffic (Data and signalling) on the interfaces between the access network and the core network in order to reduce the amount of backhaul traffic. |
|  |  | be activated/deactivated either manually (e.g., on demand), or automatically (e.g., reaching a threshold). |
|  |  | Service may be restricted to authorized users |
|  |  | The base station may be in listen mode |
|  |  | Traffic density: provide the essential services with traffic density of 16 Mbps / km2 |
|  | x |  |
|  | inform UEs what capabilities are supported (e.g., to allow UEs to determine if the network provides the required capabilities). |  |
| 5.36 In-network caching | x |  |
| 5.37 Routing path optimization when server changes | x |  |
| 5.38 ICN Based Content Retrieval | x |  |
| 5.39 Wireless Briefcase | x |  |
| 5.34 Mobility on demand |  | define different levels of mobility support for different UEs. |
|  |  | update the level of mobility support provided for a UE. |
| 5.52 Wireless Self-Backhauling |  | x |
| 5.55 High Accuracy Enhanced Positioning (ePositioning) | support co-existence with legacy 3GPP positioning service and migration to higher accuracy positioning service |  |
| 5.56 Broadcasting Support | x |  |
| 5.57 Ad-Hoc Broadcasting | x |  |

- Comparison of companies grouping proposals

## We considered contributions received from VF, QC and ALU and Nokia.and also added in 1st column the list of requirements for each use case.

| Color | group |
| --- | --- |
|  | eMBB: Characterised by very high traffic, high bit rate. No Reliability needed, no accuracy need |
|  | Critical Communication (CC): Characterised by low latency, ultra high reliability |
|  | V2X: Characterised by high reliability, low rate , high speed and high positioning accuracy |
|  | Massive MTC: Characterised by high density, low mobility for majority, no latency, no reliability needs |
|  | Network operation (NO) |
|  | Ultra Reliable (Alcatel-Lucent) |
|  | Network Flexibility incl. Routing (Alcatel Lucent) |
|  | Backward compatibility (Alcatel-Lucent) |
|  | Alternate Access mechanism (Alcatel-Lucent) |
|  | Flexible Mobility Management (Alcatel-Lucent) |
|  | Optimizing access/Routing (Alcatel-Lucent) |
|  | Broadcast (Alcatel-Lucent) |
|  | Energy Efficiency (Alcatel-Lucent) |
|  | Network optimization |
|  | “Miscellaneous” (Alcatel-Lucent) |

| Use cases | QC use case grouping S1-153077 | VF use case grouping S1-153015, 3016, 3017, 3018 | ALU use case grouping S1-153035 | Nokia Use case grouping S1-153041 | HW Use case grouping S1-153040 | Comments, for SA1 debate |
| --- | --- | --- | --- | --- | --- | --- |
| 5.5 Mobile broadband for indoor scenario |  |  |  |  |  | 5.5 in eMBB |
| 5.6 Mobile broadband for hotspots scenario |  |  |  |  |  | 5.6 in eMBB |
| 5.7 On-demand Networking |  |  |  |  |  | Part of requirement is about QoE data rate and connection density so part of 5.7 in eMBB |
|  |  |  |  |  |  | Part of requirement is: adjust the network capacities dynamically so part of 5.7 in NO |
| 5.10 Mobile broadband services with seamless wide-area coverage anytime and anywhere, e.g., 100Mbps + 500 active UEs simultaneously. + low latency for high speed + 500km/h. |  |  |  |  |  | Part of this use case requirements is 5.10 V2X: High mobility requirement (e.g., 500km/h.) and low latency for high speed scenario |
|  |  |  |  |  |  | Part of this use case requirements is part of 5.10eMBB: QoE data rate e.g. 100MB/s anytime and anywhere |
| 5.30 Connectivity Everywhere |  |  |  |  |  | 5.30 is eMBB |
| 5.11 Virtual presence: high bandwidth (bidirectional)+ low latency |  |  |  |  |  | Requirement on high bandwidth and low latency looks to be eMBB: so we suggest 5.11 in eMBB |
| 5.32 Improvement of network capabilities for vehicular case Streaming:[100] Mbps (DL), Video call: > [10] Mbps (DL and UL), Gaming: > [50-120] Mbps (DL or UL) (end-to-end) is less than [1]s, (over the air) is less than [200] ms, (over the air) is less than [30] ms, (over the air) is less than [1.5] ms up to 100 km/h (end-to-end) is less than [150]ms, (end-to-end) is less than [7.5]ms |  |  |  |  |  | Requirement on QoE data rate and latency with high mobility (up to 100KM/h) so adapted to V2X group |
| 5.50 Low-delay speech coding |  |  |  |  |  | Only 1 requirement on very low one-way service latency [10 ms] so this makes the use case 5.50 belongs to CC |
| 5.15 Localized real-time control UE-UE latency: low latency [1-10 ms] Extremely high reliability self-organized dynamic networking for multi-hop localized network |  |  |  |  |  | Low latency and high reliability makes this 5.15 closer to eMBB |
| 5.45 Industrial Factory Automation cycle times of [1ms to 2ms.] reliability [below 10usecs.] Comm. Range:10-20m |  |  |  |  |  | Majority of 5.45 for CC |
|  |  |  |  |  |  | Part “sufficiently integrity- and confidentiality” of 5.45 requirement is about network operation |
| 5.46 Industrial Process Automation 10k sensor /10sqkm Packet loss <10-5 50-100ms two way Battery: multiple years, transaction rate of one every few seconds sufficiently integrity- and confidentiality-protected |  |  |  |  |  | Majority of 5.46 for CC |
|  |  |  |  |  |  | Part “sufficiently integrity- and confidentiality-protected” of 5.46 requirement is about network operation |
| 5.1 Ultra-reliable communications |  |  |  |  |  | 5.1 for CC |
| 5.12 Connectivity for drones |  |  |  |  |  | 5.12 for CC |
| 5.13 Industrial Control |  |  |  |  |  | 5.13 for CC |
| 5.14 Tactile Internet low latency (~1 ms) high reliability very difficult to block, modify, or hijack |  |  |  |  |  | Performances requirements in 5.14 look to be for CC |
|  |  |  |  |  |  | Part of 5.14 requirement “very difficult to block, modify, or hijack” is about network operation |
| 5.17 Extreme real-time communications and the tactile internet 0.5ms one-way delay |  |  |  |  |  | “0.5ms one-way delay” looks to be a CC requirement, so 5.17 for CC |
| 5.18 Remote Control |  |  |  |  |  | 5.18 for CC |
| 5.44 Cloud Robotics UL: synchronized audio, video and data in real time end to end latency lower than [10ms] high density distribution |  |  |  |  |  | need for latency in 5.44 proposes it as CC |
| 5.54 Local UAV Collaboration [10 ms] latency Reliability near [100%] Priority, Precedence, Preemption (PPP) |  |  |  |  |  | Requirements in 5.44 proposes it as CC |
| 5.41 Domestic Home Monitoring Payload size: messages up to 256 octets per message. Support of a UE connected to remote monitoring IoT Security is provided only by the connected UE. |  |  |  |  |  | Payload size of 5.41 requirement is for mMTC |
|  |  |  |  |  |  | Part of 5.41 requirement on remote monitoring and secutrity is about network operation |
| 5.59 Massive Internet of Things M2M and device identification Connection densities of [200,000 / km2] Support network servers/applications and devices to identify, address and reach other devices |  |  |  |  |  | “connection density” of 5.59 is for mMTC |
|  |  |  |  |  |  | Server Part of 5.59 requirement is about network operation |
| 5.33 Connected vehicles 1 millisecond end-to-end latency Reliability nearly 100% tens of Mbps per device in a dense environment high mobility (e.g., absolute speed more than 200 km/h while relative speed more than 400 km/h). high positioning accuracy (e.g. 0.1 meters) high density of connections for vehicles |  |  |  |  |  | Requirements about high mobility and reliability appears linked to V2X |
| 5.29 Higher User Mobility Enhanced mobile broadband services in fast moving vehicles (e.g. up to 500 km/h) Enhanced connectivity services in fast moving airplanes (e.g. up to 1000 km/h) provide the mobile broadband service in fast moving vehicles with enhanced system experience. provide the airplanes connectivity service with enhanced system experience. |  |  |  |  |  | Requirements are about high mobility so should be considered in V2X group |
| 5.53 Vehicular Internet& Infotainment Internet access: at least [0.5Mb/sec] Music streaming at least [1Mb/sec] video streaming at least [5Mb/sec] UHD streaming at least [15 Mb/sec] no more than [100ms] for internet browsing |  |  |  |  |  | Requirements about high mobility |
| 5.19 Light weight device configuration Support devices (e.g., smart meter) with limited communication requirements and capabilities (e.g., devices without an IMS client). Support a lighter weight signalling for device configuration (i.e., service parameters) than is currently available in EPS. |  |  |  |  |  | 5.19 proposed for mMTC |
| 5.20 Wide area sensor monitoring and event driven alarms |  |  |  |  |  | 5.20 for mMTC |
| 5.24 Bio-connectivity |  |  |  |  |  | 5.24 for mMTC |
| 5.42 Low mobility devices Accept information from large numbers of stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). Support a resource efficient mechanism to provide information to a stationary device provide resource efficient Support for stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). |  |  |  |  |  | 5.42 is proposed for mMTC |
| 5.40 Devices with variable data |  |  |  |  |  | 5.40 for mMTC |
| 5.25 Wearable Device Communication |  |  |  |  |  | 5.25 for mMTC |
| 5.43 Materials and inventory management and location tracking |  |  |  |  |  | 5.43 for mMTC |
| 5.37 Routing path optimization when server changes The 3GPP system shall efficiently support a mobile optimised ICN/CCN protocol. The 3GPP system shall support flexible placement of ICN/CCN caches by enabling flexible user-plane termination (including user-plane termination at the base station). The 3GPP system shall support efficient transport of a mobile optimised ICN/CCN protocol via the 5G and evolved LTE radio. The 3GPP system shall provide charging, LI and QoS support for the ICN/CCN protocol. |  |  | Routing | Routing |  |  |
| 5.36 In-network caching Subject to the service agreement between the operator and the service provider, the 3GPP network shall enable hosting of services (including both MNO provided services and 3rd party provided services) closer to the end user to improve user experience and save backhaul resources. The 3GPP network shall be able to support routing of data traffic to the entity hosting services closer to the end user for specific services of a UE. The 3GPP network shall support efficient user-plane paths between a UE and the entity hosting the service closer to the end user even if the UE changes its location during communication. The 3GPP network shall be able to support charging, QoS, and Lawful Interception (LI) for services hosted closer to the end user. |  |  |  | Routing |  |  |
| 5.38 ICN Based Content Retrieval The 3GPP system shall efficiently support a mobile optimised ICN/CCN protocol. The 3GPP system shall support flexible placement of ICN/CCN caches by enabling flexible user-plane termination (including user-plane termination at the base station). The 3GPP system shall support efficient transport of a mobile optimised ICN/CCN protocol via the 5G and evolved LTE radio. The 3GPP system shall provide charging, LI and QoS support for the ICN/CCN protocol. |  |  |  | Routing |  |  |
| 5.39 Wireless Briefcase The 3GPP system shall support the ability to securely store the personal data information/files of a user in such a way that they are retrievable with no perceptible delay to the user. The 3GPP System shall support a mechanism to control the upload and download of personal information/files between the 3GPP device and a server in the network (e.g., Flat Distributed Personal Cloud). |  |  |  | Routing |  |  |
| 5.3 Lifeline communications / natural disaster Minimal services necessary in case of disaster that are conditional on e.g. subscriber class (i.e. access class), communication class Available in case of disaster. During the recovery phase of disaster, the service continuity of those minimal services that start being provided should be ensured. |  |  |  |  |  |  |
| 5.31 Temporary Service for Users of Other Operators in Emergency Case Support temporary service for users of other than home operators as temporary users in emergency case by serving operator policy. Support defining the limited set of necessary communication services and acceptable terminal features for temporary users by serving operator policy. Maintain an appropriate level of communications security for temporary service |  |  | Access | Acccess |  |  |
| 5.48 Provision of essential services for very low-ARPU areas Support network sharing Efficient support for low or no mobility UEs (e.g. up to 50 km/h). Efficient use of the control plane Efficient use of the data plane Support APIs that provide network status information to applications Minimise as much as possible the traffic (Data and signalling) on the interfaces between the AN and CN Optimise to support UEs with minimal functionality (e.g. user experienced data rate of 10 Mbps at DL and 10 Mbps at UL with E2E latency of 50 ms). Facilitate very large cells (e.g.: link budget better than 160 dB, relaxed timing on random access and other procedures to enable very long range beyond 50km). |  |  |  |  |  |  |
| 5.52 Wireless Self-Backhauling 5G Radio Interface Technology (RIT) designed with features and optimizations to provide a backhaul function. Flexible partitioning of resources between access and backhaul functions when supported in a common band, including quasi-static provisioning of separate access and backhaul resources, and dynamic allocation of access and backhaul resources, e.g., based on current local conditions. Autonomous neighbour discovery and link setup, self configuration of addressing and forwarding plane, and autonomous integration into core/OAM. Use of multiple RITs to increase service availability and network resiliency. Multihop wireless network topologies. Network topologies with redundant connectivity and paths to minimize service disruptions due to network dynamics. Dynamic adaptation to topology changes (e.g., due to node additions, node failures, link fluctuations.) |  |  |  |  |  |  |
| 5.2 Network Slicing |  |  |  |  |  | 5.2 proposed for NO |
| 5.9 Flexibility and scalability |  |  |  |  |  | 5.9 proposed for NO |
| 5.35 Context Awareness to support network elasticity |  |  |  |  |  | 5.35 proposed for NO |
| 5.51 Network enhancements to support scalability and automation |  |  |  |  |  | 5.51 proposed for NO |
| 5.8 Flexible application traffic routing Efficient user-plane paths between UEs attached to the same network, even if the UEs change their location during communication. Efficient user-plane paths between a UE attached to the mobile network and communication peers outside of the mobile network (e.g. Internet hosts). |  |  |  | Routing |  |  |
| 5.49 Network capability exposure |  |  |  |  |  | 5.49 proposed for NO |
| 5.4 Migration of Services from earlier generations |  |  |  |  |  | 5.4 proposed for NO |
| 5.16 Coexistence with legacy systems |  |  |  |  |  | 5.16 proposed for NO |
| 5.21 IoT Device Initialization Secure mechanism (e.g. a factory installed certificate) that enables a device (e.g. IoT device) that has not been provided with a 3GPP subscription to establish access to a 3GPP network. Secure mechanism to remotely provide the device with a particular 3GPP subscription and other, third party provided, device configuration credentials when the device was not previously provided with a 3GPP subscription |  |  |  |  |  | Requirement about security of IoT could be considered generic so for horizontal NO |
| 5.22 Subscription security credentials update Secure mechanism to update the subscription security credentials for an IoT device. Secure way of storing subscription security credentials in the IoT device |  |  |  |  |  | Requirement about security of IoT could be considered generic so for horizontal NO |
| 5.34 Mobility on demand Enable operators to define different levels of mobility support for different UEs. Mobility support consists of providing none, any one or any combination of the following: - minimizing packet loss during inter- and/or intra-RAT cell changes, - maintaining the same IP address assigned to a UE across different cells, - minimizing interruption time until a UE can continue to communicate with a potentially different IP address |  |  |  | Routing |  |  |
| 5.47 SMARTER Service Continuity Shall be able to change the IP anchoring point for a UE with minimal impact on the user experience. |  |  |  | Routing |  |  |
| 5.58 Use Case for Green Radio 1000] times energy efficiency compared to legacy system. |  |  | Energy Efficiency | Acccess |  |  |
| 5.56 Broadcasting Support Support an interface from external Broadcasters’ management systems. Reserve groups of resources for Broadcast channels Broadcast of lossless state of the art video streams such as 4k UHD. Receive a reduced quality version of the broadcast for display on their device screen (typically less than 12”) or a full quality version of the same channel for presentation to a video presentation monitor (typically much larger than their device , ie:32” to 72”screen size) Allow the UE to receive broadcasts selected by the user (from the Broadcaster’s management system) in accordance with any appropriate authorisations by the Broadcaster. |  |  |  |  |  |  |
| 5.57 Ad-Hoc Broadcasting Support an interface from an external Ad-Hoc Broadcast management system that manages broadcast requests. Reserve groups of resources temporarily for scheduled Ad-Hoc Broadcasts. Allow the UE to receive broadcasts selected by the user (from the Ad-Hoc Broadcast management system) in accordance with any appropriate authorisations. |  |  |  |  |  |  |
| 5.55 High Accuracy Enhanced Positioning (ePositioning) Support higher accuracy location capability less than [3 m] at [80%] of occasions. Support different configuration for accuracy according to different service requirements. Initial position fix time of UE shall be less than [10] seconds, and subsequent position fixes shall take no longer than [10~15ms], if required. The two-way delay for positioning shall be no more than [10-15 ms]. Power consumption due to the continuous use of positioning service shall be minimized Support co-existence with legacy 3GPP positioning service and migration to higher accuracy positioning service. |  |  |  |  |  |  |
| 5.27 Multi Access network integration Provide 3GPP services to the UE using various 3GPP and non-3GPP access networks (e.g. WLAN, Fixed broadband access, Bluetooth, etc). To support at least: - Inter-system mobility between 3GPP and non-3GPP networks with optional session continuity, - capability for the UE based on network control to select the access, - simultaneous connection to different accesses, capability for the UE to access the 3GPP services provided by the 5G network using non-3GPP access e.g. FMSS…, - authentication to access to future network through a non-3GPP access shall use 3GPP credentials. Enable the UE to simultaneously connect to the network via different non-3GPP and 3GPP accesses. Manage the addition or dropping of the various accesses dynamically during the session according to the quality of the individual connections. Data transmissions that leverage both types of access. Aggregate the UE data transfer via one access or a combination of accesses which may be operated by different mobile network operators, to provide the best user experience during an ongoing data transmission. Support the interworking with fixed broadband network defined by BBF. The traffic from different subscribers using a non-3GPP access shall be isolated from each other. |  |  |  | Acccess |  |  |
| 5.28 Multiple RAT connectivity and RAT selection Provide data transmission by using both the <5G New RATs> and E-UTRA simultaneously. Select a radio access (either a <5G New RAT> or E-UTRA) to assign each data flow, taking into account |  |  |  | Acccess |  |  |
| 5.26 Best Connection per Traffic Type Provide a mechanism such that a specific traffic type (from a specific application or service) to/from a UE can be routed via specific RAN nodes, and traffic in one RAN node can be offloaded towards a defined IP network close to the UE's point of attachment to the access network, while other traffic type to/from that same UE is not offloaded. |  |  |  | Acccess |  |  |
| 5.23 Access from less trusted networks UE may use a temporary identifier for initial attach that hides its long-term identity. The home network shall be able to associate this temporary identifier to the long-term identity. |  |  |  | Acccess |  |  |

- Proposals for TR changes

***** BEGIN 1st CHANGE *****

## 6.3	Considerations on grouping of requirements

### 6.3.1	Collected potential requirements

#### 6.3.1.1	Potential requirements for vertical scenarios (eMBB, mIOT, CC)

The following table collects potential service requirements from use case related to verticals as examples to illustrate the main difference between verticals.

| Vertical group | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Traffic density | Conn. density | UE speed | Position accuracy | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group eMBB | Very High data rate (e.g. peak rate 10 Gbps, up to 10 Gbps when the user is moving slowly, DL 300Mbps with DL 50Mbps, 100Mbps) | Very low latency, low latency for high speed, reliable low-latency connectivity between aerial objects |  |  | High traffic density (e.g Tbps/ km2） | High density for UE (e.g. 200-2500 /km2, 2000/km2, 50 active UEs simultaneously ) |  |  | Characterised by very high traffic, high bit rate. No Reliability needed, no accuracy need |
| Group Critical Comm. |  | Realtime low latency (e.g. as low as 1 ms end-to-end; the case “Smart grid system”: less than 8 ms, Round trip latency less than [150 ms], low latency (~1 ms), UE-UE latency: low latency [1-10 ms], 0.5ms one-way delay, Round trip latency less than [150 ms]) | Ultra high Reliability, high availability (e.g. limit the duration of service interruption for mission critical traffic, Packet loss rate: as low as 1e-04; delivered in 8 ms, Realibity with Priority, Precedence, Preemption (PPP) mechanisms) |  | high density distribution (e.g. 10k sensor /10sqkm) |  |  | Precise position within [10 cm] in densely populated areas. | Characterised by low latency, ultra high reliability |
| Group mIOT |  |  |  | Coverage enhancement, Efficient resource and signalling to support low power, support devices (e.g., smart meter) with limited communication requirements and capabilities | High density massive connections (e.g.1 million connections per square kilometre), to accept information from large numbers of locally dense devices, possibly simultaneously | Low mobility (for majority of MTC cases except for inventory) |  | High positioning accuracy in both outdoor and indoor scenarios (e.g., 0.5m) | Difference with CC is: No low Latency |
| Group eV2X | Medium Rate (10 of Mbps per device) | Low latency (e.g. 1 millisecond end-to-end latency) | High Reliability (nearly 100%) |  | Medium traffic density | Medium connection Density (e.g. the number of vehicles can exceed 10000 in scenarios with multiple lanes and multiple levels and types of roads) | High mobility(e.g. up to 500 KM/h, absolute speed more than 200 km/h while relative speed more than 400 km/h). | High positioning accuracy (e.g. 0.1 meters) | V2X differs from eMBB due to its high reliability , lower rate , high speed and high positioning accuracy |

***** END OF CHANGE *****