3GPP TSG-SA WG1 Meeting #72	S1-154327

Anaheim, CA, 16-20 November 2015	Revision of S1-154242

Title:	Use case for QoS and Policy

Agenda Item:	8.3 New Cric Use Case

Source:	Samsung

Contact:	Jayshree Bharatia (Jayshree.b@partner.samsung.com)

Abstract: This paper proposes use case for QoS and Policy requirements

## Proposal

## 5.1.x	Use case for Priority, QoS and Policy Control

### 5.1.x.1	Description

5G network will be supporting massive number of devices with various data, transmission and bandwidth requirements. Due to a wide range of devices, services targeted for 5G can be very different from existing services in terms of amount, type and pattern of data exchange over the network. In order to cope with diverse service requirements, it is imperative that intelligent decisions are made at the network such as allocation of resources, scheduling of resources and adapt the network to meet these service requirements.

Many devices in a 5G network such as metering devices and monitoring sensors are expected to operate unattended. A large number of such devices may not be capable to alter their behaviour and adapt their participation as required to meet diverse service requirements. It is therefore vital for the network to assist these devices to operate such that services are able to utilize these resources and alter their behaviour when necessary to do so.

The 5G network will also be supporting many commercial (e.g., medical) and regional/national regulatory specific (e.g., MPS, emergency services) critical communications applications with requirements for priority treatment. During certain events (e.g., disaster events and network congestion), relative priority decisions will need to be made based on priority characteristics as:

- efficient and rapid execution of the needed priority is desired without alteration of the underlying QoS specification,

- the priority of a particular application may need to be different (higher or lower) during a crisis event from that normally adopted by the system, and

- the priority of any given application may need to be different, e.g., elevated, for a particular user of that application based on operational needs and regional/national regulations.

The network must offer a means to provide high reliability, predictable latency and ability to adapt and when necessary prioritize resources to meet specific service requirements. Existing QoS and policy frameworks merely handle predictable latency and improving reliability by traffic engineering. In order to support diverse 5G service requirements, it is necessary for the network to offer QoS and policy control for reliable communication with predictable latency and also enable the resource adaptations as necessary.

The network also need to support flexible means to make relative priority decisions based on the state of the network (e.g., during disaster events and network congestion) recognizing that the priority needs may change in time during a crisis event. Since it is outside the scope of specifications to provide a specific ranking of priority amongst services, flexibility needs to be provided to conform to operator objectives and regional/national regulatory requirements.

The following example depicts a scenario involving MTC devices used for a sample healthcare monitoring service. The upper figure shows that periodic health monitoring reports are sent for a patient every six hours to a health monitoring system. These reports are communicated using a best effort QoS classification since it is not critical for the service operation.

In the middle of one of these reporting periods, the patient falls down and starts having health complications. This scenario is shown in the lower figure. A sensor detects this catastrophic event and reports it to the service. The service now needs additional health monitoring reports urgently. Moreover, depending on the initial reported problem, the service may direct the network to coordinate group conversations involving appropriate health facilities and designated individuals associated with the patient. Some aspects of this communication may require the network to directly adapt existing communication and setup new resources to meet service requirement. The behaviour of monitoring sensors must be altered such that they deliver their reports more aggressively, in great details and to a larger audience. The service may further request network to tune the QoS depending on the patient’s subscription type.

This example shows that the network must offer flexible means to adjust QoS and priority treatment andalter its behaviour based on service state.. Such adaptive measures can be feasible only if the network can adjust its behaviour to accommodate the QoS and priority treatment requirements..

Figure 5.x.1 1 – Example showing different QoS/Policy requirements based on service execution status

### 5.1.x.2	Potential Service Requirements

The 3GPP system shall offer a means to provide high reliability, required latency in communication and ability to adapt and when necessary prioritize resources to meet specific service requirements.

The 3GPP system shall allow flexible means to make and enforce relative priority decisions among the different application services based on the state of the network (e.g., during disaster events and network congestion) recognizing that the priority needs may change in time during a crisis event.

The 3GPP system shall be able to support QoS based on state of a service.

### 5.1.x.3	Potential Operational Requirements