3GPP TSG-SA WG1 Meeting #73	S1-160330

# Okinawa, Japan, 1-5 February 2016			 									(revision of S1-160142)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | 0510 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for enhanced TV service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnTV |  |  |  |  |  |  | Date: |  | 2016-02-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduce new requirements for 3GPP enhancement for TV application support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds new requirements including requirement for unicast TV reception. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# X 	3GPP enhancement for TV

# application support

## X.1	Requirements

### X.1.1	General requirements

The 3GPP network shall be able to support on-demand network capacity assignment for MNO/OTT provider’s TV program.

The 3GPP system shall allow the MNO to implement a network supporting downlink only broadcasting based on a set of EPS functionalities and entities which are required to offer linear TV programmes.

The 3GPP network shall be able to support on-demand network broadcast geographic area coverage management for MNO/OTT provider’s TV program.

The 3GPP network shall support UHD content delivery.

The 3GPP network shall be able to provide TV FTA programs receivable by UEs subscribed to the MNO and roaming UEs of other MNOs.

A UE may support a mode whereby only eMBMS broadcast reception functions are supported.

Note: The support of this mode does not require the UE to be registered to the broadcasting MNO.

A UE supporting enhanced TV service shall be able to receive broadcasted (using eMBMS bearer services) TV service when in limited service state, or using receive-only mode.

The MNO is responsible for maintaining the privacy & anonymity of the receiving user

The 3GPP network shall provide the ability for an MNO to meet regulatory requirements for privacy and anonymity.

The 3GPP network shall provide mechanisms to restrict the reception of some or all TV programs to many subscribers (e.g. based on the recipients of the programs are subscribers of the MNO, roaming subscribers of other MNOs, or not subscribed to any MNO).

The quality of experience of the TV program shall be independent from the size of the concurrent audience.

While receiving TV programs, subscribers of the MNO and roaming subscribers shall be able to access other subscribed services such as voice, data or SMS.

The 3GPP network shall support combinations of SD, FHD and UHD resolution programs on LTE based TV program.

The 3GPP network shall allow the MNO to provide a TV program in a resource efficient manner over a large area possibly consisting of urban and rural parts, up to the size of a country.

The 3GPP network shall enable the quality of experience of the TV program to be ensured throughout the coverage area'

### X.1.2	Requirement for broadband broadcast performance enhancement

The 3GPP system shall be able to support [8Mbps-20Mbps] over one broadcast channel.

The 3GPP network shall be able to support at least [10] high quality video channels of 12Mbps each simultaneously via broadcast.

The 3GPP network shall support flexible change between broadcast and unicast per traffic demand over the same carrier.

The 3GPP network shall be able to support using a full carrier for broadcast.

The 3GPP network shall be able to convert network unicast capacity to network broadcast capacity and vice versa.

The 3GPP network shall be able to support using one carrier for both unicast and broadcast distribution of TV programmes.

If allowed by the operator and a spare carrier is available, the 3GPP network shall be able to make use of spare carriers or free up carriers if not required anymore.

### X.1.3	Requirement for broadcast network flexibility

The 3GPP network shall be able to support network broadcast capacity assignment considering following criteria:

- OTT provider request (including TV program information(e.g., codec information, media type information, etc))

- available network unicast/broadcast capacity of 3GPP network

Within the radio resources dedicated to LTE based TV program in a geographic area the 3GPP network shall allow the MNO to specify the allocation of radio resource by the following:

- static allocation, i.e., pre-defined maximum percentage to be used for unicast and broadcast

- dynamic allocation, i.e., the pre-defined maximum percentage for both unicast and broadcast to be changed at a time-resolution of [one] minute over a period of [24] hours

The 3GPP network shall be able to support network broadcast geographic area coverage management considering following criteria:

- OTT provider request (including the potential coverage information of TV program information)

- available network unicast/broadcast capacity of 3GPP network

- number of users under broadcast network coverage

- The location information of UE

The 3GPP network shall be able to deliver media content to a dynamically formed broadcast group in an efficient manner.

The 3GPP system shall be capable of ensuring the timing sequence of different media content received by different UEs belonging to the same user, via different transport path, aligning with the timing sequence of TV program of the OTT provider.

A UE shall be able to receive eMBMS from one cell while receiving other services from the same cell or another possibly cell.

A MNO shall be able to service all UEs in an area with eMBMS independent of which MNO network(s) they have subscription to.

### X.1.4	Requirement for TV application support

The eMBMS service layer should support audio and video formats typically supported by TV Content Providers for HD TV services and UHD TV programs.

The eMBMS service layer should support codecs typically supported by TV Content Providers for HD TV programs and UHD TV programs.

The eMBMS service layer should support accessibility functions typically supported by TV Content Providers (e.g. subtitling, closed captioning, audio descriptions, anonymous reception, reporting to support ratings, reporting enforcement, etc.).

The eMBMS service layer should support regulatory mandates typically supported by TV Content Providers (blackouts, emergency alerts, etc.).

The eMBMS service layer should support interactivity functions typically supported by TV Content Providers (interactive services, second screen, personalization, etc.).

The eMBMS service layer should support ad insertion use cases typically supported by TV Content Providers (targeted ad insertion, ad replacement, etc.),

The eMBMS service layer should support encryption, security and conditional access functions typically supported by TV Content Providers.

The eMBMS service layer should support efficient concurrent delivery of multiple application components (TV program application signalling, statistical multiplexing, etc.).

The eMBMS service layer should support random access and channel change times comparable to existing HD TV programs.

The eMBMS service layer should support program content delivery over broadcast only, unicast only and combinations of the two.

The eMBMS service layer should support delivery of real-time and non-real-time content.

The eMBMS service layer should enable extensibility and forward-compatibility to new requirements, formats, codecs and other functions to the extent possible.

An eMBMS transport layer should be able to transport TV service streams formatted not compliant to 3GPP standards.

A UE should be able to access an eMBMS transport session with the support of only transport metadata.

An eMBMS system should be able to be initiated by a UE with sufficient metadata provided by a mechanism other than User Service Description for non 3GPP transport services.

### X.1.5	Requirement for RAN sharing

A participating eMBMS-enabled RAN shall be capable of distributing the content including both free-to-air content and subscribed content in conjunction with other participating eMBMS-enabled RANs.

The hosting eMBMS-enabled RAN network provider shall be able to manage the shared eMBMS network to each of the participating MNOs.

Each RAN provider shall report events supporting the accounting of network resource usage separately for each Participating MNO. This includes:

- Start of service in the shared eMBMS network for a UE of the Participating MNO

- End of service in the shared eMBMS network for a UE of the Participating MNO

### X.1.6	Requirement for wide area support

The eMBMS network shall be able to support designated coverage area, for example national, regional and local coverage areas.

### X.1.7	Requirement for capability exposure

The 3GPP system shall be able to support an MNO offering on-demand broadcast availability to MNO/OTT provider’s TV program.

Subject to MNO and 3rd OTT party agreement, to enable the MNO to evaluate the impact on distribution of data transfer in the network, the 3rd OTT party shall be able to indicate to the MNO the desired media format information: the volume of the data traffic expected to be broadcast in this geographic area, indication whether the streaming content can be accessed when the mobile users moves out/in of broadcast coverage area, indication whether the streaming content can be cached in the MNO network, association information among users i.e. user information, expected media content information, device information and group information of TV program and the timing relationship information of different media content.

The 3GPP network shall be able to allow MNO to inform the result (e.g., selected media format information, mode of content delivery (broadcast only, broadcast/unicast fallback support, MooD), QoE of content delivery, cached content indication) of delivery to 3rd party.

### X.1.8	Fixed reception for TV program

The 3GPP network shall allow a UE to receive a broadcast TV content using existing TV antenna equipment.

The 3GPP network shall allow a UE to receive unicast TV content using existing TV antenna equipment, at least for the downlink

Note: This might imply the uplink to take another path e.g. via a different antenna if existing TV antenna equipment is not suitable for transmission.