3GPP TSG-SA WG1 Meeting #74	S1-161376

Venice, Italy, 9-13 May 2016	(revision of S1-161074)

Title:	pCR – MCCoRe – MCVideo – MCData Changes to transmission control requirements

Agenda Item:	7.3, 7.5, 7.6

Source:	UK Home Office

Contact:	david.freeman1@homeoffice.gsi.gov.uk

Abstract: Floor control works well for speech whereas for MCVideo there needs to be some form of transmission control but a network based floor control function is unlikely to provide the optimum solution. For Data there is probably no floor control function required as such but there will be data access queuing control and notification of receiving members.

The functions for MCPTT, video and data are sufficiently different that the requirements for each should remain in the service specification rather than the combined requirements specification (MCCoRe).

It is suggested to incorporate a definition for Participant. Currently MCCore has an empty definition for Participant, MCVideo has none and MCPTT has:-

Participant: An MCPTT User who is currently receiving and/or transmitting in an MCPTT Group Call or a Private Call.

Start of change to 22.280

### 6.2.3	Floor control

#### 6.2.3.1	General aspects

#### 6.2.3.2	Requesting permission to transmit

End of change to 22.280

Start of change to 22.281

### 5.2.6	Transmission control

Editors Note: Rapporteur to find suitable location in document and consequential numbering.

#### 5.2.6.1	Overview

MCVideo provides a method of working where each video being sent is delivered as an invitation to the affiliated group members for them to elect to commence receiving or not as suits their situation. It is far better not to send video to a user who is unable to view it. Conflicting or simultaneous video communications would result in simultaneous (or near simultaneous) invitations to join and the users can select which, if any, they choose to join. Forced reception of video can be configured by an authorised user In this case, suitably authorised receiving users could be permitted to opt out of viewing, perhaps because they have a specific task to watch some other stream.

As a consequence a video group communication can allow the

user to accept the video stream before receiving a video and can allow the

user to choose from available video streams amongst a list of ongoing video streams available to the user.

#### 5.2.6.2	Transmission control requirements

##### 5.2.6.2.1	General

[R-5.2.6.2.1-001] The MCVideo Service shall determine which Participant(s) are allowed to transmit a video stream.

[R-5.2.6.2.1-002] An authorized Participant shall be able to request to transmit to an MCVideo Group or an individual.

[R-5.2.6.2.1-003] When a request to transmit a video stream to an MCVideo Group is granted the MCVideo Service shall send an invitation to all affiliated members of the selected group to allow them to accept to receive the video.

[R-5.2.6.2.1-004] An MCVideo Group shall have the flexibility to be configured to allow simultaneous transmitting MCVideo Group Members.

[R-5.2.6.2.1-005] MCVideo Service shall provide a mechanism for the MCVideo Administrator to configure the maximum number of simultaneous transmissions in a single MCVideo Group.

Note:	No limit is a valid setting.

[R-5.2.6.2.1-006] The MCVideo Service shall determine who is allowed to transmit when there are requests to transmit more than the maximum number of simultaneous communications within the same group.

[R-5.2.6.2.1-007] The MCVideo Service may withhold permission to transmit for service based pre-emptive capacity reasons.

[R-5.2.6.2.1-008] Following an MCVideo Service request for permission to transmit on the Selected MCVideo Group, the Affiliated MCVideo Group Member that made and was granted the request shall be given an indication of being granted permission to transmit.

[R-5.2.6.2.1-009] Following an MCVideo Private Communication request for permission to transmit, an MCVideo User that is allowed to transmit shall be given an indication that the user is allowed to transmit to the targeted MCVideo User(s).

[R-5.2.6.2.1-010] When a user is not allowed to start a communication the request may be queued or rejected.

[R-5.2.6.2.1-011] Following an MCVideo Service request for permission to transmit on the Selected MCVideo Group, an Affiliated MCVideo Group Member that made but was not granted the request shall be given an indication that permission to transmit was queued or rejected.

[R-5.2.6.2.1-012] Following an MCVideo Private Communication request for permission to transmit, an MCVideo User that is not allowed to transmit shall be given an indication that the permission to transmit was queued or rejected.

##### 5.2.6.2.2	Receiving video streams

[R-5.2.6.2.2-001] When a video group communication is started all affiliated group members shall be sent an invitation notifying them of the new video stream.

[R-5.2.6.2.2-002] The MCVideo Service shall provide a mechanism to include conditions for acceptance or rejection of the invitation (e.g. forced, unforced).

[R-5.2.6.2.2-003] In the case of unforced invitation to receive an MCVideo user shall receive the video stream of a MCVideo communication only after having accepted the invitation.

[R-5.2.6.2.2-004] MCVideo Service shall provide a mechanism for the MCVideo Administrator to configure the maximum number of simultaneous streams received by a MCVideo User.

Note:	No limit is a valid setting.

[R-5.2.6.2.2-005] As a configuration option, the MCVideo system shall provide a means for an authorized MCVideo user to automatically receive video communications.

Note:	When configured for the User to receive a video stream automatically, the transmission starts without acceptance of the receiving MCVideo User.

[R-5.2.6.2.2-006] As a configuration option, the MCVideo service shall provide a means for an MCVideo user to automatically receive emergency video streams.

[R-5.2.6.2.2-007] As a configuration option, the MCVideo service shall provide a means for an authorized MCVideo user to automatically receive imminent peril video streams.

[R-5.2.6.2.2-008] An authorised user shall be able to configure an MCVideo group to support individual user/UE opt in to receive communications or mandatory/automatic reception.

[R-5.2.6.2.2-009] An MCVideo user shall be able to opt in to receive communications for any recently invited group communications.

Note:	A list of recent invitations to opt in should be stored and presented to the user on demand. Recent may be time limited, size of list limited or any other limit. Ceased communications should be removed from the opt in invitation list.

[R-5.2.6.2.2-010] When a video private communication is started the receiving participant(s) shall be given the option of accepting or rejecting the communication.

##### 5.2.6.2.3	Entering an ongoing video stream

[R-5.2.6.2.3-001] The MCVideo Service shall provide to the MCVideo User the list of ongoing/currently transmitting video group communications, for which he is affiliated, periodically and on demand.

[R-5.2.6.2.3-002] The MCVideo user shall be able to choose one or more video group communications subject to MCVideo settings and UE capabilities (e.g. MCVideo UE display capabilities).

[R-5.2.6.2.3-003]The MCVideo Service shall provide a means to separately control the sound of the different video communications received.

##### 5.2.6.2.4	Video streams reconfiguration and termination

[R-5.2.6.2.4-001] The MCVideo Service shall make efficient use of resources (e.g.use of eMBMS).

[R-5.2.6.2.4-002] The MCVideo Service shall provide a notification to a transmitting MCVideo Group Member if there are no other MCVideo Group Members who have affiliated to the MCVideo Group.

[R-5.2.6.2.4-003] The MCVideo Service shall provide a notification to a transmitting MCVideo Group Member if there are no MCVideo Group Members receiving their communication.

Note:	When an opt in communication is commenced, a reasonable time is allowed for other users to opt to receive the communication before such a notification is sent.

[R-5.2.6.2.4-004] The MCVideo Service may interrupt the transmission of an MCVideo communication when there is no MCVideo User receiving it.

[R-5.2.6.2.5-005] The MCVideo Service may restart the transmission of an MCVideo communication as soon as any affiliated/authorized user wants to receive the MCVideo communication.

End of change to 22.281

Start of change to 22.282

#### 5.2.6.2	Transmission control requirements

##### 5.2.6.2.1	General

[R-5.2.6.2.1-001] The MCData Service shall determine which Participant(s) are allowed to transmit data.

[R-5.2.6.2.1-002a] The MCData Service shall provide a mechanism so that small data items can be automatically sent to the selected receiving users (private communication) or to affiliated members of the selected MCData group.

[R-5.2.6.2.1-002b] The MCData Service shall provide a mechanism so that when a MCData request to transmit is granted the data can be automatically sent on to the selected receiving users (private communication) or to affiliated members of the selected MCData group (i.e. without prior acceptance from the receiving user).

[R-5.2.6.2.1-002c] The MCData Service shall provide a mechanism so that when a MCData request to transmit is granted the data can be temporarily stored in the system and an invitation to receive is sent to the selected receiving users (private communication) or to affiliated members of the selected MCData group.

Note:	This is for support of sending large data files, allowing the receiving user to choose when to receive.

[R-5.2.6.2.1-002d] The MCData Service may provide a configurable default time to live value for data waiting to be delivered to a receiving user.

[R-5.2.6.2.1-002e] The MCData Service may terminate all remaining invitations to receive after expiry of the time to live.

[R-5.2.6.2.1-003] The MCData Service shall provide a mechanism for the MCData Administrator to configure the maximum data size for automatic data transmission.

[R-5.2.6.2.1-004] When a MCData UE has begun to transmit data or when the data transmission has completed the MCData Service shall begin to send the data to all affiliated members of the selected group.

Note:	Where the MCData service chooses to use eMBMS to broadcast the data to several users, the UE will need to be notified to select the relevant channel before the data is sent.

Note: 	The MCData Service may choose to temporarily store the transmitted data and deliver it independently to receiving users as appropriate.

[R-5.2.6.2.1-005] The MCData Service may withhold permission to transmit for service based pre-emptive capacity reasons.

[R-5.2.6.2.1-006] Following an MCData Service request for permission to transmit on the Selected MCData Group, an Affiliated MCData Group Member that made but was not granted the request shall be given an indication that permission to transmit was not given at that time.

[R-5.2.6.2.1-007] When a user is not allowed to start a communication the request may be queued or rejected.

##### 5.2.6.2.2	Receiving data

[R-5.2.6.2.2-001] An MCData user shall be able to select to download data for any recently invited group communications.

Note:	A list of available data should be stored and presented to the user on demand. Recent may be time limited, size of list limited or any other limit.

##### 5.2.6.2.3	Accessing data

[R-5.2.6.2.3-001] The MCData Service shall provide, periodically and on demand, to the MCData User the list of available recently invited data group communications for which he is affiliated.

[R-5.2.6.2.3-002] The MCData user shall be able to affiliate to one or more MCData Groups subject to MCData settings and UE capabilities.

##### 5.2.6.2.4	Data reconfiguration and termination

[R-5.2.6.2.4-001] The MCData Service shall make efficient use of resources.

[R-5.2.6.2.4-002] The MCData Service shall provide a notification to a transmitting MCData Group Member if there are no other MCData Group Members who have affiliated to the MCData Group.

[R-5.2.6.2.4-003] Following an MCService notification that there are no other MCData Group Members affiliated to the MCData Group and if the transmitting MCData Group Member does not terminate their request to transmit, the MCData Service may; terminate the permission to transmit or allow the transmission to continue and store the transmitted data for a time to live time and deliver invitations to receive to subsequent affiliating members.

End of change to 22.282