3GPP TSG-SA WG1 Meeting #74	S1-161551

(revision of S1-161428)

Title:	Use Case: Information exchange within platoon

Agenda Item:	5 - New Study and Work Items (including related contributions)

Source:	China Mobile Communications Corporation

Contact:		Yuan Li, liyuanyjy@chinamobile.com

Abstract: V2I and V2V system can help the platoon realise the real-time traffic and road information exchange and sharing so that the high-precision dynamic driving maps will be built up. It will provide the necessary guarantees for automated driving.

3	Definitions and abbreviations

3.1	Definitions

3.2	Abbreviations

V2V  Vehicle to Vehicle

V2I   Vehicle to Infrastructure

RSU  Road Side Unit

4	Overview

5	Use Case: Information exchange within platoon

### 5.X.1	Description

When the vehicles are travelling on the road, they can dynamically form a platoon. The platoon creator is responsible for platoon management. The manager should real-time updates surrounding traffic data reported by group members, and reported it to RSU; At the same time, the platoon manager should real-time receive RSU messages which include road conditions and traffic information far away from them, and share them with platoon members. All the platoon members can also share the information within the group through V2V.

All the platoon members can obtain information through two ways. One is within platoon through V2V. Another is come from RSU which based on the platoon manager relay. All the information obtains will be used to build high-precision dynamic driving maps.

Information exchange between vehicles can be the new type of “ask-response”.

### 5.X.1.1	Pre-Conditions

All the vehicles support V2V communication and the platoon manager should support V2V and V2I communication. Each vehicle has a unique label for identification when interacting with others.

The platoon has more than 1 active vehicle which is driving in the same lane. All the platoon members are within the communication range with its direct neighbour which also is the platoon member.

The platoon management is responsible for platoon management and information exchange with the RSU.

### 5.X.1.2	Service Flows

Vehicle A/B/C/D are within the same platoon, and vehicle A is the team manager.

Vehicle A/B/C/D sharing the surrounding real-time traffic and road information within the platoon. Vehicle A reports all the information to RSU.

Vehicle A receives traffic and road information from RSU, found there are road congestion caused by traffic accidents in the front corner of the road which are out of their V2V communication range.

Vehicle A sharing the information to vehicle B/C/D. Vehicle B/C/D update the driving map in real time and the platoon slow down and change the route.

### 5.X.1.3	Post-Conditions

Vehicles within platoon can build the high-precision dynamic driving map by real-time sharing information with platoon members which can provide reliable protection for traffic safety and efficiency.

Vehicles which only support V2V can obtain dynamic information out of the V2V communication range through platoon information exchange.

Also the vehicles without HD camera and radar can obtain the surrounding information from other platoon members.

### 5.X.2  Potential Requirements

[PR.5.X.2-001] The 3GPP system shall be able to support a maximum latency of [TBD]ms for the transport of V2X messages between a UE supporting V2X application and a RSU via another UE supporting V2X application.

[PR.5.X.2-002] The 3GPP system shall be able to support the transport of [TBD] V2X messages per second between a UE supporting V2X application and a RSU via another UE supporting V2X application..

[PR.5.X.2-003] The 3GPP system shall be able to support the transport of V2X messages with variable payload of [TBD] bytes between a UE supporting V2X application and a RSU via another UE supporting V2X application.

.