3GPP TSG-SA WG1 Meeting #75	S1-162241

San Francisco, CA, USA, 22-26 August 2016	(revision of S1-162110)

Title:	Dynamic Control of Quality of Service, Priority, and Pre-emption

Agenda Item:	8.1 Smarter: New Services and Markets Technology Enablers [SP-160486]

Source:	U.S. Department of Commerce

Contact:	Ihab.Guirguis@FirstNet.gov

Andrew@its.bldrdoc.gov

Abstract: This document proposes to transition reqiuirements from the SMARTER – Critical Communications feasability study into the SMARTER normative specification.

# Proposal

This document reviews and proposes consolidated requirements on dynamic control of quality of service, priority, and pre-emption features from TR 22.862 for the SMARTER TS 22.XXX.

--- Beginning of Change ---

# 7	Performance requirements

## 7.X Critical communication services

### 7.x.1 Requirements

The 3GPP system shall support the prioritisation of users and traffic, during connection setup and when connected.

NOTE:	Relative prioritization, pre-emption, and precedence of critical traffic associated with certain priority services (e.g., MPS and Emergency) are subject to regional/national regulatory and operator policies.

The 3GPP system shall support optimised signalling for prioritised users and traffic.

The 3GPP system shall allow flexible means to make and enforce relative priority decisions among the different mission critical service flows, as authorized by operator policy, used by third parties, i.e., authorized public safety users.

The 3GPP system shall be able to provide a real-time dynamic control function that adapts the prioritized access, QoS, and policies based on various criteria (user status (e.g. location, emergency), network status (e.g. congestion), service data (e.g. required QoS), and incident or other dynamic data).

The 3GPP system shall be able to support a QoS and policy framework that can be communicated manually/automatically to/from non-3GPP systems.

The 3GPP system shall support efficient mechanisms to allow the dynamic allocation of Priority, QoS and Policies for prioritised communications based on real time network status data.

The 3GPP system shall support the allocation of resources dedicated to certain users.

The 3GPP system shall support the isolation of dedicated system resources from other system resources.

The 3GPP system shall support different levels of protection for users and traffic.

The 3GPP system shall support different levels of resilience, availability, coverage, and reliability to offer different levels of guaranteed communications.

The 3GPP system shall support alternative access technologies (e.g., satellite) to increase availability and coverage.

The 3GPP system shall support direct device connections, also for high data rates (e.g., 100 Mbps).

The 3GPP system shall provide an enhanced multicast service that provides equivalent coverage and latency performance to the unicast service.

The 3GPP system shall support different service architectures (e.g., locally, in the core network, and in the cloud).

The 3GPP system shall support dynamic adaptation of the service architecture.

The 3GPP system shall support different sets of supported services for different user (e.g., based on the type of user)

The 3GPP system shall support the use of services of a network without a roaming agreement.

# 8	Security

--- End of Change ---